STANDARD MICROSYSTEMS CORPORATION

SMC EtherCard PLUS Family LAN Adapter Drivers

PCNFS Environment

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

INSTALLATION INSTRUCTIONS

-------------------------

1. Insert the Installation and Connections diskette into the

diskette drive (A:\ or B:\) and type: install. Press Enter.

2. Enter the destination drive letter (usually C:\). Press Enter.

3. Specify "Ethernet" and press Enter.

4. Select the "NDIS" option.

5. Follow all the prompts that appear.

6. Specify the PC name, IP address, server name, and server

server IP address.

7. Follow all the prompts to finish the installation.

8. After you are finished, copy the smcmac.dos file from the

\NDIS subdirectory on your SuperDisk to the \LANMAN directory

on your destination driver.

10. Using a text editor, edit the CONFIG.SYS. Add this

line below the NFS_NDIS driver specification:

device=c:\lanman\smcmac.dos

11. Edit the PROTOCOL.INI file.

a. Replace

[YOUR-MAC-MODULE]

with

[SMC].

b. Replace

drivername = yourmac$

with

drivername = smcmac$

c. Specify the correct IOBase or SlotNumber value

for this adapter depending on whether the adapter

is an ISA-bus or Micro Channel computer.

d. Replace

bindings = YOUR-MAC-MODULE

with

bindings = SMC

12. Reboot the computer.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.