STANDARD MICROSYSTEMS CORPORATION

Novell NetWare Remote Program Load(RPL) driver

For EtherCard PLUS Family LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

INSTALLATION INSTRUCTIONS:

On the NetWare 386 3.11 Server do the following:

1. Load SMCPLUSS.LAN using the 802.2 frame type.

2. Bind IPX to SMCPLUSS.LAN

3. Load Novel's RPL Protocol Stack "RPL.NLM" on the server. This .NLM

is distributed by Novell and is available on NetWire.

4. Bind RPL to SMCPLUSS.LAN

5. Copy $0000.RPL from the SMC SuperDisk to the LOGIN sub-directory

on the File Server.

6. Build the NET$DOS.SYS following the Novell instructions and copy

it into the LOGIN sub-directory on the File Server.

Workstations configured with the SMC RPL ROM can now remote boot from

the server.

NOTE: This version of $0000.RPL does not support Western Digital

EtherCard PLUS adapters: 8003EBT and 8013EBT.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.