Resident Alarm Clock - ACLOCK v.1.2

(c) 1993 by MarkoTech, A. Danileiko

e-mail: danileiko@vtcc1.cc.vt.edu, adanil@g386bsd.first.gmd.de

ACLOCK is a small TSR program (784 bytes in memory) that shows real time

in display upper right corner, beeps every hour and plays music at the

selected time.

Usage:

ACLOCK /option -color alarm_time

Possible options are:

U to unload the program;

D to disable displayed time.

COLOR is hexadecimal attribute number. Default one is 1F.

ALARM_TIME is time in form hour:minute to be set as alarm time.

It is possible to operate with the program via INT 2F:

====================== INT 2F =======================

Loading check:

Input: AX = 9200h

If ACLOCK is leaded then

Output: AX = 1234h

ES = program CS

-----------------------------------------------------

Set attribute & disable flag:

Input: AX = 9201h

BH = attribute

BL = 1 to disable displaying or 0 to enable it

Output: None

-----------------------------------------------------

Set alarm time:

Input AX = 9202h

CH = hour

CL = minute

Output: None

-----------------------------------------------------