FIXCLOCK 2.0

Copyright (c) Irving Maron 1992-95  All Rights Reserved

Introduction

FIXCLOCK.EXE permits you to adjust the clock rate of the computer system real-

time clock (to speed-up or slow-down clocks that are not running accurately).

In addition FIXCLOCK 2.0 can be directed to automatically perform the clock

shifts required when changing between standard and daylight saving times

(with daylight saving time defined according to the U.S.A. definition,

i.e., starting at 2:00am on the first Sunday in April and ending at 2:00am

on the last Sunday in October). The actual clock resets take place at the

first boot-up after a time changeover has occurred.

First Time Installation Procedure

The clock rate adjustment is achieved by copying FIXCLOCK.EXE to a convenient

drive:path, and inserting the command [drive:][path]FIXCLOCK in the

AUTOEXEC.BAT file. The first boot-up after insertion of the FIXCLOCK command

will be interrupted by FIXCLOCK's installation procedure, which requires

entering two clock rate correction constants (in seconds per week). Subsequent

boot-ups are not interrupted. FIXCLOCK then checks the system clock at each

boot-up, and resets the clock, based upon the clock rate correction constants

that you entered during installation. The [drive:][path]FIXCLOCK command

inserted in the AUTOEXEC.BAT file should be placed with the setup commands,

and before any command that changes the directory from the root directory of

the boot drive, or that starts an application, or that starts WINDOWS.

The first correction constant entered, called "Real-time clock rate adjustment",

accounts for clock drift on normal days when the computer is turned on and

used. The second constant, called "Additional adjustment for non-operating

days", is provided for computers that exhibit clock drift, during periods of

a day or more when the computer is not booted-up, that is different from the

drift when the computer is operated. The additional adjustment is an

incremental correction to the first correction constant. If the additional

correction is not required, or if you are not sure whether or not it is

required, enter zero.

During the installation procedure, you are given the opportunity to accept or

reject an option for permitting FIXCLOCK to take charge of the  clock resets

at the boundaries between standard and daylight saving times.

Upgrade Installation

If you are upgrading from a previous version of FIXCLOCK, just replace the

old version with FIXCLOCK 2.0 and reboot. The bootup will be interrupted with

the installation dialogue, asking you to confirm or change your original

correction constants, and providing you the opportunity to accept or decline

the daylight-saving/standard clock reset option.

Changing Correction Constant

To change the correction constants or the daylight/standard clock-shift option

anytime after the initial installation, enter <[drive:][path]FIXCLOCK S> at

the DOS prompt from the root directory of the boot drive. Use this procedure

to take account of any remaining clock rate inaccuracy noted after initial

setup. When entering a new correction constant, use the previous constant

adjusted for the additional clock drift. For example, if you initially

observed that for periods when the computer is operated, the uncorrected clock

gained 30 seconds/week, then the installation procedure would have guided you

to enter -30 as the real-time clock rate adjustment (the negative sign for

slowdown). If you then note that the clock still gains 5 seconds every 2 weeks

(2.5 seconds/week), you should then change the constant to -32.5. (If the

clock had begun to lose 5 seconds/week after the initial installation, you

would have had to change the constant to -27.5 instead.)

The correction constants can be adjusted as many times as you like in order

to zero-in on the correct values for your clock. When you use the setup

correction process, the time since the previous correction (in number of weeks

and days) is noted at the top of the screen. This should assist you in comput-

ing corrected constants, if you always reset your clock after any correction

during the zeroing-in period.

Additional Note

If you enter the command <FIXCLOCK> from a directory which is not the root

directory of the boot drive, FIXCLOCK will go through its installation

procedure (if you permit) and create a configuration file, FIXCLOCK.INS, in

that directory. Then if you should enter <FIXCLOCK> again from that same

directory, FIXCLOCK will adjust the system clock according to the correction

constant recorded in the FIXCLOCK.INS file. (It will not correct more than

once each day, and not correct at all on any day that the INS file is created

or updated.) Therefor, if you are experimenting with FIXCLOCK and find

yourself being invited to an installation, you should decline. If you are too

polite to decline, then be sure to enter zero for the correction constant.

* * * *

FIXCLOCK 2.0 is freeware. Permission is granted for unrestricted noncommercial

use.

Irving Maron

Compuserve address: 76614,2666

e-mail address: 76614.2666@compuserve.com

September 3, 1995