Data Used For 3CX10000B7 and 3CX15000B7 Triodes

Name					3CX10KB7

Cooling					Forced Air

Max DC Plate Voltage			6500

DC Plate Voltage LO for EGG Curve	500

DC Plate Voltage HI for EGG Curve	1500

Amplification Factor			200

Cin  Cath-Grid Capacitance		79.4

Cout Plate-Grid Capacitance		34

Max Plate Diss.				10000

Max Grid  Diss.				25

8 IB Data Points			2,4,6,8,10,12,14,16

8 corresponding Grid Volts @ EB Low	38,63,82.5,102,120,138,160,185

8 corresponding Grid Volts @ EB High	30,52,70,88,107.5,125,145,165

8 corresponding Grid Currents @ EB Low	.05,.11,.29,.5,.88,1.19,1.75,2.25

8 corresponding Grid Currents @ EB High	.021,.026,.031,.035,.04,.044,.075,.166

Name					3CX15KB7

Cooling					Forced Air

Max DC Plate Voltage			6500

DC Plate Voltage LO for EGG Curve	600

DC Plate Voltage HI for EGG Curve	1800

Amplification Factor			200

Cin  Cath-Grid Capacitance		93

Cout Plate-Grid Capacitance		28

Max Plate Diss.				15000

Max Grid  Diss.				25

8 IB Data Points			3,5,7.5,10,12.5,15,17.5,20

8 corresponding Grid Volts @ EB Low	32,50,71,94,105,120,136,153

8 corresponding Grid Volts @ EB High	22,41,57,77.5,91,107,124,139

8 corresponding Grid Currents @ EB Low	.02,.025,.117,.2,.4,.7,1,2.78

8 corresponding Grid Currents @ EB High	0,.005,.01,.02,.031,.05,.068,.086