@echo off

echo �

echo CP3204  version 1.00  August 28, 1991

echo Fixed Disk Preparation and Partitioning Software.

echo Copyright (C) 1991 Dennis W. Person.  All rights reserved.

echo �

echo  ����������������������������������������������������������������������������Ŀ

echo  �  This DOC file is also the program BAT file.                               �

echo  �  You should print this DOC file before you try to run the program.         �

echo  �                                                                            �

echo  �  !!!!!!!!!! THIS PROGRAM CAN DESTROY DATA ON YOUR FIXED DISK. !!!!!!!!!!   �

echo  �                                                                            �

echo  �  Read and understand the information contained in this DOC file before     �

echo  �  you attempt to use this program to partition your fixed disk.             �

echo  ������������������������������������������������������������������������������

echo �

echo Press CTRL-C to terminate this program or . . .

pause

if [%4]==[]    goto GETFILE

if [%4]==[rem] goto GETFILE

if [%4]==[REM] goto GETFILE

goto HELP

:GETFILE

set OUTFILE=NUL

if [%3]==[]    goto OPTIONS

if [%3]==[NUL] goto OPTIONS

set OUTFILE=%3.LST

:OPTIONS

set OPTION=PREPARE

if [%2]==[p] goto GETDRIV

if [%2]==[P] goto GETDRIV

set OPTION=RESTORE

if [%2]==[r] goto GETDRIV

if [%2]==[R] goto GETDRIV

goto HELP

:GETDRIV

if [%1]==[/1] goto CONTINUE

if [%1]==[/2] goto CONTINUE

goto HELP

:CONTINUE

echo BEGIN - creation of CP3204.SCR

rem Check for existence of DEBUG script file.

if [%1]==[/1] if exist CP3204.1%2%4 ren CP3204.1%2%4 CP3204.SCR

if [%1]==[/2] if exist CP3204.2%2%4 ren CP3204.2%2%4 CP3204.SCR

rem Use existing DEBUG script file to partition drive to save processing time.

if exist CP3204.SCR goto RUN

goto BEGIN

rem

:HELP

cls

echo This program will partition the Conner Peripherals CP-3204 IDE fixed disk.

echo The CP-3204 physically has 1369 cylinders, 8 heads, and 38 sectors per track.

echo Since the number of cylinders is greater than the 1024 allowed by DOS, the

echo logical setup used is 544 cylinders, 15 heads, and 51 sectors per track.

echo The CP-3204 does the logical to physical translation.  The logical setup is

echo user defined and entered by the user using a BIOS that allows for a user

echo defined fixed disk type.  In my case, I patched the BIOS ROM to allow for the

echo user type I had chosen.

echo �

echo Be aware that DOS uses 1 less cylinder than specified in the fixed disk

echo parameter table.  This is because the last cylinder is considered the

echo diagnostic cylinder. If you wish to use the diagnostic cylinder for data,

echo increase the number of cylinders specified during the setup by 1.  Only do

echo this if you do not want the diagnostic cylinder available.  In my case,

echo I entered 545 as the number of cylinders during the setup.  This allows

echo me to use all 544 cylinders for data.

echo �

echo One additional note, the astute reader will have discovered that the

echo physical parameters give 416,176 sectors available on the CP-3204.  The

echo chosen logical parameters give 416,160 sectors available.  The chosen

echo parameters give the closest match to using all the available sectors.

echo �

echo Now, to see the command syntax to run this program . . .

pause

cls

echo �

echo Command:  CP3204 [/1�/2] [R�P] [list_file�NUL] [REM]

echo �

echo           /1  Select 1st fixed disk.

echo           /2  Select 2nd fixed disk.

echo            R  Restore fixed disk partitions.

echo            P  Prepare and partition fixed disk.

echo    list_file  Filename.LST to capture DEBUG output if desired.

echo               Do NOT enter the default .LST extension.

echo          NUL  Use NUL as the filename if no output file is desired but,

echo               the REM option is desired.

echo          REM  Remarks only {will still create DEBUG script file}.

echo               Useful to check script file before actual disk partitioning.

echo �

echo Options must be entered in the order shown and separated by a space.

goto EXIT

rem

:BEGIN

rem  Partition Program Code

rem  ����������������������

rem  The program code to write the partition sector, boot sector, and first FAT

rem  sector to fixed disk partition.  This code will be executed for each

rem  logical drive on the fixed disk.  The DEBUG script that follows will setup

rem  the partition sector and boot sector data for each logical drive.  The DEBUG

rem  script will then control execution of the program code.

rem  Clear program work area.

echo F 0100 07FF 0                                                             > CP3204.SCR

echo A 100                                                                    >> CP3204.SCR

echo MOV AX,CS   ; get CS register                                            >> CP3204.SCR

echo MOV DS,AX   ; initialize DS register                                     >> CP3204.SCR

echo MOV ES,AX   ; initialize DS register                                     >> CP3204.SCR

echo ;                                                                        >> CP3204.SCR

echo MOV AH,30   ; get function code to check DOS version                     >> CP3204.SCR

echo INT 21      ; call DOS routine to get version number                     >> CP3204.SCR

echo CMP AL,5    ; check for DOS version 5.00 or higher                       >> CP3204.SCR

echo JGE 112     ; JGE CONTINUE to partition fixed disk                       >> CP3204.SCR

echo ;             otherwise, incorrect DOS version                           >> CP3204.SCR

echo MOV AL,80   ; set ERRORLEVEL value to indicate incorrect DOS version     >> CP3204.SCR

echo JMP 130     ; JMP ERROR                                                  >> CP3204.SCR

echo ;                                                                        >> CP3204.SCR

echo ;[CONTINUE]                                                              >> CP3204.SCR

echo MOV SI,0003 ; initalize loop counter                                     >> CP3204.SCR

echo ;                                                                        >> CP3204.SCR

echo ;[LOOP]                                                                  >> CP3204.SCR

echo MOV AH,03   ; get function code to write to disk                         >> CP3204.SCR

rem If [P] option chosen, write the partition, boot, and 1st FAT sectors.

if [%2]==[P] echo MOV AL,03   ; get number of sectors to write                >> CP3204.SCR

if [%2]==[p] echo MOV AL,03   ; get number of sectors to write                >> CP3204.SCR

rem If [R] option chosen, only write the partition and boot sectors

if [%2]==[R] echo MOV AL,02   ; get number of sectors to write                >> CP3204.SCR

if [%2]==[r] echo MOV AL,02   ; get number of sectors to write                >> CP3204.SCR

echo MOV BX,0200 ; ES:BX points to beginning of data                          >> CP3204.SCR

echo MOV CX,0001 ; set CH to low order byte for cylinder number               >> CP3204.SCR

echo ;             set CL lower 6-bits to sector number                       >> CP3204.SCR

echo ;             set CL upper 2-bits to high order bits for cylinder number >> CP3204.SCR

echo MOV DH,00   ; get number for head (side)                                 >> CP3204.SCR

if [%1]==[/1] echo MOV DL,80   ; get drive number for 1st fixed disk          >> CP3204.SCR

if [%1]==[/2] echo MOV DL,81   ; get drive number for 2nd fixed disk          >> CP3204.SCR

echo INT 13      ; call BIOS routine which provides disk services             >> CP3204.SCR

echo JNC 0134    ; JNC WRITEOK if no error                                    >> CP3204.SCR

echo ;             otherwise, . . .                                           >> CP3204.SCR

echo MOV AH,0    ; get function code to reset disk system                     >> CP3204.SCR

echo INT 13      ; call BIOS routine which provides disk services             >> CP3204.SCR

echo DEC SI      ; decrement loop counter                                     >> CP3204.SCR

echo JNZ 0115    ; JNZ LOOP to try to write to disk again                     >> CP3204.SCR

echo ;             otherwise, . . .                                           >> CP3204.SCR

echo MOV AL,02   ; set ERRORLEVEL value to indicate error                     >> CP3204.SCR

echo ;           ; and . . .                                                  >> CP3204.SCR

echo ;[ERROR]      terminate program with ERRORLEVEL value                    >> CP3204.SCR

echo MOV AH,4C   ; get function code to teminate program                      >> CP3204.SCR

echo INT 21      ; and exit program                                           >> CP3204.SCR

echo ;                                                                        >> CP3204.SCR

echo ;[WRITEOK]                                                               >> CP3204.SCR

echo MOV AL,00   ; set ERRORLEVEL value to indicate success                   >> CP3204.SCR

echo MOV AH,4C   ; get function code to teminate program                      >> CP3204.SCR

echo INT 21      ; and exit program                                           >> CP3204.SCR

echo NOP                                                                      >> CP3204.SCR

echo #                                                                       >> CP3204.SCR

rem  Use CTRL-H (BACKSPACE) character for ^H shown on the line above.

echo   creation of Partition Program complete . . .

rem If 2nd fixed disk option chosen, do NOT put Partiton Boot Program on disk.

if [%1]==[/2] goto PUTABLE

rem

rem  Partition Boot Program

rem  ����������������������

rem  This is the partition boot program for the 1st fixed disk.  The program

rem  must be physically located on the 1st sector of the fixed disk, i.e.,

rem  cylinder 0, head 0, sector 1.

echo A 200                  >> CP3204.SCR

echo CLI                    >> CP3204.SCR

echo XOR AX,AX              >> CP3204.SCR

echo MOV DS,AX              >> CP3204.SCR

echo MOV ES,AX              >> CP3204.SCR

echo MOV SS,AX              >> CP3204.SCR

echo MOV SP,7C00            >> CP3204.SCR

echo MOV SI,SP              >> CP3204.SCR

echo STI                    >> CP3204.SCR

echo CLD                    >> CP3204.SCR

echo MOV DI,600             >> CP3204.SCR

echo MOV CX,100             >> CP3204.SCR

echo REPNZ                  >> CP3204.SCR

echo MOVSW                  >> CP3204.SCR

echo JMP 0:61D              >> CP3204.SCR

echo MOV SI,7BE             >> CP3204.SCR

echo MOV BL,4               >> CP3204.SCR

echo CMP BYTE PTR [SI],80   >> CP3204.SCR

echo JZ 235                 >> CP3204.SCR

echo CMP BYTE PTR [SI],0    >> CP3204.SCR

echo JNZ 248                >> CP3204.SCR

echo ADD SI,+10             >> CP3204.SCR

echo DEC BL                 >> CP3204.SCR

echo JNZ 222                >> CP3204.SCR

echo INT 18                 >> CP3204.SCR

echo MOV DX,[SI]            >> CP3204.SCR

echo MOV CX,[SI+2]          >> CP3204.SCR

echo MOV BP,SI              >> CP3204.SCR

echo ADD SI,+10             >> CP3204.SCR

echo DEC BL                 >> CP3204.SCR

echo JZ 25D                 >> CP3204.SCR

echo CMP BYTE PTR [SI],0    >> CP3204.SCR

echo JZ 23C                 >> CP3204.SCR

echo MOV SI,68B             >> CP3204.SCR

echo LODSB                  >> CP3204.SCR

echo CMP AL,0               >> CP3204.SCR

echo JZ 25B                 >> CP3204.SCR

echo PUSH SI                >> CP3204.SCR

echo MOV BX,7               >> CP3204.SCR

echo MOV AH,E               >> CP3204.SCR

echo INT 10                 >> CP3204.SCR

echo POP SI                 >> CP3204.SCR

echo JMP 24B                >> CP3204.SCR

echo JMP 25B                >> CP3204.SCR

echo MOV DI,3               >> CP3204.SCR

echo PUSH DI                >> CP3204.SCR

echo MOV BX,7C00            >> CP3204.SCR

echo MOV AX,201             >> CP3204.SCR

echo INT 13                 >> CP3204.SCR

echo POP DI                 >> CP3204.SCR

echo JNC 278                >> CP3204.SCR

echo XOR AX,AX              >> CP3204.SCR

echo INT 13                 >> CP3204.SCR

echo DEC DI                 >> CP3204.SCR

echo JNZ 260                >> CP3204.SCR

echo MOV SI,6A5             >> CP3204.SCR

echo JMP 24B                >> CP3204.SCR

echo MOV SI,6C6             >> CP3204.SCR

echo MOV DI,7DFE            >> CP3204.SCR

echo CMP WORD PTR [DI],AA55 >> CP3204.SCR

echo JNZ 24B                >> CP3204.SCR

echo MOV SI,BP              >> CP3204.SCR

echo JMP 0:7C00             >> CP3204.SCR

echo #                     >> CP3204.SCR

rem  Use CTRL-H (BACKSPACE) character for ^H shown on the line above.

echo E 28B D A "Invalid Partition Table" 0        >> CP3204.SCR

echo E 2A5 D A "Error Loading Operating System" 0 >> CP3204.SCR

echo E 2C6 D A "Missing Operating System" 0       >> CP3204.SCR

rem

:PUTABLE

rem  The master partition table also resides on the 1st sector following the

rem  master boot program.  The following line is to initialize the partition

rem  table.  See below for information on the partition table.

echo F 2E1 3FD 0 >> CP3204.SCR

rem  The two bytes below MUST be as shown in order for the BIOS the recognize

rem  the fixed disk.  The bytes represent that the partition sector is valid.

rem  Write signature to indicate valid partition sector.

echo E 3FE 55 AA >> CP3204.SCR

echo   creation of partition sector complete . . .

rem          F I X E D   D I S K   D R I V E   P A R T I T I O N   T A B L E

rem  �������������������������������������������������������������������������������

rem  �����������������������������������������������������������������������������Ŀ

rem  �      �    � Starting Location  �  Ending Location   �  Relative � Number of �

rem  �System�Boot�Side Cylinder Sector�Side Cylinder Sector�  Sectors  �  Sectors  �

rem  �����������������������������������������������������������������������������Ĵ

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �������������������������������������������������������������������������������

rem  NOTE:  The partition table allows a maximum of four (4) partitions.

rem         See the following pages for sector offset to each item in the table.

rem         Only one partition can be marked as the boot partition with the code

rem         80 hex.  The other partitions MUST contain 0 as the boot byte.

rem         Invalid codes will produce the "Invalid Partition Table" message and

rem         prevent your system from booting.

rem  For a fixed disk with a PRIMARY partition, FDISK will reserve the entire first

rem  track.  Programs such as Ontrack's Disk Manager use those reserved sectors to

rem  store their own disk information.  So, if you are using one of those programs,

rem  do NOT use this program.

rem  For a fixed disk with only an EXTENDed partition, FDISK will reserve the entire

rem  first cylinder of the disk for the Master Partition Table.  Each logical drive

rem  in an EXTENDed partition also requires a partition table.  So, FDISK will also

rem  reserve the first track of each logical drive for the partition table.

rem  Since the partition table only requires 1 sector, DOS is wasting your hard disk

rem  space.  The information on the following pages will allow you to reclaim the

rem  disk space that DOS does not use.  Remember, the procedure can only be used

rem  if your fixed disk does NOT require a program such as Ontrack's Disk Manager.

rem  Additional information regarding systems with 2 fixed disks is shown on the

rem  following page.  The information will allow you to decide whether to partition

rem  the 2nd fixed disk with a PRIMARY partition and an EXTENDEDed partition OR with

rem  only an EXTENDEDed partition.  I chose to partition the 2nd fixed disk with only

rem  an EXTENDEDed partition.

rem

rem  Shown below are the drive letters assigned by DOS to the logical drives on each

rem  fixed disk.  Note that the PRIMARY partitions have priority in drive letter

rem  assignments.  The PRIMARY partition on the 1st fixed disk is always drive C:.

rem  The PRIMARY partition on the 2nd fixed disk is always drive D:.  Additional

rem  logical drives are then assigned to the 1st fixed disk, then to the 2nd fixed

rem  disk.

rem       PRIMARY and EXTENDed partition on 1st fixed disk

rem       ������������������������������������������������

rem       PRIMARY partition  �� C:

rem       EXTENDed partition �� D: E: F: , etc.

rem       Extended partition on 2nd fixed disk

rem       ������������������������������������

rem       EXTENDed partition �� G: H: I: J: , etc.

rem       PRIMARY and EXTENDed partition on 1st fixed disk

rem       ������������������������������������������������

rem       PRIMARY partition  �� C:

rem       EXTENDed partition �� E: F: G: , etc.

rem       PRIMARY and EXTENDed partition on 2nd fixed disk

rem       ������������������������������������������������

rem       PRIMARY partition  �� D:

rem       EXTENDed partition �� H: I: J: , etc.

rem

rem  Shown below is the location of each byte in the partition table.  A maximum

rem  of four partitions may be defined.

rem  sector

rem  offset  Parameters for 1st Partition:

rem  ������  �����������������������������

rem   01BE   00          ; partition status (active [80] or non-active [00])

rem   01BF   00          ; starting side

rem   01C0   00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem   01C1   00          ; starting cylinder (lower byte)

rem   01C2   00          ; system (valid codes listed later in document)

rem   01C3   00          ; ending side

rem   01C4   00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem   01C5   00          ; ending cylinder (lower byte)

rem   01C6   00 00 00 00 ; relative sectors

rem   01CA   00 00 00 00 ; number of sectors

rem  sector

rem  offset  Parameters for 2nd Partition:

rem  ������  �����������������������������

rem   01CE   00          ; partition status (active [80] or non-active [00])

rem   01CF   00          ; starting side

rem   01D0   00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem   01D1   00          ; starting cylinder (lower byte)

rem   01D2   00          ; system (valid codes listed later in document)

rem   01D3   00          ; ending side

rem   01D4   00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem   01D5   00          ; ending cylinder (lower byte)

rem   01D6   00 00 00 00 ; relative sectors

rem   01DA   00 00 00 00 ; number of sectors

rem  sector

rem  offset  Parameters for 3rd Partition:

rem  ������  �����������������������������

rem   01DE   00          ; partition status (active [80] or non-active [00])

rem   01DF   00          ; starting side

rem   01E0   00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem   01E1   00          ; starting cylinder (lower byte)

rem   01E2   00          ; system (valid codes listed later in document)

rem   01E3   00          ; ending side

rem   01E4   00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem   01E5   00          ; ending cylinder (lower byte)

rem   01E6   00 00 00 00 ; relative sectors

rem   01EA   00 00 00 00 ; number of sectors

rem  sector

rem  offset  Parameters for 4th Partition:

rem  ������  �����������������������������

rem   01EE   00          ; partition status (active [80] or non-active [00])

rem   01EF   00          ; starting side

rem   01F0   00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem   01F1   00          ; starting cylinder (lower byte)

rem   01F2   00          ; system (valid codes listed later in document)

rem   01F3   00          ; ending side

rem   01F4   00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem   01F5   00          ; ending cylinder (lower byte)

rem   01F6   00 00 00 00 ; relative sectors

rem   01FA   00 00 00 00 ; number of sectors

rem  See the following page for an explanation of each item in the partition

rem  table.

rem  DOS numbers sides (heads) and cylinders starting with 0.  Sectors are

rem  numbered starting with 1.

rem  Note:  The upper 2-bits of the cylinder number is contained in the upper

rem         2-bits of the byte for the sector number.  This means the maximum

rem         sector number is 63 and the maximum cylinder number is 1023.

rem         Remember this when you are ready to prepare the partition tables

rem         for your fixed disk.  Study the partition tables shown later in

rem         this document.

rem

rem  The PARTITION STATUS byte is actually the drive number select byte.

rem  Since the 1st hard disk is assigned the drive number 80, the partition

rem  with the status byte equal to 80 will be the boot partition.  Only one

rem  partition can be the active (boot) partition.  By changing the PARTITION

rem  STATUS byte and the partition boot program active partition byte to 81,

rem  you could boot from the 2nd fixed disk instead of the 1st fixed disk.

rem  For a DOS partition, the STARTING parameters define the physical location

rem  of the logical drive boot sector.  For an EXTENDed partition, the STARTING

rem  parameters define the physical location of the beginning of the EXTENDed

rem  partition.  This will correspond to the physical location of the partition

rem  table for the first logical drive in the EXTENDed partition.

rem  The SYSTEM byte indicates the type of partition.  Valid SYSTEM bytes are

rem  listed on the page before the partition table definition for drive C: shown

rem  later in this DOC file.

rem  The ENDING parameters define the physical location of the end of the

rem  partition for both a DOS partition and an EXTENDed partition.

rem  For a DOS partition, RELATIVE SECTORS is the number of physical sectors

rem  in the partition before the logical drive boot sector.  This number will

rem  correspond to the number of hidden sectors in the BIOS Parameter Table.

rem  For the fixed disk EXTENDed partition, RELATIVE SECTORS is the number of

rem  physical sectors in the DOS partition.  This corresponds to the number of

rem  physical sectors before the beginning of the EXTENDed partition.  For an

rem  EXTENDed partition for a logical drive, RELATIVE SECTORS is the number of

rem  physical sectors in the fixed disk EXTENDed partition that are before the

rem  partition sector for the logical drive.

rem  For a DOS partition, NUMBER OF SECTORS is the number of sectors allocated

rem  to the logical drive.  For the fixed disk EXTENDed partition, NUMBER OF

rem  SECTORS is the number of sectors allocated to the EXTENDed partition.  For

rem  an EXTENDed partition for a logical drive, NUMBER OF SECTORS is the number

rem  of sectors allocated to the next logical drive.

rem  The partition table for each logical drive I have setup on my fixed disks

rem  is shown later in this DOC file.  Study and understand the partition tables

rem  before you try to setup the partition tables for your fixed disk.  The

rem  partition tables are what allow the fixed disk to be divided into logical

rem  drives.  Be careful as you setup the partition tables on your fixed disk.

rem  A few additional items and caveats.  You MUST use DOS version 5 or higher

rem  in order for the partition structure described in this document to be

rem  recognized by DOS.  Also, FDISK does not recognize the partition structure

rem  I have setup on my fixed disks.  But, since the partition structure is

rem  known, FDISK is NOT really needed.  In fact, the system will hang if I try

rem  to run FDISK.

rem

:BOOT

rem                           D O S   B O O T   S E C T O R

rem  �������������������������������������������������������������������������������

rem  The JMP instruction is to end of BIOS Parameter Block.  Space is prvided for

rem  a 3-byte JMP instruction.  DOS reads this instruction to determine the length

rem  of the BIOS Parameter Block.  So, this instruction CANNOT be eliminated from

rem  the logical drives in EXTENDed partitions.

echo A 400   >> CP3204.SCR

echo JMP 43E >> CP3204.SCR

echo NOP     >> CP3204.SCR

echo #      >> CP3204.SCR

rem  Use CTRL-H (BACKSPACE) character for ^H shown on the line above.

rem  sector

rem  offset         B I O S   P A R A M E T E R   B L O C K   D A T A

rem  �������������������������������������������������������������������������������

rem   0003                            OEM ID: 8-characters

rem   000B                  Bytes per sector: 0..65,535 (2-bytes)

rem   000D               Sectors per cluster: 0..255    (1-byte)

rem   000E     Reserved sectors at beginning: 0..65,535 (2-bytes)

rem   0010                        FAT Copies: 0..255    (1-byte)

rem   0011            Root directory entries: 0..65,535 (2-bytes)

rem   0013             Total sectors on disk: 0..65,535 (2-bytes)

rem   0015             Media descriptor byte: 1-byte (see table below)

rem   0016                   Sectors per FAT: 0..65,535 (2-bytes)

rem   0018                 Sectors per track: 0..65,535 (2-bytes)

rem   001A                             Sides: 0..65,535 (2-bytes)

rem   001C            Special hidden sectors: 0..4,294,967,295 (4-bytes)

rem  Extended BIOS Parameter Block supported by DOS 4 and DOS 5.  This is where

rem  DOS 'breaks' the 32Mb disk barrier but leaves the 64K cluster barrier.

rem  �������������������������������������������������������������������������������

rem   0020       Big total number of sectors: 0..    (4-bytes)

rem   0024             Physical drive number: 0..255 (1-byte)

rem   0025                           unknown: 0..255 (1-byte)

rem   0026    Extended Boot Record Signature: 29 hex (1-byte)

rem   0027              Volume Serial Number: 4-byte time & date code

rem   002B                      Volume Label: 11-characters

rem   0036                    File System ID: 8-characters (usually FAT12 or FAT16)

rem              V A L I D   M E D I A   D E S C R I P T O R   B Y T E S

rem  �������������������������������������������������������������������������������

rem   F8 �� Fixed Disk or RAM disk

rem   ?? �� 3� inch, 2 sides, 80 tracks, 36 sectors per track �� 2.88M

rem   F0 �� 3� inch, 2 sides, 80 tracks, 18 sectors per track �� 1.44M

rem   F9 �� 3� inch, 2 sides, 80 tracks,  9 sectors per track �� 720K

rem   F9 �� 5� inch, 2 sides, 80 tracks, 15 sectors per track �� 1.2M

rem   FC �� 5� inch, 1 side,  40 tracks,  9 sectors per track �� 180K

rem   FD �� 5� inch, 2 sides, 40 tracks,  9 sectors per track �� 360K

rem   FE �� 5� inch, 1 side,  40 tracks,  8 sectors per track �� 160K

rem   FF �� 5� inch, 2 sides, 40 tracks,  8 sectors per track �� 320K

rem   Values above are hex codes.

rem  NOTE:  The descriptor byte F9 is used for both the 5� inch 1.2Mb floppy

rem         and the 3� inch 720Kb floppy.  (I don't know why DOS does that.)

rem         The descriptor byte for the 2.88Mb floppy supported by DOS 5.00

rem         is not known.

rem

rem  The specific BIOS Parameter Block for the chosen logical disk size is

rem  shown below.  Note the cluster (allocation unit) size of 1K, 2 sectors per

rem  cluster.  Although this increases the FAT size, the data area lost for

rem  files will be made up by reduced waste at the end of files.

rem  The number of clusters on the disk cannot exceed 65,534.  This is because

rem  the cluster number is represented by 16-bits.  Since the first four bytes

rem  of the FAT are reserved for the media description, the first cluster

rem  available is cluster 2.  So, DOS will adjust the number of sectors per

rem  cluster to 2, 4, 8, etc. to keep to number of clusters less than 65,534.

rem  Use the following formulas to determine the the number of FAT sectors

rem  required for each FAT.  DOS requires that 2 FAT's exist on all fixed disks.

rem  You cannot eliminate a FAT to gain data space.

rem                     total_data_sectors      2 bytes_per_cluster (FAT16)

rem  sectors_per_FAT =  �������������������  X  ��������������������

rem                     sectors_per_cluster     512 bytes_per_sector

rem  total_data_sectors = total_disk_sectors - reserved_sectors - root_directory_sectors

rem                       - 2 X sectors_per_FAT

rem                                                    32 bytes_per_entry

rem  root_directory_sectors = root_directory_entries X ��������������������

rem                                                    512 bytes_per_sector

rem  sector

rem  offset               B I O S   P A R A M E T E R   B L O C K   D A T A

rem  �������������������������������������������������������������������������������

rem   0003                            OEM ID: DWPerson

rem   000B                  Bytes per sector: 512           ; all DOS disks use 512

rem   000D               Sectors per cluster: 2             ; 1K cluster size

rem   000E     Reserved sectors at beginning: 1             ; number of boot sectors

rem   0010                        FAT Copies: 2             ; must be 2

rem   0011            Root directory entries: 512           ; default value

rem   0013             Total sectors on disk: 0             ; 0 indicates BIGDOS

rem   0015             Media descriptor byte: F8 hex        ; indicates fixed disk

rem   0016                   Sectors per FAT: 203           ; use formulas shown above

rem   0018                 Sectors per track: 51            ; fixed disk parameter

rem   001A                             Sides: 15            ; fixed disk parameter

rem   001C            Special hidden sectors: 1             ; partition sector

rem   0020       Big total number of sectors: 104039        ; logical disk size

rem   0024             Physical drive number: 128           ; 1st fixed disk number

rem   0025                           unknown: 0             ; reserved parameter byte

rem   0026    Extended Boot Record Signature: 29 hex        ; significance unknown

rem   0027              Volume Serial Number: 1991-0831 hex ; choosen serial number

rem   002B                      Volume Label: FDISK1 VOL1   ; same as in directory

rem   0036                    File System ID: FAT16         ; use 2 bytes in FAT

rem  The actual BIOS Parameter Block data.

rem  Remember, data order is low-byte, then high-byte.

echo E 403 "DWPerson"       >> CP3204.SCR

echo E 40B 00 02            >> CP3204.SCR

echo E 40D 02               >> CP3204.SCR

echo E 40E 01 00            >> CP3204.SCR

echo E 410 02               >> CP3204.SCR

echo E 411 00 02            >> CP3204.SCR

echo E 413 00 00            >> CP3204.SCR

echo E 415 F8               >> CP3204.SCR

echo E 416 CB 00            >> CP3204.SCR

echo E 418 33 00            >> CP3204.SCR

echo E 41A 0F 00            >> CP3204.SCR

echo E 41C 01 00 00 00      >> CP3204.SCR

echo E 420 67 96 01 00      >> CP3204.SCR

if [%1]==[/1] echo E 424 80 >> CP3204.SCR

if [%1]==[/2] echo E 424 81 >> CP3204.SCR

echo E 425 00               >> CP3204.SCR

echo E 426 29               >> CP3204.SCR

echo E 427 31 08 91 19      >> CP3204.SCR

echo E 42B "FDISK"          >> CP3204.SCR

if [%1]==[/1] echo E 430 "1" >> CP3204.SCR

if [%1]==[/2] echo E 430 "2" >> CP3204.SCR

echo E 431 " VOL"            >> CP3204.SCR

echo E 435 "?"               >> CP3204.SCR

echo E 436 "FAT16   "        >> CP3204.SCR

rem

if [%1]==[/2] goto PUTSIGN

rem  DOS Boot Program

rem  ����������������

rem  This is the DOS boot program which resides as the first logical sector of

rem  of the boot disk.  The boot sector includes the BIOS Parameter Block (BPB)

rem  at the beginning of the boot sector.  At boot, DOS reads the BPB of each

rem  logical disk to determine the logical parameters of the disk.

echo A 43E                  >> CP3204.SCR

echo CLI                    >> CP3204.SCR

echo XOR AX,AX              >> CP3204.SCR

echo MOV ES,AX              >> CP3204.SCR

echo MOV SS,AX              >> CP3204.SCR

echo MOV SP,7C00            >> CP3204.SCR

echo MOV BX,78              >> CP3204.SCR

echo SS: LDS SI,[BX]        >> CP3204.SCR

echo PUSH DS                >> CP3204.SCR

echo PUSH SI                >> CP3204.SCR

echo PUSH SS                >> CP3204.SCR

echo PUSH BX                >> CP3204.SCR

echo MOV DI,7C3E            >> CP3204.SCR

echo MOV CX,B               >> CP3204.SCR

echo CLD                    >> CP3204.SCR

echo REPZ                   >> CP3204.SCR

echo MOVSB                  >> CP3204.SCR

echo PUSH ES                >> CP3204.SCR

echo POP DS                 >> CP3204.SCR

echo MOV BYTE PTR [DI-2],F  >> CP3204.SCR

echo MOV CX,[7C18]          >> CP3204.SCR

echo MOV [DI-7],CL          >> CP3204.SCR

echo MOV [BX+2],AX          >> CP3204.SCR

echo MOV WORD PTR [BX],7C3E >> CP3204.SCR

echo STI                    >> CP3204.SCR

echo INT 13                 >> CP3204.SCR

echo JC 4ED                 >> CP3204.SCR

echo XOR AX,AX              >> CP3204.SCR

echo CMP [7C13],AX          >> CP3204.SCR

echo JZ 484                 >> CP3204.SCR

echo MOV CX,[7C13]          >> CP3204.SCR

echo MOV [7C20],CX          >> CP3204.SCR

echo MOV AL,[7C10]          >> CP3204.SCR

echo MUL WORD PTR [7C16]    >> CP3204.SCR

echo ADD AX,[7C1C]          >> CP3204.SCR

echo ADC DX,[7C1E]          >> CP3204.SCR

echo ADD AX,[7C0E]          >> CP3204.SCR

echo ADC DX,0               >> CP3204.SCR

echo MOV [7C50],AX          >> CP3204.SCR

echo MOV [7C52],DX          >> CP3204.SCR

echo MOV [7C49],AX          >> CP3204.SCR

echo MOV [7C4B],DX          >> CP3204.SCR

echo MOV AX,20              >> CP3204.SCR

echo MUL WORD PTR [7C11]    >> CP3204.SCR

echo MOV BX,[7C0B]          >> CP3204.SCR

echo ADD AX,BX              >> CP3204.SCR

echo DEC AX                 >> CP3204.SCR

echo DIV BX                 >> CP3204.SCR

echo ADD [7C49],AX          >> CP3204.SCR

echo ADC WORD PTR [7C4B],0  >> CP3204.SCR

echo MOV BX,500             >> CP3204.SCR

echo MOV DX,[7C52]          >> CP3204.SCR

echo MOV AX,[7C50]          >> CP3204.SCR

echo CALL 560               >> CP3204.SCR

echo JC 4ED                 >> CP3204.SCR

echo MOV AL,1               >> CP3204.SCR

echo CALL 581               >> CP3204.SCR

echo JC 4ED                 >> CP3204.SCR

echo MOV DI,BX              >> CP3204.SCR

echo MOV CX,B               >> CP3204.SCR

echo MOV SI,7DE6            >> CP3204.SCR

echo REPZ                   >> CP3204.SCR

echo CMPSB                  >> CP3204.SCR

echo JNZ 4ED                >> CP3204.SCR

echo LEA DI,[BX+20]         >> CP3204.SCR

echo MOV CX,B               >> CP3204.SCR

echo REPZ                   >> CP3204.SCR

echo CMPSB                  >> CP3204.SCR

echo JZ 505                 >> CP3204.SCR

echo MOV SI,7D9E            >> CP3204.SCR

echo CALL 552               >> CP3204.SCR

echo XOR AX,AX              >> CP3204.SCR

echo INT 16                 >> CP3204.SCR

echo POP SI                 >> CP3204.SCR

echo POP DS                 >> CP3204.SCR

echo POP [SI]               >> CP3204.SCR

echo POP [SI+2]             >> CP3204.SCR

echo INT 19                 >> CP3204.SCR

echo POP AX                 >> CP3204.SCR

echo POP AX                 >> CP3204.SCR

echo POP AX                 >> CP3204.SCR

echo JMP 4ED                >> CP3204.SCR

echo MOV AX,[BX+1A]         >> CP3204.SCR

echo DEC AX                 >> CP3204.SCR

echo DEC AX                 >> CP3204.SCR

echo MOV BL,[7C0D]          >> CP3204.SCR

echo XOR BH,BH              >> CP3204.SCR

echo MUL BX                 >> CP3204.SCR

echo ADD AX,[7C49]          >> CP3204.SCR

echo ADC DX,[7C4B]          >> CP3204.SCR

echo MOV BX,700             >> CP3204.SCR

echo MOV CX,3               >> CP3204.SCR

echo PUSH AX                >> CP3204.SCR

echo PUSH DX                >> CP3204.SCR

echo PUSH CX                >> CP3204.SCR

echo CALL 560               >> CP3204.SCR

echo JC 500                 >> CP3204.SCR

echo MOV AL,1               >> CP3204.SCR

echo CALL 581               >> CP3204.SCR

echo POP CX                 >> CP3204.SCR

echo POP DX                 >> CP3204.SCR

echo POP AX                 >> CP3204.SCR

echo JC 4ED                 >> CP3204.SCR

echo ADD AX,1               >> CP3204.SCR

echo ADC DX,0               >> CP3204.SCR

echo ADD BX,[7C0B]          >> CP3204.SCR

echo LOOP 520               >> CP3204.SCR

echo MOV CH,[7C15]          >> CP3204.SCR

echo MOV DL,[7C24]          >> CP3204.SCR

echo MOV BX,[7C49]          >> CP3204.SCR

echo MOV AX,[7C4B]          >> CP3204.SCR

echo JMP 70:0               >> CP3204.SCR

echo LODSB                  >> CP3204.SCR

echo OR AL,AL               >> CP3204.SCR

echo JZ 580                 >> CP3204.SCR

echo MOV AH,E               >> CP3204.SCR

echo MOV BX,7               >> CP3204.SCR

echo INT 10                 >> CP3204.SCR

echo JMP 552                >> CP3204.SCR

echo CMP DX,[7C18]          >> CP3204.SCR

echo JNC 57F                >> CP3204.SCR

echo DIV WORD PTR [7C18]    >> CP3204.SCR

echo INC DL                 >> CP3204.SCR

echo MOV [7C4F],DL          >> CP3204.SCR

echo XOR DX,DX              >> CP3204.SCR

echo DIV WORD PTR [7C1A]    >> CP3204.SCR

echo MOV [7C25],DL          >> CP3204.SCR

echo MOV [7C4D],AX          >> CP3204.SCR

echo CLC                    >> CP3204.SCR

echo RET                    >> CP3204.SCR

echo STC                    >> CP3204.SCR

echo RET                    >> CP3204.SCR

echo MOV AH,2               >> CP3204.SCR

echo MOV DX,[7C4D]          >> CP3204.SCR

echo MOV CL,6               >> CP3204.SCR

echo SHL DH,CL              >> CP3204.SCR

echo OR DH,[7C4F]           >> CP3204.SCR

echo MOV CX,DX              >> CP3204.SCR

echo XCHG CH,CL             >> CP3204.SCR

echo MOV DL,[7C24]          >> CP3204.SCR

echo MOV DH,[7C25]          >> CP3204.SCR

echo INT 13                 >> CP3204.SCR

echo RET                    >> CP3204.SCR

echo #                     >> CP3204.SCR

rem  Use CTRL-H (BACKSPACE) character for ^H shown on the line above.

echo E 59E D A "Non-system disk or disk error"        >> CP3204.SCR

echo E 5BD D A "Replace and press any key when ready" >> CP3204.SCR

echo E 5E3 D A 0                                      >> CP3204.SCR

rem  Define DOS system file names (11-characters each)

echo E 5E6 "IO      SYS" >> CP3204.SCR

echo E 5F1 "MSDOS   SYS" >> CP3204.SCR

:PUTSIGN

rem  The two bytes below MUST be as shown in order for the BIOS the recognize

rem  the fixed disk.  The bytes represent that the boot sector is valid.

rem  Write signature to indicate valid boot sector.

echo E 5FE 55 AA >> CP3204.SCR

echo   creation of DOS boot sector complete . . .

rem  Write FAT ID bytes to 1st FAT sector.

echo E 600 F8 FF FF FF >> CP3204.SCR

rem  �������������������������������������������������������������������������������

rem  Shown on the following pages is the partition table for each logical drive.

rem  The partition table for each logical drive fits nicely on one page.  There

rem  are four (4) partition tables for the 1st fixed disk and five (5) partition

rem  tables for the 2nd fixed disk.  The entire 2nd fixed disk is setup as an

rem  EXTENDed partition and so requires a Master Partition Table.  An explanation

rem  is contained on the page where the Master Partition Table for the 2nd fixed

rem  disk is defined.

rem  By studying the partition tables, you should be able to create a DOC file

rem  to partition your particular fixed disk.  Use this DOC file as a guide and

rem  work carefully.  Remember to backup your fixed disk before using the program

rem  to partition your fixed disk.

rem  Shown below for reference is the list of valid system ID codes which can

rem  be entered into the partition table.  The SYSTEM codes were determined using

rem  the Norton Utilites v5.0 DISKEDIT program to look at the fixed disk partition

rem  tables.

rem                      V A L I D   S Y S T E M   I D   C O D E

rem  �������������������������������������������������������������������������������

rem   00 �� indicates an unused partition

rem   01 �� DOS-12

rem   02 �� XENIX

rem   04 �� DOS-16

rem   05 �� EXTEND

rem   06 �� BIGDOS

rem   07 �� HPFS

rem   08 �� Split

rem   50 �� DM

rem   56 �� GB

rem   61 �� NOVELL

rem   63 �� PCIX

rem   64 �� CP/M

rem   75 �� Speed

rem   DB �� BBT

rem   FF �� 386/ix

rem  NOTE:  The above values are in hex.

rem

:DISK1

rem If 2nd fixed disk option was chosen,

rem jump to partition information for 2nd fixed disk.

if [%1]==[/2] goto DISK2

rem          F I X E D   D I S K   D R I V E   P A R T I T I O N   T A B L E

rem  �������������������������������������������������������������������������������

rem  FDISK1 VOL1 �� logical drive C:

rem  �����������������������������������������������������������������������������Ŀ

rem  �      �    � Starting Location  �  Ending Location   �  Relative � Number of �

rem  �System�Boot�Side Cylinder Sector�Side Cylinder Sector�  Sectors  �  Sectors  �

rem  �����������������������������������������������������������������������������Ĵ

rem  �BIGDOS� Yes�  0       0      2  � 14     135     51  �        1  �   104039  �

rem  �EXTEND� No �  0     136      1  � 14     543     51  �   104040  �   312120  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �������������������������������������������������������������������������������

rem  Listed below is the RELATIVE location of each byte in the partition table.

rem  The 16-byte list is repeated for each line in the partition table.  A

rem  maximum of 4 partitions may be defined. The partition table begins at

rem  sector offset 01BE.

rem  0000  00          ; partition status (active [80] or non-active [00])

rem  0001  00          ; starting side

rem  0002  00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem  0003  00          ; starting cylinder (lower byte)

rem  0004  00          ; system (see information on previous page for valid codes)

rem  0005  00          ; ending side

rem  0006  00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem  0007  00          ; ending cylinder (lower byte)

rem  0008  00 00 00 00 ; relative sectors

rem  000C  00 00 00 00 ; number of sectors

rem  Finally, the actual partition table data.

rem  Remember, data order is low-byte, then high-byte.

echo E 3BE 80          >> CP3204.SCR

echo E 3BF 00          >> CP3204.SCR

echo E 3C0 02          >> CP3204.SCR

echo E 3C1 00          >> CP3204.SCR

echo E 3C2 06          >> CP3204.SCR

echo E 3C3 0E          >> CP3204.SCR

echo E 3C4 33          >> CP3204.SCR

echo E 3C5 87          >> CP3204.SCR

echo E 3C6 01 00 00 00 >> CP3204.SCR

echo E 3CA 67 96 01 00 >> CP3204.SCR

echo E 3CE 00          >> CP3204.SCR

echo E 3CF 00          >> CP3204.SCR

echo E 3D0 01          >> CP3204.SCR

echo E 3D1 88          >> CP3204.SCR

echo E 3D2 05          >> CP3204.SCR

echo E 3D3 0E          >> CP3204.SCR

echo E 3D4 B3          >> CP3204.SCR

echo E 3D5 1F          >> CP3204.SCR

echo E 3D6 68 96 01 00 >> CP3204.SCR

echo E 3DA 38 C3 04 00 >> CP3204.SCR

rem  Update volume label in BIOS Parameter Block.

echo E 435 "1" >> CP3204.SCR

rem  If REM option was chosen, don't execute partition program.  Otherwise,

rem  go write partition sector, boot sector, and 1st FAT sector.

echo %4 G=100 138 >> CP3204.SCR

rem  Clear the Partition Boot Program and DOS Boot Program.

rem  They are NOT needed on logical drives in EXTENDed partitions.

echo F 200 2E0 0 >> CP3204.SCR

echo F 43E 5FD 0 >> CP3204.SCR

echo   creation of partition table for 1st logical drive complete . . .

rem

rem          F I X E D   D I S K   D R I V E   P A R T I T I O N   T A B L E

rem  �������������������������������������������������������������������������������

rem  FDISK1 VOL2 �� logical drive D:

rem  �����������������������������������������������������������������������������Ŀ

rem  �      �    � Starting Location  �  Ending Location   �  Relative � Number of �

rem  �System�Boot�Side Cylinder Sector�Side Cylinder Sector�  Sectors  �  Sectors  �

rem  �����������������������������������������������������������������������������Ĵ

rem  �BIGDOS� No �  0     136      2  � 14     271     51  �        1  �   104039  �

rem  �EXTEND� No �  0     272      1  � 14     407     51  �   104040  �   104040  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �������������������������������������������������������������������������������

rem  Listed below is the RELATIVE location of each byte in the partition table.

rem  The 16-byte list is repeated for each line in the partition table.  A

rem  maximum of 4 partitions may be defined. The partition table begins at

rem  sector offset 01BE.

rem  0000  00          ; partition status (active [80] or non-active [00])

rem  0001  00          ; starting side

rem  0002  00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem  0003  00          ; starting cylinder (lower byte)

rem  0004  00          ; system (see information on previous page for valid codes)

rem  0005  00          ; ending side

rem  0006  00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem  0007  00          ; ending cylinder (lower byte)

rem  0008  00 00 00 00 ; relative sectors

rem  000C  00 00 00 00 ; number of sectors

rem  Finally, the actual partition table data.

rem  Remember, data order is low-byte, then high-byte.

echo E 3BE 00          >> CP3204.SCR

echo E 3BF 00          >> CP3204.SCR

echo E 3C0 02          >> CP3204.SCR

echo E 3C1 88          >> CP3204.SCR

echo E 3C2 06          >> CP3204.SCR

echo E 3C3 0E          >> CP3204.SCR

echo E 3C4 73          >> CP3204.SCR

echo E 3C5 0F          >> CP3204.SCR

echo E 3C6 01 00 00 00 >> CP3204.SCR

echo E 3CA 67 96 01 00 >> CP3204.SCR

echo E 3CE 00          >> CP3204.SCR

echo E 3CF 00          >> CP3204.SCR

echo E 3D0 41          >> CP3204.SCR

echo E 3D1 10          >> CP3204.SCR

echo E 3D2 05          >> CP3204.SCR

echo E 3D3 0E          >> CP3204.SCR

echo E 3D4 73          >> CP3204.SCR

echo E 3D5 97          >> CP3204.SCR

echo E 3D6 68 96 01 00 >> CP3204.SCR

echo E 3DA 68 96 01 00 >> CP3204.SCR

rem  Update partition program code to write data on correct cylinder.

echo E 11D 01 88 >> CP3204.SCR

rem  Update volume label in BIOS Parameter Block.

echo E 435 "2" >> CP3204.SCR

rem  If REM option was chosen, don't execute partition program.  Otherwise,

rem  go write partition sector, boot sector, and 1st FAT sector.

echo %4 G=100 138 >> CP3204.SCR

echo   creation of partition table for 2nd logical drive complete . . .

rem

rem          F I X E D   D I S K   D R I V E   P A R T I T I O N   T A B L E

rem  �������������������������������������������������������������������������������

rem  FDISK1 VOL3 �� logical drive E:

rem  �����������������������������������������������������������������������������Ŀ

rem  �      �    � Starting Location  �  Ending Location   �  Relative � Number of �

rem  �System�Boot�Side Cylinder Sector�Side Cylinder Sector�  Sectors  �  Sectors  �

rem  �����������������������������������������������������������������������������Ĵ

rem  �BIGDOS� Yes�  0     272      2  � 14     407     51  �        1  �   104039  �

rem  �EXTEND� No �  0     408      1  � 14     543     51  �   208080  �   104040  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �������������������������������������������������������������������������������

rem  Listed below is the RELATIVE location of each byte in the partition table.

rem  The 16-byte list is repeated for each line in the partition table.  A

rem  maximum of 4 partitions may be defined. The partition table begins at

rem  sector offset 01BE.

rem  0000  00          ; partition status (active [80] or non-active [00])

rem  0001  00          ; starting side

rem  0002  00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem  0003  00          ; starting cylinder (lower byte)

rem  0004  00          ; system (see information on previous page for valid codes)

rem  0005  00          ; ending side

rem  0006  00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem  0007  00          ; ending cylinder (lower byte)

rem  0008  00 00 00 00 ; relative sectors

rem  000C  00 00 00 00 ; number of sectors

rem  Finally, the actual partition table data.

rem  Remember, data order is low-byte, then high-byte.

echo E 3BE 00          >> CP3204.SCR

echo E 3BF 00          >> CP3204.SCR

echo E 3C0 42          >> CP3204.SCR

echo E 3C1 10          >> CP3204.SCR

echo E 3C2 06          >> CP3204.SCR

echo E 3C3 0E          >> CP3204.SCR

echo E 3C4 73          >> CP3204.SCR

echo E 3C5 97          >> CP3204.SCR

echo E 3C6 01 00 00 00 >> CP3204.SCR

echo E 3CA 67 96 01 00 >> CP3204.SCR

echo E 3CE 00          >> CP3204.SCR

echo E 3CF 00          >> CP3204.SCR

echo E 3D0 41          >> CP3204.SCR

echo E 3D1 98          >> CP3204.SCR

echo E 3D2 05          >> CP3204.SCR

echo E 3D3 0E          >> CP3204.SCR

echo E 3D4 B3          >> CP3204.SCR

echo E 3D5 1F          >> CP3204.SCR

echo E 3D6 D0 2C 03 00 >> CP3204.SCR

echo E 3DA 68 96 01 00 >> CP3204.SCR

rem  Update partition program code to write data on correct cylinder.

echo E 11D 41 10 >> CP3204.SCR

rem  Update volume label in BIOS Parameter Block.

echo E 435 "3" >> CP3204.SCR

rem  If REM option was chosen, don't execute partition program.  Otherwise,

rem  go write partition sector, boot sector, and 1st FAT sector.

echo %4 G=100 138 >> CP3204.SCR

echo   creation of partition table for 3rd logical drive complete . . .

rem

rem          F I X E D   D I S K   D R I V E   P A R T I T I O N   T A B L E

rem  �������������������������������������������������������������������������������

rem  FDISK1 VOL4 �� logical drive F:

rem  �����������������������������������������������������������������������������Ŀ

rem  �      �    � Starting Location  �  Ending Location   �  Relative � Number of �

rem  �System�Boot�Side Cylinder Sector�Side Cylinder Sector�  Sectors  �  Sectors  �

rem  �����������������������������������������������������������������������������Ĵ

rem  �BIGDOS� No �  0     408      2  � 14     543     51  �        1  �   104039  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �������������������������������������������������������������������������������

rem  Listed below is the RELATIVE location of each byte in the partition table.

rem  The 16-byte list is repeated for each line in the partition table.  A

rem  maximum of 4 partitions may be defined. The partition table begins at

rem  sector offset 01BE.

rem  0000  00          ; partition status (active [80] or non-active [00])

rem  0001  00          ; starting side

rem  0002  00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem  0003  00          ; starting cylinder (lower byte)

rem  0004  00          ; system (see information on previous page for valid codes)

rem  0005  00          ; ending side

rem  0006  00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem  0007  00          ; ending cylinder (lower byte)

rem  0008  00 00 00 00 ; relative sectors

rem  000C  00 00 00 00 ; number of sectors

rem  Finally, the actual partition table data.

rem  Remember, data order is low-byte, then high-byte.

echo E 3BE 00          >> CP3204.SCR

echo E 3BF 00          >> CP3204.SCR

echo E 3C0 42          >> CP3204.SCR

echo E 3C1 98          >> CP3204.SCR

echo E 3C2 06          >> CP3204.SCR

echo E 3C3 0E          >> CP3204.SCR

echo E 3C4 B3          >> CP3204.SCR

echo E 3C5 1F          >> CP3204.SCR

echo E 3C6 01 00 00 00 >> CP3204.SCR

echo E 3CA 67 96 01 00 >> CP3204.SCR

rem  No EXTENDed partition data in partition table of last logical drive.

echo F 3CE 3DD 0 >> CP3204.SCR

rem  Update partition program code to write data on correct cylinder.

echo E 11D 41 98 >> CP3204.SCR

rem  Update volume label in BIOS Parameter Block.

echo E 435 "4" >> CP3204.SCR

rem  If REM option was chosen, don't execute partition program.  Otherwise,

rem  go write partition sector, boot sector, and 1st FAT sector.

echo %4 G=100 >> CP3204.SCR

rem  Finally, end of DEBUG script for 1st fixed disk.

echo Q >> CP3204.SCR

echo   creation of partition table for 4th logical drive complete . . .

goto RUN

rem

:DISK2

rem          F I X E D   D I S K   D R I V E   P A R T I T I O N   T A B L E

rem  �������������������������������������������������������������������������������

rem  Fixed Disk 2 �� Master Partition Table

rem  �����������������������������������������������������������������������������Ŀ

rem  �      �    � Starting Location  �  Ending Location   �  Relative � Number of �

rem  �System�Boot�Side Cylinder Sector�Side Cylinder Sector�  Sectors  �  Sectors  �

rem  �����������������������������������������������������������������������������Ĵ

rem  �EXTEND� No �  0       0      2  � 14     543     51  �        1  �   416159  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �������������������������������������������������������������������������������

rem  Listed below is the RELATIVE location of each byte in the partition table.

rem  The 16-byte list is repeated for each line in the partition table.  A

rem  maximum of 4 partitions may be defined. The partition table begins at

rem  sector offset 01BE.

rem  0000  00          ; partition status (active [80] or non-active [00])

rem  0001  00          ; starting side

rem  0002  00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem  0003  00          ; starting cylinder (lower byte)

rem  0004  00          ; system (see information on previous page for valid codes)

rem  0005  00          ; ending side

rem  0006  00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem  0007  00          ; ending cylinder (lower byte)

rem  0008  00 00 00 00 ; relative sectors

rem  000C  00 00 00 00 ; number of sectors

rem  Finally, the actual partition table data.

rem  Remember, data order is low-byte, then high-byte.

echo E 3BE 00          >> CP3204.SCR

echo E 3BF 00          >> CP3204.SCR

echo E 3C0 02          >> CP3204.SCR

echo E 3C1 00          >> CP3204.SCR

echo E 3C2 05          >> CP3204.SCR

echo E 3C3 0E          >> CP3204.SCR

echo E 3C4 B3          >> CP3204.SCR

echo E 3C5 1F          >> CP3204.SCR

echo E 3C6 01 00 00 00 >> CP3204.SCR

echo E 3CA 9F 59 06 00 >> CP3204.SCR

echo   creation of master partition table complete . . .

rem  A few words about partitioning the 2nd fixed disk are in order.  When the

rem  entire 2nd fixed disk is setup as an EXTENDed partition, the first sector

rem  contains the master partition table.  This is shown above.  Since there

rem  is an additional logical drive on the disk, an additional partition table

rem  is required for the first logical drive on the disk.  In order to conserve

rem  disk space and maintain the same logical structure as the 1st fixed disk,

rem  the additional partition table is placed in the 2nd sector on the disk.

rem

rem  The second sector then contains the partition table for the first logical

rem  drive AND the BIOS Parameter Block.  This is possible because the DOS boot

rem  program is not required.  The starting location entered in the partition

rem  table for first logical disk is the same as the starting location entered

rem  in the master partition table.  Study the master partition table above and

rem  the partition table for the first logical drive on the next page.  Make

rem  sure you understand what is happening before you try this with your

rem  particular system.

rem

rem          F I X E D   D I S K   D R I V E   P A R T I T I O N   T A B L E

rem  �������������������������������������������������������������������������������

rem  FDISK2 VOL1 �� logical drive G:

rem  �����������������������������������������������������������������������������Ŀ

rem  �      �    � Starting Location  �  Ending Location   �  Relative � Number of �

rem  �System�Boot�Side Cylinder Sector�Side Cylinder Sector�  Sectors  �  Sectors  �

rem  �����������������������������������������������������������������������������Ĵ

rem  �BIGDOS� No �  0       0      2  � 14     135     51  �        1  �   104039  �

rem  �EXTEND� No �  0     136      1  � 14     271     51  �   104040  �   104040  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �������������������������������������������������������������������������������

rem  Listed below is the RELATIVE location of each byte in the partition table.

rem  The 16-byte list is repeated for each line in the partition table.  A

rem  maximum of 4 partitions may be defined. The partition table begins at

rem  sector offset 01BE.

rem  0000  00          ; partition status (active [80] or non-active [00])

rem  0001  00          ; starting side

rem  0002  00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem  0003  00          ; starting cylinder (lower byte)

rem  0004  00          ; system (see information on previous page for valid codes)

rem  0005  00          ; ending side

rem  0006  00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem  0007  00          ; ending cylinder (lower byte)

rem  0008  00 00 00 00 ; relative sectors

rem  000C  00 00 00 00 ; number of sectors

rem  Finally, the actual partition table data.

rem  Remember, data order is low-byte, then high-byte.

echo E 5BE 00          >> CP3204.SCR

echo E 5BF 00          >> CP3204.SCR

echo E 5C0 02          >> CP3204.SCR

echo E 5C1 00          >> CP3204.SCR

echo E 5C2 06          >> CP3204.SCR

echo E 5C3 0E          >> CP3204.SCR

echo E 5C4 33          >> CP3204.SCR

echo E 5C5 87          >> CP3204.SCR

echo E 5C6 01 00 00 00 >> CP3204.SCR

echo E 5CA 67 96 01 00 >> CP3204.SCR

echo E 5CE 00          >> CP3204.SCR

echo E 5CF 00          >> CP3204.SCR

echo E 5D0 01          >> CP3204.SCR

echo E 5D1 88          >> CP3204.SCR

echo E 5D2 05          >> CP3204.SCR

echo E 5D3 0E          >> CP3204.SCR

echo E 5D4 73          >> CP3204.SCR

echo E 5D5 0F          >> CP3204.SCR

echo E 5D6 68 96 01 00 >> CP3204.SCR

echo E 5DA 68 96 01 00 >> CP3204.SCR

rem  Update volume label in BIOS Parameter Block.

echo E 435 "1" >> CP3204.SCR

rem  If REM option was chosen, don't execute partition program.  Otherwise,

rem  go write partition sector, boot sector, and 1st FAT sector.

echo %4 G=100 138 >> CP3204.SCR

rem  Clear partition table in boot sector.

echo F 5BE 5FD 0 >> CP3204.SCR

echo   creation of partition table for 1st logical drive complete . . .

rem

rem          F I X E D   D I S K   D R I V E   P A R T I T I O N   T A B L E

rem  �������������������������������������������������������������������������������

rem  FDISK2 VOL2 �� logical drive H:

rem  �����������������������������������������������������������������������������Ŀ

rem  �      �    � Starting Location  �  Ending Location   �  Relative � Number of �

rem  �System�Boot�Side Cylinder Sector�Side Cylinder Sector�  Sectors  �  Sectors  �

rem  �����������������������������������������������������������������������������Ĵ

rem  �BIGDOS� No �  0     136      2  � 14     271     51  �        1  �   104039  �

rem  �EXTEND� No �  0     272      1  � 14     407     51  �   208080  �   104040  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �������������������������������������������������������������������������������

rem  Listed below is the RELATIVE location of each byte in the partition table.

rem  The 16-byte list is repeated for each line in the partition table.  A

rem  maximum of 4 partitions may be defined. The partition table begins at

rem  sector offset 01BE.

rem  0000  00          ; partition status (active [80] or non-active [00])

rem  0001  00          ; starting side

rem  0002  00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem  0003  00          ; starting cylinder (lower byte)

rem  0004  00          ; system (see information on previous page for valid codes)

rem  0005  00          ; ending side

rem  0006  00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem  0007  00          ; ending cylinder (lower byte)

rem  0008  00 00 00 00 ; relative sectors

rem  000C  00 00 00 00 ; number of sectors

rem  Finally, the actual partition table data.

rem  Remember, data order is low-byte, then high-byte.

echo E 3BE 00          >> CP3204.SCR

echo E 3BF 00          >> CP3204.SCR

echo E 3C0 02          >> CP3204.SCR

echo E 3C1 88          >> CP3204.SCR

echo E 3C2 06          >> CP3204.SCR

echo E 3C3 0E          >> CP3204.SCR

echo E 3C4 73          >> CP3204.SCR

echo E 3C5 0F          >> CP3204.SCR

echo E 3C6 01 00 00 00 >> CP3204.SCR

echo E 3CA 67 96 01 00 >> CP3204.SCR

echo E 3CE 00          >> CP3204.SCR

echo E 3CF 00          >> CP3204.SCR

echo E 3D0 41          >> CP3204.SCR

echo E 3D1 10          >> CP3204.SCR

echo E 3D2 05          >> CP3204.SCR

echo E 3D3 0E          >> CP3204.SCR

echo E 3D4 73          >> CP3204.SCR

echo E 3D5 97          >> CP3204.SCR

echo E 3D6 D0 2C 03 00 >> CP3204.SCR

echo E 3DA 68 96 01 00 >> CP3204.SCR

rem  Update partition program code to write data on correct cylinder.

echo E 11D 01 88 >> CP3204.SCR

rem  Update volume label in BIOS Parameter Block.

echo E 435 "2" >> CP3204.SCR

rem  If REM option was chosen, don't execute partition program.  Otherwise,

rem  go write partition sector, boot sector, and 1st FAT sector.

echo %4 G=100 138 >> CP3204.SCR

echo   creation of partition table for 2nd logical drive complete . . .

rem

rem          F I X E D   D I S K   D R I V E   P A R T I T I O N   T A B L E

rem  �������������������������������������������������������������������������������

rem  FDISK2 VOL3 �� logical drive I:

rem  �����������������������������������������������������������������������������Ŀ

rem  �      �    � Starting Location  �  Ending Location   �  Relative � Number of �

rem  �System�Boot�Side Cylinder Sector�Side Cylinder Sector�  Sectors  �  Sectors  �

rem  �����������������������������������������������������������������������������Ĵ

rem  �BIGDOS� Yes�  0     272      2  � 14     407     51  �        1  �   104039  �

rem  �EXTEND� No �  0     408      1  � 14     543     51  �   312120  �   104040  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �������������������������������������������������������������������������������

rem  Listed below is the RELATIVE location of each byte in the partition table.

rem  The 16-byte list is repeated for each line in the partition table.  A

rem  maximum of 4 partitions may be defined. The partition table begins at

rem  sector offset 01BE.

rem  0000  00          ; partition status (active [80] or non-active [00])

rem  0001  00          ; starting side

rem  0002  00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem  0003  00          ; starting cylinder (lower byte)

rem  0004  00          ; system (see information on previous page for valid codes)

rem  0005  00          ; ending side

rem  0006  00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem  0007  00          ; ending cylinder (lower byte)

rem  0008  00 00 00 00 ; relative sectors

rem  000C  00 00 00 00 ; number of sectors

rem  Finally, the actual partition table data.

rem  Remember, data order is low-byte, then high-byte.

echo E 3BE 00          >> CP3204.SCR

echo E 3BF 00          >> CP3204.SCR

echo E 3C0 42          >> CP3204.SCR

echo E 3C1 10          >> CP3204.SCR

echo E 3C2 06          >> CP3204.SCR

echo E 3C3 0E          >> CP3204.SCR

echo E 3C4 73          >> CP3204.SCR

echo E 3C5 97          >> CP3204.SCR

echo E 3C6 01 00 00 00 >> CP3204.SCR

echo E 3CA 67 96 01 00 >> CP3204.SCR

echo E 3CE 00          >> CP3204.SCR

echo E 3CF 00          >> CP3204.SCR

echo E 3D0 41          >> CP3204.SCR

echo E 3D1 98          >> CP3204.SCR

echo E 3D2 05          >> CP3204.SCR

echo E 3D3 0E          >> CP3204.SCR

echo E 3D4 B3          >> CP3204.SCR

echo E 3D5 1F          >> CP3204.SCR

echo E 3D6 38 C3 04 00 >> CP3204.SCR

echo E 3DA 68 96 01 00 >> CP3204.SCR

rem  Update partition program code to write data on correct cylinder.

echo E 11D 41 10 >> CP3204.SCR

rem  Update volume label in BIOS Parameter Block.

echo E 435 "3" >> CP3204.SCR

rem  If REM option was chosen, don't execute partition program.  Otherwise,

rem  go write partition sector, boot sector, and 1st FAT sector.

echo %4 G=100 138 >> CP3204.SCR

echo   creation of partition table for 3rd logical drive complete . . .

rem

rem          F I X E D   D I S K   D R I V E   P A R T I T I O N   T A B L E

rem  �������������������������������������������������������������������������������

rem  FDISK2 VOL4 �� logical drive J:

rem  �����������������������������������������������������������������������������Ŀ

rem  �      �    � Starting Location  �  Ending Location   �  Relative � Number of �

rem  �System�Boot�Side Cylinder Sector�Side Cylinder Sector�  Sectors  �  Sectors  �

rem  �����������������������������������������������������������������������������Ĵ

rem  �BIGDOS� No �  0     408      2  � 14     543     51  �        1  �   104039  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �unused� No �  0       0      0  �  0       0      0  �        0  �        0  �

rem  �������������������������������������������������������������������������������

rem  Listed below is the RELATIVE location of each byte in the partition table.

rem  The 16-byte list is repeated for each line in the partition table.  A

rem  maximum of 4 partitions may be defined. The partition table begins at

rem  sector offset 01BE.

rem  0000  00          ; partition status (active [80] or non-active [00])

rem  0001  00          ; starting side

rem  0002  00          ; starting sector (lower 6-bits, upper 2-bits for cylinder)

rem  0003  00          ; starting cylinder (lower byte)

rem  0004  00          ; system (see information on previous page for valid codes)

rem  0005  00          ; ending side

rem  0006  00          ; ending sector (lower 6-bits, upper 2-bits for cylinder)

rem  0007  00          ; ending cylinder (lower byte)

rem  0008  00 00 00 00 ; relative sectors

rem  000C  00 00 00 00 ; number of sectors

rem  Finally, the actual partition table data.

rem  Remember, data order is low-byte, then high-byte.

echo E 3BE 00          >> CP3204.SCR

echo E 3BF 00          >> CP3204.SCR

echo E 3C0 42          >> CP3204.SCR

echo E 3C1 98          >> CP3204.SCR

echo E 3C2 06          >> CP3204.SCR

echo E 3C3 0E          >> CP3204.SCR

echo E 3C4 B3          >> CP3204.SCR

echo E 3C5 1F          >> CP3204.SCR

echo E 3C6 01 00 00 00 >> CP3204.SCR

echo E 3CA 67 96 01 00 >> CP3204.SCR

rem  No EXTENDed partition data in partition table of last logical drive.

echo F 3CE 3DD 0 >> CP3204.SCR

rem  Update partition program code to write data on correct cylinder.

echo E 11D 41 98 >> CP3204.SCR

rem  Update volume label in BIOS Parameter Block.

echo E 435 "4" >> CP3204.SCR

rem  If REM option was chosen, don't execute partition program.  Otherwise,

rem  go write partition sector, boot sector, and 1st FAT sector.

echo %4 G=100 >> CP3204.SCR

rem  Finally, end of DEBUG script for 2nd fixed disk.

echo Q >> CP3204.SCR

echo   creation of partition table for 4th logical drive complete . . .

rem

:RUN

echo END - creation of CP3204.SCR

echo �

goto @%OPTION%

:@PREPARE

if NOT EXIST prepare.bin goto ERR0

ren prepare.bin prepare.com

echo BEGIN:  Fixed disk [%1] preparation . . .

%4 prepare %1 /P

echo �

if ERRORLEVEL 1 goto ERR1

if ERRORLEVEL 0 goto @RESTORE

echo ERROR:  Invalid ERRORLEVEL during fixed disk [%1] preparation.

goto EXIT

:@RESTORE

echo BEGIN:  Fixed disk [%1] partitioning . . .

debug < CP3204.SCR > %OUTFILE%

echo �

if ERRORLEVEL 128 goto ERR128

if ERRORLEVEL 2   goto ERR2

if ERRORLEVEL 0   goto OK

echo ERROR:  Invalid ERRORLEVEL during fixed disk [%1] partitioning.

goto EXIT

:ERR0

echo ERROR:  The file PREPARE.BIN is missing.  Unable to prepare fixed disk [%1].

goto EXIT

:ERR1

echo ERROR:  Unable to clear sectors during fixed disk [%1] preparation.

goto EXIT

:ERR2

echo ERROR:  Unable to write partition sectors to fixed disk [%1].

goto EXIT

:ERR128

echo ERROR:  Incorrect DOS version.  This program required DOS 5.00 or higher.

goto EXIT

:@

echo ERROR:  Environment variable OPTION not set.  Unable to continue.

goto EXIT

:OK

echo Partition sectors sucessfully written to fixed disk.

echo �

echo !!!!!!!!!!!!!!!!!!!! W A R N I N G !!!!!!!!!!!!!!!!!!!!

echo Do NOT use the FORMAT command to format fixed disk [%1].

echo All logical drives have been formatted by this program.

echo !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

echo �

echo Press CTRL-ALT-DEL to reboot . . .

:EXIT

set OPTION=

set OUTFILE=

if EXIST prepare.com ren prepare.com prepare.bin

if [%1]==[/1] if exist CP3204.SCR ren CP3204.SCR CP3204.1%2%4

if [%1]==[/2] if exist CP3204.SCR ren CP3204.SCR CP3204.2%2%4

echo �