# CHAPTER 10	OVERVIEW§§

## 1.10	INTRODUCTION

The technology to add sound to the personal computer has been available for many years.  However, music and voice capability on the PC is just beginning to become accepted as a standard feature, largely due to the advent of multimedia.  Until now, add-on sound boards have provided either low quality sound (the FM synthesizer products, such as AdLib and Sound Blaster) or relatively expensive, professional quality sound (such as the Roland products).  These sound systems are also very limited in their range of capability -- they can only perform specific, hardware-based sound processing tasks.

Sierra Semiconductor Corporation has developed a new and versatile sound technology for the PC.  Based upon a high-speed digital signal processor (DSP), the "Aria Multimedia Audio System" provides high-quality music synthesis, digital audio recording and playback, and a variety of data interface options.  The ability to execute an unlimited number of DSP software applications makes the Aria system the best choice for integrating sound with entertainment, business, and educational software.  Aria is also backwards compatible with existing AdLib and Sound Blaster software applications.

This documentation is intended as a guide and reference manual for software developers wishing to enhance their products with high-quality audio features.  As part of Sierra Semiconductor's developer program, each developer will also receive an Aria development board, software tools and coding examples, device drivers, and technical phone support.

## 1.20	CAPABILITIES

### 1.2.1	Aria Wave Synthesis

Wave synthesis is a technique which can accurately reproduce a particular musical instrument sound by reading and processing a stored digital waveform.  Typically, melodic sounds such as piano, violin and trumpet can be created by looping one cycle of their periodic waveform.  Percussive sounds and complex melodic instruments require storage of a more complex and larger representation of their waveform.  These are not looped, but instead played only once.

µ §

Figure 1.1§  LOOPED AND NON-LOOPED WAVEFORMS

The Aria SynthesizerÔ provides both looped and non-looped methods of wave synthesis, known as Aria Wave Table Synthesis.  These methods can be combined to produce very realistic acoustic instrument sounds.  Envelope attack phases can be non-looped samples with sustaining portions being looped.

Richer and more dynamic sounds can be created by combining more than one looped waveform.  An instrument "patch" can define up to eight waveforms per voice.  Up to 32 waveforms (or operators) may be played simultaneously.  Each individual operator can specify detuning, transposition, an eight stage amplitude envelope and a variety of low frequency oscillators.XE "Aria Synthesizer" \r "Synthesis"§XE "Wave Synthesis" \r "Synthesis"§XE "Aria Wave Synthesis" \r "Synthesis"§

### 2.2.2	Digital Audio Recording and Playback

The Aria system can record and playback PCM digital audioXE "Digital Audio Recording"§XE "Digital Audio Playback"§ at rates of 11.025 KHz, 22.05 KHz and 44.1 KHz.  Up to two channels can be recorded simultaneously (stereo) and up to four channels may be played back simultaneously.  Both 8-bit and 16-bit data resolutions can be processed, as well as ADPCMXE "ADPCM Compression"§XE "Data Compression"§ data compression/decompression and 
A-Law/m-LawXE "A-Law Companding"§XE "Mu-Law Companding"§ companding.

### 2.2.3	Audio Mixer

Most Aria systems provide a 5 channel stereo mixerXE "Mixer"§ with tone control.  An additional mono input channel is available for items such as the PC speaker or telephone audio.  Please note that some older Aria systems provide only a 3 channel stereo mixer.

### 2.2.4	MIDI Interface

A Musical Instrument Digital Interface (MIDI)XE "MIDI"§ port is provided, allowing external keyboards and synthesizers to be attached to the computer.  The Aria interface is compatible with the UARTXE "UART"§ and recording modes of the industry-standard Roland 
MPU-401XE "MPU-401"§ MIDI interface.

### 2.2.5	Sound Blaster Emulation

The Aria system has the ability to emulate the Creative Labs Sound BlasterXE "Sound Blaster Emulation"§ audio board for synthesized music, digital audio recording and digital audio playback.  Products designed for use with the Sound Blaster which utilize these features are compatible with the Aria system.  Sound Blaster MIDI, composite speech mode (CSM) and hardware compression of digital audio functions are not currently supported.  Support for ADPCM compression is provided only in Aria mode.

### 2.2.6	Joystick

A standard IBM joystickXE "Joystick"§/game adapterXE "Game Adaptor"§ port is provided and allows for access of up to two separate joysticks.

### 2.2.7	Optional CD-ROM Interface

The Aria system allows for optional installation of a Small Computer System Interface (SCSI) XE "SCSI"§port.  This can be used as a standard data interface to an internal CD-ROMXE "CD-ROM"§ drive.

Audio output from a CD-ROM drive can be mixed directly with the synthesized and digitized sounds by using the auxiliary input channel of the Aria mixer.  Volume control is provided for both left and right channels.

XE "Capabilities" \r "Capabilities"§

## 2.30	AUDIO HARDWAREXE "Hardware"§

### 2.3.1	Description

The Aria system is a multimedia audio board based on a Digital Signal Processor (DSP)XE "DSP"§ running at 42 MHz, a system controller chip, an analog converter/mixer chip and system ROM chip(s).  The audio board is intended for all IBM AT compatible, 16-bit Industry Standard Architecture (ISA) bus computers.

Key features of the Aria system include:

·	16-bit I/O Registers for data transfers between ISA bus and DSP, as well as 8-bit registers for Sound Blaster emulation.

·	16-bit Control Registers for ISA bus control of the multimedia sound system.

·	A 16-bit Stereo DACXE "DAC"§ and a 16-bit Stereo ADCXE "ADC"§.  Some older Aria systems contain only a 10-bit ADC capable of stereo 44.1 KHz sampling.

Hardware sample rates are selectable from 44.1 KHz, 31.5 KHz, 22.05 KHz, or 15.75 KHz.  A 512 word hardware buffer is provided through the use of the DMA interface to DSP memory.

·	An 11 channel (5 stereo + 1 mono) Mixer XE "Mixer"§which controls DAC output, microphone input, external  input, CD-ROM input, auxiliary input and PC speaker/telephone audio input.  Each channel has an independent 8-bit linear level control.  Treble and bass tone control is also provided.  Some older Aria systems only contain a 6 channel mixer.

·	A Musical Instrument Digital InterfaceXE "MIDI"§ (MIDI) XE "MIDI"§port similar to the industry standard Roland MPU-401.  An on-board microcontroller provides both UARTXE "UART"§ mode and "smart" MIDI functions, such as time stamping and tempo adjustment.  A subset of the MPU-401XE "MPU-401"§ command set is supported.

·	1 Mb or 512 Kb of ROM wave table and PCM sample storage options.  The ROM is socketed, so it may be upgraded with new features and sounds.  The Aria system is capable of accessing up to 16 Mb of ROM.

·	Direct Memory Access (DMA)XE "DMA"§ transfer logic to handle high speed 16-bit transfers between PC memory and DSP memory in both directions (not simultaneous).  Additional logic handles 8-bit DMA transfers when in Sound Blaster emulation mode.  DAC output and ADC input to the DSP memory make use of high performance DMA transfers.

·	A standard IBM-style Joystick PortXE "Joystick"§XE "Game Adaptor"§.  This port has two selectable speeds for use in fast host machines.

### 2.3.2	Block Diagram

µ §

Figure 2.2§  BLOCK DIAGRAM

Figure 2.3 shows an overall block diagram of the audio system and major components.

### 2.3.3	PC Requirements

An IBM AT or compatible personal computer is requiredXE "Requirements"§ to run the audio hardware.  This can be any machine with an Intel 80286, 80386 or 80486 microprocessor running at a minimum clock speed of 10 MHz.  The PC must also contain at least one available 16-bit expansion slot that can accept a full-length peripheral board.  Minimum requirements specified by Multimedia PC (MPC)XE "MPC"§ manufacturers are:

·	2 Mb of RAM

·	VGA color display monitor and adapter

·	CD-ROM drive with audio outputs

·	Two-button mouse

·	3.5-inch, 1.44 Mb floppy disk drive

·	30 Mb hard drive

·	Serial port

·	Parallel port

The Aria system meets or exceeds all requirements for MIDI, joystick, music synthesis, sound processing and audio mixing.

## XE "Hardware" \r "Hardware"§

## 2.40	AUDIO SOFTWARE

### 2.4.1	Voyetra Multimedia Software

Voyetra TechnologiesXE "Voyetra Multimedia Software"§, a leading developer of PC music software, has created a multimedia system interface for use in the DOS environment.  Included with the Aria Synthesizer Software Development Kit is an evaluation copy of the Voyetra Application Programmer Interface (API) called Sound Factory XE "Sound Factory"§and a set of Voyetra DOS applications.

Sound Factory serves as a device-independent audio programming library for creating DOS-based multimedia applications.  Much like the architecture of Microsoft Windows Multimedia Extensions (MME), Sound Factory is structured on two levels:

·	The Voyetra Multimedia Player (VMP) XE "VMP"§is a high-level, memory-resident API that is functionally analogous to the audio portion of the MME's MCI layer.  VMP provides an easy way to play digital audio and MIDI files as a background task.

·	The low-level APIs (VAPI XE "VAPI"§for MIDI, SAPI XE "SAPI"§for synthesis, and AAPI XE "AAPI"§for digital audio) provide device independence and a set of function calls for directly controlling the Aria system hardware.  Using the VMP and low-level APIs, a DOS developer may easily add sound to an existing application while supporting a wide range of sound hardware.

Several Voyetra DOS applications are included to control the Aria Multimedia Audio System.  These utilities are designed to control the audio hardware, manipulate MIDI and PCM data, and demonstrate PC sound features.  In addition, several Voyetra Windows applications have been included.

Programmer support files for the Sound Factory API are provided and include object and header files, C source code for the Voyetra Command Line Players, and .MAC files for assembly programmers.

Chapter 4 of this Software Development Kit contains an overview of the Sound Factory API.  Detailed documentation for Sound Factory and the other Voyetra utilities is included in a separate manual, along with licensing information.

### 2.4.2	Microsoft Multimedia Windows Driver

Included with this development kit is a Microsoft Windows Multimedia Extensions (MME) device driver.  This device driver provides support for the following Windows Multimedia Application Programmer Interface (API) function calls:

·	Waveform Output

·	Waveform Input

·	Auxiliary Audio

·	MIDI Output

·	MIDI Input

·	Extended Aria Control Interface

The device driver translates these API calls into appropriate communications to the audio hardware.  The specific API calls supported and the functionality they provide is covered in Chapter 5.

This driver is intended for use with either Microsoft Windows 3.0 with Multimedia Extensions or Microsoft Windows 3.1.

XE "Microsoft Windows" \r "Windows"§XE "Windows" \r "Windows"§

### 2.4.3	Sound Blaster Emulation

A software emulation of the Sound BlasterXE "Sound Blaster Emulation"§ audio board is built-in to the Aria system.  On power-up, the system will default to Sound Blaster mode.  Most Sound Blaster software applications should be capable of running normally with no modification required.  Both synthesized music and digital audio are supported.

Sound Blaster MIDI, AdLib Composite Speech Mode (CSM) and digital audio compression (ADPCM) are not supported in the Sound Blaster emulation mode.

### 2.4.4	Development Tools

Included with this development kit is a C language libraryXE "Development Tools"§ of functions that provide a simple DOS-based application interface to the Aria system.  All source code files and compiled libraries are provided on diskette.  Each library functionXE "DOS Application Interface"§ is fully documented in Chaper 6, "DOS APPLICATION INTERFACE."

Chapter 7, "TOOLS," XE "Tools"§covers all supplied software utilities and tools.  An instrument patch editing utility, "MAKEPAT.EXE," XE "Patch Editor"§XE "MAKEPAT.EXE"§is provided allowing the creation of new synthesized sounds.  Other utilities for digital audio playback and recording, MIDI file playback, and

Aria board testing are included with complete source code.

XE "Software" \r "Software"§