# CHAPTER 10	MICROSOFT WINDOWS MULTIMEDIA EXTENSIONS§§

## 1.10	AUDIO DRIVER FEATURES

The Microsoft Windows Multimedia Extensions (MME) driver supports digitized audio playback and recording, volume control, MIDI data input/output, and music/sound synthesis conforming to the MIDI 1.0 and General MIDI specifications.  This driver is also for use with Microsoft Windows 3.1.

The audio functions are supported through the use of separate logical devices.  The Aria system is capable of simultaneous operations of some combinations of these logical devices.

Defined constants for application programming can be found in the file "ARIA.H" included with the Aria Development Kit software.  For all additional application programming information, refer to Microsoft's Multimedia Development Kit (MDK).

## 1.20	DRIVER CONFIGURATIONXE "Windows:Driver Configuration"§XE "Microsoft Windows:Driver Configuration"§

The Aria configuration dialog box, accessible from the DRIVERS control panel, allows the following driver parameters to be configured:

ARIA I/O PORT	Default is 290h (switchable to 280h, 2A0h or 2B0h).

ARIA INTERRUPT	Default is IRQ 10 (switchable to IRQ 11 or 12).

DISABLE	Allows driver to be disabled, for use with Sound Blaster Emulation.

SAVE SETTINGS	Allows mixer settings to be saved upon exiting Windows.

PATCH BANK FILE	Default patch bank file is GENMIDI1.BNK for the 1/2 Mb Sound ROM or GENMIDI2.BNK for the 1 Mb Sound ROM.

MIDI I/O PORT	Default is 330h (switchable to 320h)

MIDI INTERRUPT	Default is IRQ 9 (switchable to IRQ 5 or 7).

## 1.30	WAVEFORM OUTPUTXE "Windows:Waveform Output"§XE "Microsoft Windows:Waveform Output"§

The audio system can process a wide variety of sample formats and can handle sample rates of 11.025 kHz, 22.05 kHz, and 44.1 kHz.  Under Microsoft Multimedia Windows or Windows 3.1, the audio system allows playback of up to four simultaneous digital samples.  Each sample is output by a different logical device.  These logical devices can be allocated in any way among different applications or multiple instances of an application.  During simultaneous music synthesis playback, only two logical devices are available.

Although the audio system has a maximum of four devices, stereo sample files use two logical devices each.  Therefore, the audio system can be used to play a maximum of four mono devices or two stereo devices, or any combination not exceeding a total of four.

For stereo files, the sample ordering is assumed to be left channel sample, right channel sample.  The audio system has the capability to play 8-bit or 16-bit, mono and stereo sample files simultaneously.  All 8-bit samples are unsigned, and 16-bit samples are signed with the most significant byte first.

All waveform output devices support the Microsoft WAVE format (.wav files).

It is recommended that the total data transfer rate to the audio system not exceed 176.4 Kbytes per second.  Therefore, the following configurations are supported:

1.	Four 8 bit mono channels at 11.025 kHz 
(data transfer rate = 44.1 Kbytes/second)

2.	Two 8 bit stereo channels at 11.025 kHz
(data transfer rate = 44.1 Kbytes/second)

3.	Four 16 bit mono channels at 11.025 kHz
(data transfer rate = 88.2 Kbytes/second)

4.	Two 16 bit stereo channels at 11.025 kHz
(data transfer rate = 88.2 Kbytes/second)

5.	Four 8 bit mono channels at 22.050 kHz
(data transfer rate = 88.2 Kbytes/second)

6.	Two 8 bit stereo channels at 22.050 kHz 
(data transfer rate = 88.2 Kbytes/second)

7.	Four 16 bit mono channels at 22.050 kHz
(data transfer rate = 176.4 Kbytes/second)

8.	Two 16 bit stereo channels at 22.050 kHz
(data transfer rate = 176.4 Kbytes/second)

9.	Two 8 bit mono channels at 44.1 kHz 
(data transfer rate = 88.2 Kbytes/second)

10.	Two 8 bit stereo channels at 44.1 kHz
(data transfer rate = 176.4 Kbytes/second)

11.	Two 16 bit mono channels at 44.1 kHz 
(data transfer rate = 176.4 Kbytes/second)

12.	One 16 bit stereo channel at 44.1 kHz
(data transfer rate = 176.4 Kbytes/second)

Response to Waveform Output Driver Messages

WODM_GETNUMDEVS

The Audio driver will return 4.  This indicates 4 monophonic devices or 2 stereo devices.  However, during music synthesis playback only 2 single channel devices or 1 stereo device will be available.

WODM_GETDEVCAPS

The Audio driver will fill out the WAVEOUTCAPS structure as follows:

The manufacturer ID is	MM_SIERRA (40).

The product ID is 	MM_SIERRA_ARIA_WAVEOUT (23).

The product names are	"Aria Waveform Output 1" for device 0,

"Aria Waveform Output 2" for device 1,

"Aria Waveform Output 3" for device 2,

"Aria Waveform Output 4" for device 3.

The format flag will contain the following values (ORed together):

WAVE_FORMAT_1M08	mono 8-bit, 11.025 kHz

WAVE_FORMAT_1S08	stereo 8-bit, 11.025 kHz

WAVE_FORMAT_1M16	mono 16-bit, 11.025 kHz

WAVE_FORMAT_1S16	stereo 16-bit, 11.025 kHz

WAVE_FORMAT_2M08	mono 8-bit, 22.05 kHz

WAVE_FORMAT_2S08	stereo 8-bit, 22.05 kHz

WAVE_FORMAT_2M16	mono 16-bit, 22.05 kHz

WAVE_FORMAT_2S16	stereo 16-bit, 22.05 kHz

WAVE_FORMAT_4M08	mono 8-bit, 44.1 kHz

WAVE_FORMAT_4S08	stereo 8-bit, 44.1 kHz

WAVE_FORMAT_4M16	mono 16-bit, 44.1 kHz

WAVE_FORMAT_4S16	stereo 16-bit, 44.1 kHz

The maximum number of channels will be 2 (stereo).

The support flag will contain the following values (ORed together):

WAVECAPS_PLAYBACKRATE

WAVECAPS_VOLUME

WAVECAPS_LRVOLUME

WODM_OPEN

The audio driver will use the information defined in the WAVEFORMAT structure to configure the appropriate logical devices.  In particular, the nSamplesPerSec, wBitsPerSample, and nChannels fields will be used to determine the playback rate, the sample resolution, and whether the samples are stereo or mono, respectively.  In particular, a value of two for nChannels indicates a stereo file, and a value of one indicates a mono file.   A value of four, which indicates quadrophonic sound, is not currently supported.

If a sample file is being played when a WODM_OPEN message is received, and if the playback rate of the new file is different from the current file, the audio driver will set the sample rate to the higher of the playback rates.  The driver will then pitch shift the lower rate file so that it "sounds" correct at the different playback rate.

The audio driver will also allow stereo and mono files, as well as 8-bit and 16-bit sample files to be played simultaneously.

The audio driver will place a unique value in the DWORD pointed to by dwUser.  This DWORD identifies the logical device(s) allocated in response to WODM_OPEN.  This DWORD will be returned to the driver in all other messages which refer to this WODM_OPEN.

If all logical devices have been allocated when a new WODM_OPEN message is sent, the audio driver will return a MMSYSERR_ALLOCATED.

WODM_CLOSE

The audio driver will turn off the logical device(s) corresponding to dwUser.

WODM_PREPARE

The audio driver prepares the header and data.

WODM_UNPREPARE

The audio driver reverses the effects of "prepare."

WODM_WRITE

The audio driver will add the waveform data block to the queue of the logical device(s) corresponding to dwUser.

WODM_PAUSE

The audio driver will pause the logical device(s) corresponding to dwUser.

WODM_RESTART

The audio driver will restart the logical device(s) corresponding to dwUser.

WODM_RESET

The audio driver will turn off all active voices of virtual devices corresponding to dwUser.

WODM_GETPOS

The audio driver will return the number of audio samples that have currently been played from the beginning of the sample file.

WODM_GETPLAYBACKRATE

The audio driver will return the integer and fractional parts of the playback rate of logical device(s) corresponding to dwUser.

WODM_SETPLAYBACKRATE

The audio driver will set the playback rate of the logical device(s) corresponding to dwUser according to the passed integer and fractional values.

WODM_GETPITCH

WODM_SETPITCH

The audio system does not currently support pitch shifting.  Therefore, the audio driver will return MMSYSERR_NOTSUPPORTED in response to these messages.

WODM_GETVOLUME

The Audio driver will return the left and right channel volume levels for the logical device(s) corresponding to dwUser.

WODM_SETVOLUME

The audio system supports individual volume levels for left and right channels.  For stereo files, the two logical devices used for left and right channels are set to extreme left and right pan by default.  The volume levels passed can be used to alter the stereo center for stereo files, or to simulate stereo effects for mono files.  The passed parameters are applied to the logical device(s) corresponding to dwUser.

WODM_BREAKLOOP

The audio driver will stop looping the logical device(s) corresponding to dwUser.

## 2.40	WAVEFORM INPUTXE "Windows:Waveform Input"§XE "Microsoft Windows:Waveform Input"§

The audio system provides recording capabilities from an external microphone or any audio device capable of producing a line out level.  Recording may also be performed from the 
CD-ROM audio or synthesizer output (using an application that can select these modes).  
The recordings can be either mono or stereo, with sampling rates of 11.025 kHz, 22.05 kHz or 
44.1 kHz.

Responses to Waveform Input Driver Messages

WIDM_GETNUMDEVS

The Audio driver will return 1.

WIDM_GETDEVCAPS

The Audio driver will fill out the WAVEINCAPS structure as follows:

The manufacturer ID is 	MM_SIERRA (40).

The product ID is 	MM_SIERRA_ARIA_WAVEIN (24).

The product name is 	"Aria Waveform Input"

The format flag will contain the following values (ORed together):

WAVE_FORMAT_1M08	mono 8-bit, 11.025 kHz

WAVE_FORMAT_1S08	stereo 8-bit, 11.025 kHz

WAVE_FORMAT_1M16	mono 16-bit,11.025 kHz

WAVE_FORMAT_1S16	stereo 16-bit, 11.025 kHz

WAVE_FORMAT_2M08	mono 8-bit, 22.05 kHz

WAVE_FORMAT_2S08	stereo 8-bit, 22.05 kHz

WAVE_FORMAT_2M16	mono 16-bit, 22.05 kHz

WAVE_FORMAT_4M08	mono 8-bit, 44.1 kHz

WAVE_FORMAT_2S16	stereo 16-bit, 22.05 kHz
WAVE_FORMAT_4S08	stereo 8-bit, 44.1 kHz
WAVE_FORMAT_4M16	mono 16-bit, 44.1 kHz
WAVE_FORMAT_4S16	stereo 16-bit, 44.1 kHz

The number of channels will be 2 (stereo).

SIERRA ADPCM COMPRESSIONXE "ADPCM Compression"§

In addition to the standard PCM format, WAVE_FORMAT_PCM, the Aria driver supports Sierra's ADPCM 4:1 compression format:

WAVE_FORMAT_SIERRA_ADPCM (0x0013).

When recording or playing using this format, no other wave audio or synthesizer functions will be available.

ADPCM WAVE Format Header

typedef struct sierra_adpcmwaveformat_tag {

EXTWAVEFORMAT	ewf;

WORD			wRevision;

} SIERRAADPCMWAVEFORMAT;

#define	WAVE_FORMAT_SIERRA_ADPCM	(0x0013)

| wFormatTag | This must be set to WAVE_FORMAT_SIERRA_ADPCM. |  |  |
| --- | --- | --- | --- |
| nChannels | Number of channels in the wave, 1 for mono, 2 for stereo. |  |  |
| nSamplesPerSec | Frequency of the sample rate of the wave file. This should be 22050 Hz. Other sample rates are not currently allowed. |  |  |
| nAvgBytesPerSec | Average data rate. |  |  |
|  | Playback software can estimate the buffer size using the <nAvgBytesPerSec> value. |  |  |
| nBlockAlign | This is dependent upon the number of bits per sample. |  |  |
|  | wBitsPerSample | nChannels | nBlockAlign |
|  | 4 | 1 | 1 |
|  | 4 | 2 | 1 |
|  | Playback software needs to process a multiple of <nBlockAlign> bytes of data at a time, so that the value of <nBlockAlign> can be used for buffer alignment. |  |  |
| wBitsPerSample | This is the number of bits per sample of Sierra ADPCM. Currently, only 4 bits per sample is defined. Other values are reserved. |  |  |
| cbExtraSize | The size in bytes of the extra information in the extended WAVE 'fmt' header. This should be 2. |  |  |
| wRevision | Revision of algorithm. This should be 0x0100 for the current definition. |  |  |

WIDM_OPEN

The audio driver will use the information defined in the WAVEFORMAT structure to configure the recording logical device.  The values of nSamplesPerSec, wBitsPerSample, and nChannels will determine the recording rate, the sample resolution, and whether the samples are stereo or mono, respectively.  If the audio system is already recording when a new WIDM_OPEN message is sent, the Audio driver will return a MMSYSERR_ALLOCATED.

WIDM_CLOSE

The audio driver closes the recorder device.

WIDM_PREPARE

The audio driver prepares the header and data buffer.

WIDM_UNPREPARE

The audio driver reverses the effects of "prepare."

WIDM_ADDBUFFER

The audio driver will add the new buffer to its buffer list.  This is used to hold the recorded samples when recording is started.

WIDM_START

The audio driver instructs the audio system to start recording.  The recorded samples are placed in the buffer(s) allocated by WIDM_ADDBUFFER.  A buffer is marked as done when it has been filled.

WIDM_STOP

The audio driver instructs the audio system to stop recording.  The position in the buffer is maintained.

WIDM_RESET

The audio driver instructs the audio system to stop recording, and marks buffers as done.

WIDM_GETPOS

The audio driver will return the number of samples that have been recorded.

## 2.50	MIDI OUTPUTXE "Windows:MIDI Output"§XE "Microsoft Windows:MIDI Output"§

The audio system has both an internal synthesizer and an external MIDI port for connection to other synthesizers.  Since the audio system is based on a DSP, it can be programmed with various synthesis algorithms to produce a wide variety of complex sounds.  The default internal synthesizer is configured to support on average 20 complex synthesis notes, depending on how many waveform operators comprise each instrument (A maximum of 32 simultaneous notes are possible).  The synthesizer is capable of supporting all melodic and percussive instruments defined by the General MIDI Mode XE "General MIDI Mode"§(GMM) specification.

The external MIDI hardware port allows applications on the PC to control other MIDI devices.  Commands can be routed to the external MIDI device by using the nDevice parameter (see MODM_GETNUMDEVS).

The audio system supports simultaneous playback of all items listed below.  The synthesis timbres (also called voices or instruments) are described for both melodic and percussive timbres.

| Synthesis (MIDI) Devices | Sampled (WAVE) Devices | Melodic Timbres | Percussive Timbres | Avg. Melodic Notes | Avg. Percussive Notes |
| --- | --- | --- | --- | --- | --- |
| nDevice 1 | 2 | 15 | 8 | 12 | 8 |

When additional synthesis notes exceeding the capacity of the audio system are encountered, they will replace (note steal) the oldest note currently playing with the new note as follows:

For instruments on melodic MIDI channels (1 - 9 and 11 - 16), only notes playing on higher MIDI channels than the new note will be replaced.  If there are such notes, the oldest note playing on the highest channel in use will be replaced, if necessary to play the new note.  If there are no such notes, the oldest note on the current channel will be replaced.

For instruments on the percussive MIDI channel (10), the new note will replace the oldest note currently playing, if necessary to play the new percussive note.

Occasionally timbre information must be replaced in the audio system from PC RAM during interrupt time.  The delay in playback of a note in the new timbre as a result of interrupt latency and loading/configuring this data, will not exceed 10 milliseconds on a 10 MHz 286-based PC running Multimedia Windows.

Response to MIDI Output Driver Messages

MODM_GETNUMDEVS

The Audio driver will return 2, corresponding to the number of devices supported.  The following table maps the parameter nDevice to the synthesis type:

nDevice	Synthesis Type

0	External Synthesizer via MIDI OUT port

1	Aria SynthesizerÔ 
Wave synthesis using Instrument (timbre) patches constructed from wave tables, played simultaneously with up to 2 logical devices of sampled wave output @ 22 kHz (or other rates with pitch shifting).

MODM_GETDEVCAPS

The audio driver will fill out the MIDIOUTCAPS structure as follows:

The manufacturer ID is	MM_SIERRA (40).

The product IDs are	MM_SIERRA_ARIA_MIDIOUT (20) and 	MM_SIERRA_ARIA_SYNTH (22).

The product names are	"Aria MIDI Output Port" for device 0,

"Aria Synthesizer" for device 1.

The type of device flag will contain one of the following values:

MOD_MIDIPORT	MIDI hardware port (device 0)

or

MOD_WAVETABLE	Wave table synthesis (device 1)

The audio system will support the following capacity for simultaneous synthesized melodic timbres (instruments) corresponding to synthesis nDevice 1.  The maximum number of timbres (often referred to as voices or instruments):

nDevice 1 will be 15

Eight additional percussive timbres will be supported for nDevice 1.

The audio system will support the following capacity for simultaneous synthesized notes corresponding to synthesis nDevice 1.  The maximum number of notes (polyphony):

nDevice 1 will be 32

Note:  The polyphony number above includes the sum of both melodic and percussive notes.

The MIDI channels on the port to which the device is attached will be 1 - 16.

The support flag will contain the following values (ORed together):

MIDICAPS_VOLUME

MIDICAPS_LRVOLUME

MODM_OPEN

The audio driver will "open" the synthesizer corresponding to the passed value of nDevice.  If the synthesizer requested has already been opened, the driver will return MMSYSERR_ALLOCATED.

MODM_CLOSE

The audio driver will turn off the synthesizer corresponding to nDevice.

MODM_DATA

The audio driver will translate the MIDI message (up to 3 bytes).  The message will be sent to the synthesizer corresponding to the passed value of nDevice. Refer to the "Aria MIDI Implementation" addendum for additional information.

MODM_LONGDATA

The audio driver will translate all the messages in the passed buffer.  The messages will be sent to the synthesizer corresponding to the passed value of nDevice.  Refer to the "Aria MIDI Implementation" addendum for additional information.

MODM_RESET

The audio driver will send a system reset command to the audio system.  This will reset all synthesizers corresponding to nDevice, resulting in all notes being turned off.

MODM_GETVOLUME

The audio driver will return the left and right channel volume levels.

MODM_SETVOLUME

The audio system supports individual volume levels for left and right channels (balance).

## 2.60	MIDI INPUTXE "Windows:MIDI Input"§XE "Microsoft Windows:MIDI Input"§

The audio system has a hardware MIDI port which allows it to be connected to external MIDI devices, and receive MIDI information from those devices.

Responses to MIDI Input Messages

MIDM_GETNUMDEVS

The audio driver will return 1.

MIDM_GETDEVCAPS

The Manufacturer ID is 	MM_SIERRA (40).

The Product ID is 	MM_SIERRA_ARIA_MIDIIN (21).

The Product Name is 	"Aria MIDI Input Port"

MIDM_OPEN

The audio driver will setup the MIDI hardware port to receive MIDI information.  If the port has already been setup to receive, the Audio driver will return a MMSYSERR_ALLOCATED.

MIDM_CLOSE

The audio driver will instruct the audio system that the MIDI port is no longer required for input.

MIDM_PREPARE

The audio driver will prepare the header and input MIDI buffer.

MIDM_UNPREPARE

The audio driver will reverse the effects of "prepare."

MIDM_ADDBUFFER

The audio driver will add the new buffer to its buffer list.  This is used to hold the input MIDI data when recording is started.

MIDM_START

The audio driver will start reading MIDI input data and placing it in the buffer(s) allocated by MIDM_ADDBUFFER.

MIDM_STOP

The audio driver will stop filling the buffers and will mark any buffer with data as done.

MIDM_RESET

The audio driver will stop all recording and will mark all buffers as done.

## 2.70	AUXILIARY AUDIOXE "Windows:Auxiliary Audio"§XE "Microsoft Windows:Auxiliary Audio"§

The audio system provides mixing capabilities that include an auxiliary analog stereo input.  This auxiliary input is line level and can be used to control audio output from a CD-ROM drive.

A second auxiliary line level input is provided on Aria systems with the SC18075 mixer chip.  This input will usually be connected internally to the audio output mix, which allows the final output to be recorded.

Responses to Auxiliary Audio Output Messages

AUXDM_GETNUMDEVS

The audio driver will return	1 for systems without the SC18075 mixer and
				2 for systems with the SC18075 mixer.

AUXDM_GETDEVCAPS

The audio driver will fill out the AUXCAPS structure as follows:

The manufacturer ID is MM_SIERRA (40).

The product IDs are	MM_SIERRA_ARIA_AUX (25) and 
	MM_SIERRA_ARIA_AUX2 (26).

The product names are	"Aria CD-ROM Audio Output" and 
"Aria Auxiliary Output"

The technology flags are AUXCAPS_CDAUDIO and AUXCAPS_AUXIN.

The support flag will contain the following values (ORed together):

AUXCAPS_VOLUME

AUXCAPS_LRVOLUME

AUXDM_GETVOLUME

The audio driver will return left and right channel volume levels.

AUXDM_SETVOLUME

The audio system supports individual volume levels for left and right channels (balance).

## 2.80	EXTENDED ARIA CONTROL INTERFACEXE "Windows:Extended Aria Control Interface"§XE "Microsoft Windows:Extended Aria Control Interface"§

In addition to the standard Microsoft audio functions and messages, the Aria driver provides a control interface for monitoring and adjusting other supported audio functions.

This interface can be used to control and retrieve status for volume levels, input monitor switches, and recording source selection.  Many other specialized functions are also accessed using this interface.  All extended messages should be directed to the following driver function:

SYNTAX	UINT WINAPI AriaMsg (wMsg, wParam, dwParam)

Parameter	Type	Description

wMsg	UINT	Specifies the message to be sent.  These are described in the following pages.

wParam	UINT	Specifies additional message information.

dwParam	DWORD	Specifies additional message information.

RETURNS		Returns 0 if the function was successful.  Otherwise, it returns an error number.  Possible error returns are:

MMSYSERR_NOTSUPPORTED

Function is not supported.

MMSYSERR_INVALPARAM

Invalid parameter.

MMSYSERR_ALLOCATED

Specified resource or related system resource is already allocated.

EXAMPLES	Set the recording source to microphone and turn on the microphone monitor:

AriaMsg (ARIA_SETRECORD, 0, 1L);

AriaMsg (ARIA_SETMONITOR, 1, 1L);

Get the current volume level of the synthesizer:

DWORD synthVolume;

AriaMsg (ARIA_GETVOLUME, ARIA_SYNTHVOL, &synthVolume);

NOTES		An application's module definition file (.DEF) must declare AriaMsg as follows in order to use the extended functions:

	IMPORTS   aria.AriaMsg

Any callback functions required by an application must be defined as:

void CALLBACK AriaFunc (hHandle, wMsg, dwInstance, dwParam1, dwParam2)

AriaFunc is a placeholder for the application-supplied function name.  The actual name must be exported by including it in an EXPORTS statement in the application's module definition file.

The various messages for the AriaMsg function are described in the following section.  Below are the actual constant values for each:

Message Name	Value	Description

ARIA_GETCAPS	0	Get Aria system capability information

ARIA_GETVOLUME	1	Get device left and right channel volume

ARIA_SETVOLUME	2	Set device left and right channel volume

ARIA_GETRECORD	3	Get current input recording source

ARIA_SETRECORD	4	Set input recording source

ARIA_GETLEVEL	5	Get current input level (mic or line level)

ARIA_SETLEVEL	6	Set input level (mic or line level)

ARIA_GETMONITOR	7	Get current input monitor switch

ARIA_SETMONITOR	8	Set input monitor switch

ARIA_GETSYSTEM	9	Get current Aria system mode

ARIA_SETSYSTEM	10	Set Aria system mode

ARIA_REGISTERISR	11	Register an Interrupt Service Routine (ISR)

ARIA_REMOVEISR	12	Remove ISR registration

ARIA_GETPLAYMODE	13	Get current Aria system playback mode

ARIA_SETPLAYMODE	14	Set Aria system playback mode

ARIA_GETMONMODE	15	Get current input monitor mode (mono or stereo)

ARIA_SETMONMODE	16	Set input monitor mode (mono or stereo)

ARIA_GETLISTENER	17	Get current Aria Listener speech recognition status

ARIA_SETLISTENER	18	Set Aria Listener speech recognition mode

ARIA_GETDSPFLAG	19	Get DSP flag (same as ARIA_GETDSPMEM)

ARIA_SENDCMD	20	Send Aria DSP command string

ARIA_GETEFFECT	21	Get current effects status (tone, reverb, etc.)

ARIA_SETEFFECT	22	Set effects mode (tone, reverb, etc.)

ARIA_OPEN_DEVICE	23	Open extended Aria device (for callback)

ARIA_CLOSE_DEVICE	24	Close extended Aria device

25	Reserved

26	Reserved

ARIA_GETMASTERMODE	27	Get master monitor mode

ARIA_SETMASTERMODE	28	Set master monitor mode

ARIA_LOADDSPAPP	29	Load a DSP application

ARIA_UNLOADDSPAPP	30	Unload a DSP application

ARIA_GETDSPUSAGE	31	Get current DSP utilization

ARIA_SETPATCHBANK	32	Select new patch bank

ARIA_GETDSPMEM	33	Read DSP memory location

ARIA_SETDSPMEM	34	Write DSP memory location

ARIA_LOADPATCHBANK	35	Load new patch bank file

ARIA_UNLOADPATCHBANK	
	36	Unload existing patch bank file

The following defines callback messages (wMsg) that Aria extended devices may send to an application (See ARIA_OPEN_DEVICE and ARIA_CLOSE_DEVICE for more information):

Message Name	Value	Description

ARIA_SR_OPEN	500	Speech recognition device opened

ARIA_SR_CLOSE	501	Speech recognition device closed

ARIA_SR_INTR	502	Speech recognition device generating interrupt

ARIA_CLOSE_DEVICEXE "Windows Extended Aria Interface:ARIA_CLOSE_DEVICE"§

This message terminates callback functions for the specified extended Aria device.

Parameter	Description

wParam	Specifies the extended Aria device ID:

0 - 4  = Reserved
	5       = Aria ListenerXE "Aria Listener"§XE "Speech Recognition"§ speech recognition

dwParam	Contains a long pointer to a ARIA_APP_PARMS data structure that contains valid callback information.

Notes	The following defines the ARIA_APP_PARMS data structure:

typedef struct tagARIA_APP_PARMS  {

DWORD	dwCallback;	// callback function address for app

DWORD	dwInstance;	// client's instance data

HANDLE	hDevice;	// handle to device

DWORD	dwFlags;	// allocation flags

}  ARIA_APP_PARMS;

Before returning from processing this message, the client's callback function will be invoked indicating that the device is closed.

ARIA_GETCAPSXE "Windows Extended Aria Interface:ARIA_GETCAPS"§XE "Capabilities"§

This message retrieves Aria system capability information.

Parameter	Description

wParam	Is not used.

dwParam	Contains a long pointer to a ARIACAPS data structure that will be filled with information upon return from processing this message.

Notes	The following defines the ARIACAPS data structure:

typedef struct ariacaps_tag  {

UINT	version;	// Aria ROM version number

UINT	ROMsize;	// ROM size (in Kbytes): 64, 128, 512, 1024, ...

UINT	ROMwidth;	// ROM wave table bit resolution:  8 or 16

UINT	mixerType;	// Mixer type:	0 = no mixer (for ext or aux)

//	1 = discrete analog mixer

//	4 = single chip integrated mixer

UINT	RAMsize;	// RAM size in Kwords: 8, 16, 32, 40 or 64

UINT	ADCbits;	// ADC bit resolution: 8, 10, 12 or 16

BOOL	phoneInterface;	// Phone interface present?  0=no, 1=yes

UINT	DSPapps;	// DSP applications available:

//      BIT   APPLICATION	BIT   APPLICATION

//        0      Reserved	4     Telephone interface

//        1      Sound Blaster	5     Aria Listener

//        2      Aria PCM audio	7     Reverb effects

//        3      Aria Synthesizer	8     QSound effects

}  ARIACAPS;

ARIA_GETDSPFLAG or ARIA_GETDSPMEMXE "Windows Extended Aria Interface:ARIA_GETDSPFLAG"§XE "Windows Extended Aria Interface:ARIA_GETDSPMEM"§

This message returns the data value found at a specified Aria DSP memory address.

Parameter	Description

wParam	DSP memory address to read.

dwParam	Contains a long pointer to a long integer that is to receive the data value.  The low-order word of the long integer will be the data value.

ARIA_GETDSPUSAGEXE "Windows Extended Aria Interface:ARIA_GETDSPUSAGE"§

This message returns the percentage of Aria DSP utilization based on the currently loaded applications.

Parameter	Description

wParam	Is not used.

dwParam	Contains a long pointer to a long integer that is to receive the value.  The low-order word of the value will range from 0 to 100 indicating the utilization percentage.

ARIA_GETEFFECTXE "Windows Extended Aria Interface:ARIA_GETEFFECT"§

This message returns the current status of the specified Aria effect.

Parameter	Description

wParam	Specifies one of the following effects:

	0 = Bass tone control
	1 = Treble tone control
	2 = ReverbXE "Reverb"§ mode
	3 = Reverb level
	4 = QSound Virtual AudioXE "QSound Virtual Audio"§ effects
	5 = Wave audio reverb level

dwParam	Contains a long pointer to a long integer that is to receive the status information.

Tone Controls
In the case of bass or treble tone controls, the return value will contain the right channel control in the high-order word and the left channel control in the low-order word.  The control values for each channel will range from 1 (maximum tone cut) to 8 (flat response) to 15 (maximum tone boost).  A value of 0 also indicates flat tone response.

Reverb Controls
The reverb mode returned will indicate one of the following in the low-order word:

	0 	= Reverb off
	1 - 3 	= Room 1, 2 and 3
	4 - 5	= Hall 1 and 2
	6 	= Plate Echo
	7 	= Delay

The high-order word of the reverb mode will be set to 1 if reverb has been temporarily disabled by the driver.  This can occur if the Aria system resources become overallocated.  Reverb will be enabled by the driver whenever it is possible.

The high-order word of the reverb level will be set the right channel reverb level and the low-order word will be set to the left channel reverb level.  Reverb levels for each channel will range from 0 to 0x7FFF.

QSound Virtual Audio Controls
The low-order word of the QSound effect will be one of the following:

	0 = QSound off or bypassed
	1 = QSound applied to PCM channels only
	2 = QSound applied to Aria Synthesizer operators only
	3 = QSound applied to both PCM audio and Aria Synthesizer

Notes	Only those Aria systems that contain integrated mixer chips will support tone controls.  Also, some systems exist that do not support reverb or QSound effects.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine support.

ARIA_GETLEVELXE "Windows Extended Aria Interface:ARIA_GETLEVEL"§

This message returns the input level setting for external input (mic or line level).

Parameter	Description

wParam	Set to a value of 1, indicating external input.

dwParam	Contains a long pointer to a long integer that is to receive the input level setting.  A returned value of 0 will indicate line level, a value of 1 indicates microphone level.

Notes	This message is not valid when used in conjunction with Aria systems that contain integrated mixer chips.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_GETLISTENERXE "Windows Extended Aria Interface:ARIA_GETLISTENER"§

This message returns the status of Aria ListenerXE "Aria Listener"§XE "Speech Recognition"§ voice recognition.

Parameter	Description

wParam	Is not used.

dwParam	Contains a long pointer to a long integer that is to receive the mode of Aria Listener.  A returned value of 0 will indicate that Aria Listener is off, a value of 1 indicates that it is active.

ARIA_GETMASTERMODEXE "Windows Extended Aria Interface:ARIA_GETMASTERMODE"§XE "Monitor"§

This message returns the current master monitor mode (Stereo or Mono) and the current state of the mixed output recording connection.  In mono mode, the left and right channel outputs are summed together.  The mixed output recording signal can be used for recording the final output via the Auxiliary input channel.

Parameter	Description

wParam	Is not used.

dwParam	Contains a long pointer to a long integer that is to receive the master monitor mode.  The low-order word of the returned value will be 0 if the monitor is in stereo mode or 1 if the monitor is in mono mode. The high-order word will be 0 if the mixed output signal is not connected for recording or 1 if the signal is connected.

Notes	This message is only available with Aria systems that contain an integrated mixer chip.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_GETMONITORXE "Windows Extended Aria Interface:ARIA_GETMONITOR"§

This message returns the current input monitor status (Enabled or Disabled).  The monitors allow input sources to be routed to the output.

Parameter	Description

wParam	Contains one of the following values:

	  0 = Return all input monitor status bits
	  1 = Return microphone monitor status
	  2 = Return CD-ROM audio monitor status
	  8 = Return external input monitor status*
	16 = Return auxiliary input monitor status*
	32 = Return PC speaker/telephone input monitor status*

dwParam	Contains a long pointer to a long integer that is to receive the monitor status.  If wParam is 0, the set bits of the low-order word of the returned value will indicate which monitors are enabled (bits correspond to input monitors as listed above).  Otherwise, the returned value will be 0 if the specific monitor is disabled, non-zero if the monitor is enabled.

Notes	Monitor sources denoted with an asterisk (*) are only available with Aria systems that contain an integrated mixer chip.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_GETMONMODEXE "Windows Extended Aria Interface:ARIA_GETMONMODE"§XE "Monitor"§

This message returns the current input monitor mode.  The monitor mode determines whether an input is mono or stereo.

Parameter	Description

wParam	Contains one of the following values:

	  1 = Return microphone monitor mode
	  2 = Return CD-ROM audio monitor mode
	  8 = Return external input monitor mode*
	16 = Return auxiliary input monitor mode*
	32 = Return PC speaker/telephone input monitor mode*

dwParam	Contains a long pointer to a long integer that is to receive the monitor mode.  The returned value will be one of the following:

	0 = Stereo
	1 = Mono (uses stereo left channel)
	2 = Mono (uses stereo right channel)*

Notes	Monitor sources and modes denoted with an asterisk (*) are only available with Aria systems that contain an integrated mixer chip.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_GETPLAYMODEXE "Windows Extended Aria Interface:ARIA_GETPLAYMODE"§

This message returns the current system playback mode.  The playback mode determines the number of synthesis operators and digitized wave channels that are available.

Parameter	Description

wParam	Is not used.

dwParam	Contains a long pointer to a long integer that is to receive the playback mode.  The returned value will be one of the following:

	0 = No synthesis operators, 4 wave channels
	1 = 20 synthesis operators, 2 wave channels
	2 = 32 synthesis operators, 2 wave channels

ARIA_GETRECORDXE "Windows Extended Aria Interface:ARIA_GETRECORD"§

This message returns a value which specifies the current recording source for digital audio.

Parameter	Description

wParam	Is not used.

dwParam	Contains a long pointer to a long integer that is to receive the recording source.  The returned value may be any one of the sources in either column below or one from each column (ORed together):

	  1 = Microphone	  2 = CD-ROM output
	  8 = External*		  4 = Synth+wave output
	16 = Auxiliary*		32 = PC Speaker/Telephone*

If the value is zero, all recording sources are disabled.

Notes	Items denoted with an asterisk (*) are only available with Aria systems that contain an integrated mixer chip.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_GETSYSTEMXE "Windows Extended Aria Interface:ARIA_GETSYSTEM"§

This message returns the Aria system status (Enabled or Disabled).  When disabled, the Aria system is in Sound Blaster emulation mode.

Parameter	Description

wParam	Is not used.

dwParam	Contains a long pointer to a long integer that is to receive the system status.  A returned value of 0 indicates that the system is disabled, a value of 1 indicates that the system is enabled.

ARIA_GETVOLUMEXE "Windows Extended Aria Interface:ARIA_GETVOLUME"§XE "Volume"§

This message returns the left and right volume level of a specific audio device.

Parameter	Description

wParam	Contains a value indicating which volume level to return:

	0 = Master volume
	1 = Synthesizer volume
	2 = Wave volume
	3 = Microphone volume
	4 = CD-ROM volume
	5 = External volume*
	6 = Auxiliary volume*
	7 = PC Speaker/Telephone volume*

dwParam	Contains a long pointer to a long integer that is to receive the volume levels.  The high-order word of the returned value will be the right channel volume and the low-order word will be the left channel volume.  Volume levels for each channel will range from 0 to 0x7FFF.

Notes	Devices denoted with an asterisk (*) are only available with Aria systems that contain an integrated mixer chip.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_LOADDSPAPPXE "Windows Extended Aria Interface:ARIA_LOADDSPAPP"§XE "DSP Applications"§

This message loads a DSP application into the Aria system.

Parameter	Description

wParam	Contains the DSP application number to load (0 to 15).

dwParam	If the DSP application resides in Aria ROM, this parameter must be set to NULL.  If the DSP application resides on the PC, this parameter must be a global handle to the application loaded in memory.  The memory pointer must be locked until the application is unloaded.

ARIA_LOADPATCHBANKXE "Windows Extended Aria Interface:ARIA_LOADPATCHBANK"§

This message loads a new synthesizer patch bank.  Up to 127 different instrument patch banks can be loaded.  The patch bank file must conform to the patch bank format (version 1) specified in Appendix A.

A return message of MMSYSERR_ALLOCATED can indicate that the selected patch bank number is already in use, the file cannot be found, or that no more memory can be allocated.  To select a new patch bank after loading, use the ARIA_SETPATCHBANK message.

Parameter	Description

wParam	Contains the patch bank number to load (1 to 127).

dwParam	Contains a long pointer to an ASCIIZ (zero-terminated ASCII) patch bank file name string.

ARIA_OPEN_DEVICEXE "Windows Extended Aria Interface:ARIA_OPEN_DEVICE"§

This message opens an Aria extended device.  Extended devices are typically those that execute special Aria DSP applications requiring interrupt generation on the PC.  A Windows callback will be issued upon open, close and interrupt signals from an extended device.

Parameter	Description

wParam	Specifies the extended Aria device ID:

0 - 4  = Reserved
	5       = Aria ListenerXE "Aria Listener"§XE "Speech Recognition"§ speech recognition

dwParam	Contains a long pointer to a ARIA_APP_PARMS data structure that contains valid callback information.

Notes	The following defines the ARIA_APP_PARMS data structure:

typedef struct tagARIA_APP_PARMS  {

DWORD	dwCallback;	// callback function address for app

DWORD	dwInstance;	// client's instance data

HANDLE	hDevice;	// handle to device

DWORD	dwFlags;	// allocation flags

}  ARIA_APP_PARMS;

Before returning from processing this message, the client's callback function will be invoked indicating that the device is open.

ARIA_REGISTERISRXE "Windows Extended Aria Interface:ARIA_REGISTERISR"§

This message registers the address of an Interrupt Service Routine (ISR) to be called when the appropriate Interrupt ID is generated by the Aria system.  This should not be used if Windows callback functions are preferred, as the ARIA_OPEN_DEVICE message automatically registers an application's ISR.

Parameter	Description

wParam	Contains the Interrupt ID to register:

0 - 4  = Reserved
	5       = Aria Listener speech recognition

dwParam	Contains the entry point address (far pointer) for the ISR that is to be registered.

Notes	The ISR must be fixed in memory.  If ISR relocation is required, this message must be sent again with the new address.

ARIA_REMOVEISRXE "Windows Extended Aria Interface:ARIA_REMOVEISR"§

This message removes a previously registered ISR function.  This should not be used if Windows callback functions are preferred, as the ARIA_CLOSE_DEVICE message automatically terminates calls to an application's ISR.

Parameter	Description

wParam	Contains the Interrupt ID to remove:

0 - 4  = Reserved
	5       = Aria Listener speech recognition

dwParam	Is not used.

ARIA_SENDCMDXE "Windows Extended Aria Interface:ARIA_SENDCMD"§XE "COMMANDS"§

This message sends a DSP command string to the Aria system.

Parameter	Description

wParam	Contains the number of words in the command string.

dwParam	Contains a far pointer to the command string to be sent.

ARIA_SETDSPMEMXE "Windows Extended Aria Interface:ARIA_SETDSPMEM"§

This message writes a data value at a specified Aria DSP memory address.

Parameter	Description

wParam	DSP memory address.

dwParam	The low-order word of this parameter contains the data value to write.

ARIA_SETEFFECTXE "Windows Extended Aria Interface:ARIA_SETEFFECT"§

This message sets the specified Aria effect.

Parameter	Description

wParam	Specifies one of the following effects:

	0 = Bass tone control
	1 = Treble tone control
	2 = ReverbXE "Reverb"§ mode
	3 = Reverb level
	4 = QSound Virtual AudioXE "QSound Virtual Audio"§ effects
	5 = Wave audio reverb level

dwParam	Contains the parameter value to select.

Tone Controls
In the case of bass or treble tone controls, the value must contain the right channel control in the high-order word and the left channel control in the low-order word.  The control values for each channel must range from 1 (maximum tone cut) to 8 (flat response) to 15 (maximum tone boost).  A value of 0 can also indicate flat tone response.

Reverb Controls
The reverb mode can be one of the following:

	0 	= Reverb off
	1 - 3 	= Room 1, 2 and 3
	4 - 5	= Hall 1 and 2
	6 	= Plate Echo
	7 	= Delay

The reverb level controls the amount or intensity of the reverb effect.  The high-order word of the value must be the right channel reverb level and the low-order word must be the left-channel reverb level.  Reverb levels for each channel can range from 0 to 0x7FFF.

QSound Virtual Audio Controls
The low-order word of the QSound effect must be one of the following:

	0 = QSound off or bypassed
	1 = QSound applied to PCM channels only
	2 = QSound applied to Aria Synthesizer operators only
	3 = QSound applied to both PCM audio and Aria Synthesizer

Notes	Only those Aria systems that contain integrated mixer chips will support tone controls.  Also, some systems exist that do not support reverb or QSound effects.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine support.

When reverb or QSound is activated, the driver adjusts the playback mode of the Aria system if necessary.  The ARIA_SETPLAYMODE message should not be used if reverb is active.

ARIA_SETLEVELXE "Windows Extended Aria Interface:ARIA_SETLEVEL"§

This message sets the input level for external input (microphone or line level).

Parameter	Description

wParam	Must contain a value of 1, indicating external input.

dwParam	Contains a 0 for line level or a 1 for microphone level.

Notes	This message is not valid when used in conjunction with Aria systems that contain integrated mixer chips.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_SETLISTENERXE "Windows Extended Aria Interface:ARIA_SETLISTENER"§

This message sets the Aria ListenerXE "Aria Listener"§XE "Speech Recognition"§ speech recognition mode of the driver to either on or off.

Parameter	Description

wParam	Is not used.

dwParam	Contains a 0 to switch Aria Listener off or a 1 to switch it on.

Notes	When Aria Listener is activated, the driver adjusts the playback mode of the Aria system if necessary.  The ARIA_SETPLAYMODE message should not be used if Aria Listener is active.

ARIA_SETMASTERMODEXE "Windows Extended Aria Interface:ARIA_SETMASTERMODE"§XE "Monitor"§

This message sets the current master monitor mode (Stereo or Mono) and the current state of the mixed output recording connection.  In mono mode, the left and right channel outputs will be summed together.  The mixed output recording signal can be used for recording the final output via the Auxiliary input channel.

Parameter	Description

wParam	Is not used.

dwParam	The low-order word contains a 0 to set the master monitor to stereo mode or a 1 to set it to mono mode.  The high-order word contains a 0 if the mixed output recording signal is to be disconnected or a 1 if the signal is to be connected.

Notes	This message is only available with Aria systems that contain an integrated mixer chip.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_SETMONITORXE "Windows Extended Aria Interface:ARIA_SETMONITOR"§XE "Monitor"§

This message sets the input monitor source.  The monitors allow input sources to be routed to the output.

Parameter	Description

wParam	Contains one of the following values:

	  0 = Set all input monitor status bits
	  1 = Set microphone monitor
	  2 = Set CD-ROM audio monitor
	  8 = Set external input monitor*
	16 = Set auxiliary input monitor*
	32 = Set PC speaker/telephone input monitor*

dwParam	If wParam is 0, the set bits of the low-order word of this parameter will indicate which monitors are enabled (bits correspond to input monitors as listed above).  Otherwise, this parameter will be 0 if the specific monitor is disabled, non-zero if the monitor is enabled.

Notes	Monitor sources denoted with an asterisk (*) are only available with Aria systems that contain an integrated mixer chip.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_SETMONMODEXE "Windows Extended Aria Interface:ARIA_SETMONMODE"§

This message sets the input monitor mode.  The monitor mode determines whether an input is mono or stereo.

Parameter	Description

wParam	Contains one of the following values:

	  1 = Set microphone monitor mode
	  2 = Set CD-ROM audio monitor mode
	  8 = Set external input monitor mode*
	16 = Set auxiliary input monitor mode*
	32 = Set PC speaker/telephone input monitor mode*

dwParam	Contains one of the following values:

	0 = Stereo
	1 = Mono (uses stereo left channel)
	2 = Mono (uses stereo right channel)*

Notes	Monitor sources and modes denoted with an asterisk (*) are only available with Aria systems that contain an integrated mixer chip.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_SETPATCHBANKXE "Windows Extended Aria Interface:ARIA_SETPATCHBANK"§

This message selects a synthesizer patch bank for a specific MIDI channel.  Up to 128 different instrument patch banks are allowed.

Use message ARIA_LOADPATCHBANK to first load a specific patch bank file.  The ARIA_SETPATCHBANK message will not take effect until a MIDI program change command is sent to select a patch from the new bank.

Parameter	Description

wParam	Contains the patch bank number to select (0 to 127).  Patch bank number 0 is the default upon loading the Aria driver.

dwParam	The low-order word must contain a MIDI channel number (0 to 15).

ARIA_SETPLAYMODEXE "Windows Extended Aria Interface:ARIA_SETPLAYMODE"§

This message sets the system playback mode.  The playback mode determines the number of synthesis operators and digitized wave channels that are available.

Parameter	Description

wParam	Is not used.

dwParam	Contains one of the following values:

	0 = No synthesis operators, 4 wave channels
	1 = 20 synthesis operators, 2 wave channels
	2 = 32 synthesis operators, 2 wave channels

Notes	When Aria Listener or reverb is active, the driver adjusts the playback mode of the Aria system if necessary.  The ARIA_SETPLAYMODE message should not be used if either of these applications are active.

ARIA_SETRECORDXE "Windows Extended Aria Interface:ARIA_SETRECORD"§

This message sets the recording source for digital audio.

Parameter	Description

wParam	Is not used.

dwParam	Contains any one of the sources in either column below or one from each column (ORed together):

	  1 = Microphone	  2 = CD-ROM output
	  8 = External*		  4 = Synth+wave output
	16 = Auxiliary*		32 = PC Speaker/Telephone*

If the value is set to zero, all recording signals will be disabled.

Notes	Items denoted with an asterisk (*) are only available with Aria systems that contain an integrated mixer chip.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_SETSYSTEMXE "Windows Extended Aria Interface:ARIA_SETSYSTEM"§

This message enables or disables the Aria system.  When disabled, the Aria system is placed in Sound Blaster emulation mode.

Parameter	Description

wParam	Is not used.

dwParam	Contains a value of 0 to disable the system or a value of 1 to enable the system.

ARIA_SETVOLUMEXE "Windows Extended Aria Interface:ARIA_SETVOLUME"§XE "Volume"§

This message sets the left and right volume level for a specific audio device.

Parameter	Description

wParam	Contains a value indicating which volume level to set:

	0 = Master volume
	1 = Synthesizer volume
	2 = Wave volume
	3 = Microphone volume
	4 = CD-ROM volume
	5 = External volume*
	6 = Auxiliary volume*
	7 = PC Speaker/Telephone volume*

dwParam	The high-order word of the value must be the right channel volume and the low-order word must be the left channel volume.  Volume levels for each channel can range from 0 to 0x7FFF.

Notes	Devices denoted with an asterisk (*) are only available with Aria systems that contain an integrated mixer chip.  If in doubt, call the AriaMsg function with the message ARIA_GETCAPS to determine mixer support.

ARIA_UNLOADDSPAPPXE "Windows Extended Aria Interface:ARIA_UNLOADDSPAPP"§XE "DSP Applications"§

This message unloads a DSP application from the Aria system.

Parameter	Description

wParam	Contains the DSP application number to unload (0 to 15).

dwParam	Is not used.

ARIA_UNLOADPATCHBANKXE "Windows Extended Aria Interface:ARIA_UNLOADPATCHBANK"§

This message unloads a previously loaded synthesizer patch bank.  Up to 127 different instrument patch banks are allowed.  To load a patch bank, use the ARIA_LOADPATCHBANK message.

Parameter	Description

wParam	Contains the patch bank number to unload (1 to 127).

dwParam	Is not used.

< This page intentionally left blank >