# APPENDIX A0	PATCH BANK FORMATS§§

A patch bank consists of up to 255 instrument definitions that can be indexed by an audio device driver or application.  The General MIDI patch banksXE "General MIDI Mode"§ provided with this development kit, "GENMIDI1.BNK" and "GENMIDI2.BNK" define 128 melodic instruments and at least 33 percussive instruments each.  "GENMIDI1.BNK" is for use with the 512 Kbyte ROM and "GENMIDI2.BNK" is for use with the 1024 Kbyte ROM.

Percussion patches are mapped to individual MIDI keys by specifying the Patch ID as key number plus 128.  For example, the acoustic snare on MIDI key 38 will have Patch ID 166.

The structure of a Version 1 patch bank is as follows:

| PATCH BANK HEADER | 15 bytes |
| --- | --- |
| PATCH DEFINITIONS | 56 bytes each |
| OPERATOR DEFINITIONS | 84 bytes each |

PATCH BANK HEADER - Version 1

| Name | Byte Offset | Size (in bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| Version Bank Name Patch Count | 0 1 14 | 1 13 1 | BYTE CHAR BYTE | Version number (will be 1) Zero-terminated ASCII string Number of patches in bank (range: 1 - 255) |

PATCH DEFINITION - Version 1

(repeated for each required patch following the PATCH BANK HEADER)

| Name | Byte Offset | Size (in bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| Patch Name Level Op ID 1 Op ID 2 ¦ Op ID 8 Patch ID Min. Operators Total Operators | 0 13 14 16 28 30 31 32 | 13 1 2 2 2 1 1 1 | CHAR BYTE WORD WORD WORD BYTE BYTE BYTE | Zero-terminated ASCII name Default volume level (range: 0 - 127) Operator Identifier 1 refers to Operator Definition (range: 1 - 2048) Operator Identifier 2 Operator Identifier 8 Patch Number (range: 0 - 127 melodic, 128 - 255 percussive) Specifies the minimum number of operators required by this patch. A larger number may be defined, but any over this minimum value may be terminated during periods when many notes are playing (range: 1 - 8) Total number of operators defined by this patch (range: 1 - 8) |

PATCH DEFINITION - Version 1 (continued)

| Name | Byte Offset | Size (in bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| Pitch Wheel Vibrato Speed Vibrato Shape Vibrato Depth Tremolo Speed Tremolo Shape Tremolo Depth Pan Mod Speed Pan Mod Shape Pan Mod Depth Controller ID 1 Controller ID 2 Controller ID 3 Controller Depth 1 Controller Depth 2 Controller Depth 3 Controller Use 1 Controller Use 2 Controller Use 3 Pan | 33 34 36 37 38 40 41 42 44 45 46 47 48 49 50 51 52 53 54 55 | 1 2 1 1 2 1 1 2 1 1 1 1 1 1 1 1 1 1 1 1 | BYTE WORD BYTE BYTE WORD BYTE BYTE WORD BYTE BYTE BYTE BYTE BYTE BYTE BYTE BYTE BYTE BYTE BYTE BYTE | Range of pitch wheel effect. Sensitivity value is the number of semitones up and down (range: 0 - 24) Vibrato speed (0.1 Hz per unit) (range: 0 - 500) Vibrato modulation type: 0 Ramp 1 Reverse ramp 2 Triangle 3 Square 4 Pulse (25% duty cycle) 5 Staircase 6 Sine wave Vibrato strength (range: 0 - 255) Tremolo speed (0.1 Hz per unit) (range: 0 - 500) Tremolo modulation type (same as Vibrato Shape) Tremolo strength (range: 0 - 255) Pan modulation speed (0.1 Hz per unit) (range: 0 - 500) Pan modulation type (same as Vibrato Shape) Pan modulation strength (range: 0 - 255) MIDI control number used to modify patch parameters (range: 1 - 127, 0 = off) Maximum controller value (range: 1 - 127) Parameter to affect: 0 Vibrato Speed 1 Vibrato Depth 2 Tremolo Speed 3 Tremolo Depth 4 Pan Modulation Speed 5 Pan Modulation Depth Default stereo pan value (range: 0 - 127) |

OPERATOR DEFINITION - Version 1

(repeated for all required operators following the last PATCH DEFINITION)

| Name | Byte Offset | Size (in bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| Operator ID Detune Transpose Velocity Table Balance Key Bias Operator Amplitude Operator Envelope 1 Operator Envelope 2 ¦ Operator Envelope 8 | 0 2 4 6 7 8 9 10 12 24 | 2 2 2 1 1 1 1 2 2 2 | WORD short short BYTE BYTE BYTE low 4 bits only BYTE WORD WORD WORD | Identifier referred to by the Patch Definition (range: 1 - 2048) Number of cents by which pitch is to be altered (range: -50 to +50) Number of semitones to transpose this operator (range: -127 to +127) Selects preset velocity curves: 0 Ignore velocity (use 64) 1 Standard (linear increase from velocity 1 through 127 in equal increments) 2 Strong increase 3 Extreme increase 4 Standard reverse 5 Strong reverse 6 Extreme reverse By using a reverse algorithm on one operator while using the standard on another, loud notes will be dominated by the operator using the standard curve and soft notes by the reverse curve. Adjusts operator balance in stereo field relative to other operators (range: 0 [left] - 64 [center] - 127 [right]) Used to increase or decrease relative amplitudes of operators from MIDI note 0 to MIDI note 127: 0 No amplitude adjustment 1 Standard increase 2 Strong increase 3 Standard decrease 4 Strong decrease Adjusts operator relative to other operators. Amplitude doubles every 16 units (range: 1 - 127) Envelope stage 1 definition: LS 6 bits Determines slope of amplitude envelope phase (range: 0 - 63) MS 10 bits Length of envelope phase in sec. x 344.5 (range: 1 - 1022) Envelope stage 2 definition Envelope stage 8 definition |

OPERATOR DEFINITION - Version 1 (continued)

| Name | Byte Offset | Size (in bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| Wave Index 1 Wave Block Count 1 Wave Loop Flag and Base Pitch 1 Wave Low Pitch 1 Wave High Pitch 1 ¦ ¦ Wave Index 8 Wave Block Count 8 Wave Loop/Base 8 Wave Low Pitch 8 Wave High Pitch 8 Minimum Pitch Maximum Pitch Minimum Velocity Maximum Velocity Pitch Scale Range Unused Unused Unused | 26 28 29 30 31 68 70 71 72 73 74 75 76 77 78 79 80 82 | 2 1 1 1 1 2 1 1 1 1 1 1 1 1 1 1 2 2 | WORD BYTE BYTE BYTE BYTE WORD BYTE BYTE BYTE BYTE BYTE BYTE BYTE BYTE BYTE BYTE WORD WORD | Waveform Table to index (range: 0 - 6143) Number of 256 byte wave blocks forming table (range: 0 - 255) LS 7 bits MIDI pitch at which this wave table will play when output at 22.05 KHz (range: 0 - 127) MS 1 bit 0 = multiple loop waveform 1 = single loop waveform Lowest MIDI pitch at which this wave table will be used (range: 0 - 127) Highest MIDI pitch at which this wave table will be used (range: 0 - 127) Wave information repeated for 8 wave tables. Low/High Pitch may not overlap for this version. If less than 8 wave tables are required, subsequent definitions should set Wave Block Count equal to zero. Lowest MIDI pitch at which this operator will be used (range: 0 - 127) Highest MIDI pitch at which this operator will be used (range: 0 - 127) Lowest velocity at which this operator will be used (range: 0 - 127) Highest velocity at which this operator will be used (range: 0 - 127) Expands an octave pitch range over more than one octave of MIDI notes. 0 = no scaling 1 = 1:2 octave scaling 2 = 1:4 octave scaling 3 = 1:8 octave scaling Reserved for future use Reserved for future use Reserved for future use |

CONDENSED PATCH BANK FORMAT

A condensed or version 2 patch bank consists of up to 255 instrument definitions that can be indexed by an audio device driver or application.  The condensed General MIDI patch banksXE "General MIDI Mode"§ provided with this development kit, "GM1.BNK" and "GM2.BNK" define 128 melodic instruments and 33 percussive instruments each.  "GM1.BNK" is for use with the 512 Kbyte ROM and "GM2.BNK" is for use with the 1024 Kbyte ROM.

Percussion patches are mapped to individual MIDI keys by specifying the Patch ID as key number plus 128.  For example, the acoustic snare on MIDI key 38 will have Patch ID 166.

The structure of a Version 2 condensed patch bank is as follows:

| PATCH BANK HEADER | 2 bytes |
| --- | --- |
| PATCH DEFINITIONS | 37 bytes each |
| OPERATOR DEFINITIONS | 30 + (6 * n) bytes each, where n is the number of wave definitions per operator. |

PATCH BANK HEADER - Version 2

| Name | Byte Offset | Size (in bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| Version Patch Count | 0 1 | 1 1 | BYTE BYTE | Version number (will be 2) Number of patches in bank (range: 1 - 255) |

PATCH DEFINITION - Version 2

(repeated for each required patch following the PATCH BANK HEADER)

| Name | Byte Offset | Size (in bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| Patch ID Level Total Operators Min. Operators Op Index 1 Op Index 2 ¦ Op Index 8 | 0 1 2 3 4 6 18 | 1 1 1 1 2 2 2 | BYTE BYTE BYTE BYTE WORD WORD BYTE | Patch Number (range: 0 - 127 melodic, 128 - 255 percussive) Default volume level (range: 0 - 127) Total number of operators defined by this patch (range: 1 - 8) Specifies the minimum number of operators required by this patch. A larger number may be defined, but any over this minimum value may be terminated during periods when many notes are playing (range: 1 - 8) Operator Indices give offset in bytes from beginning of file to Operator Definitions Operator Index 2 Operator Index 8 |

PATCH DEFINITION - Version 2 (continued)

| Name | Byte Offset | Size (in bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| Pitch Wheel Vibrato Speed Vibrato Shape Vibrato Depth Tremolo Speed Tremolo Shape Tremolo Depth Pan Mod Speed Pan Mod Shape Pan Mod Depth Controller ID Controller Depth Controller Use Pan | 20 21 23 24 25 27 28 29 31 32 33 34 35 36 | 1 2 1 1 2 1 1 2 1 1 1 1 1 1 | BYTE WORD BYTE BYTE WORD BYTE BYTE WORD BYTE BYTE BYTE BYTE BYTE BYTE | Range of pitch wheel effect. Sensitivity value is the number of semitones up and down (range: 0 - 24) Vibrato speed (0.1 Hz per unit) (range: 0 - 500) Vibrato modulation type: 0 Ramp 1 Reverse ramp 2 Triangle 3 Square 4 Pulse (25% duty cycle) 5 Staircase 6 Sine wave Vibrato strength (range: 0 - 255) Tremolo speed (0.1 Hz per unit) (range: 0 - 500) Tremolo modulation type (same as Vibrato Shape) Tremolo strength (range: 0 - 255) Pan modulation speed (0.1 Hz per unit) (range: 0 - 500) Pan modulation type (same as Vibrato Shape) Pan modulation strength (range: 0 - 255) MIDI control number used to modify patch parameters (range: 1 - 127, 0 = off) Maximum controller value (range: 1 - 127) Parameter to affect: 0 Vibrato Speed 1 Vibrato Depth 2 Tremolo Speed 3 Tremolo Depth 4 Pan Modulation Speed 5 Pan Modulation Depth Default stereo pan value (range: 0 - 127) |

OPERATOR DEFINITION - Version 2

(repeated for all required operators following the last PATCH DEFINITION)

| Name | Byte Offset | Size (in bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| Detune Transpose Velocity Table Balance Key Bias Operator Amplitude Operator Envelope 1 Operator Envelope 2 ¦ Operator Envelope 8 Minimum Pitch Maximum Pitch Minimum Velocity Maximum Velocity Pitch Scale Range | 0 2 4 5 6 7 8 10 22 24 25 26 27 28 | 2 2 1 1 1 1 2 2 2 1 1 1 1 1 | short short BYTE BYTE BYTE low 4 bits only BYTE WORD WORD WORD BYTE BYTE BYTE BYTE BYTE | Number of cents by which pitch is to be altered (range: -50 to +50) Number of semitones to transpose this operator (range: -127 to +127) Selects preset velocity curves: 0 Ignore velocity (use 64) 1 Standard (linear increase from velocity 1 through 127 in equal increments) 2 Strong increase 3 Extreme increase 4 Standard reverse 5 Strong reverse 6 Extreme reverse By using a reverse algorithm on one operator while using the standard on another, loud notes will be dominated by the operator using the standard curve and soft notes by the reverse curve. Adjusts operator balance in stereo field relative to other operators (range: 0 [left] - 64 [center] - 127 [right]) Used to increase or decrease relative amplitudes of operators from MIDI note 0 to MIDI note 127: 0 No amplitude adjustment 1 Standard increase 2 Strong increase 3 Standard decrease 4 Strong decrease Adjusts operator relative to other operators. Amplitude doubles every 16 units (range: 1 - 127) Envelope stage 1 definition: LS 6 bits Determines slope of amplitude envelope phase (range: 0 - 63) MS 10 bits Length of envelope phase in sec. x 344.5 (range: 1 - 1022) Envelope stage 2 definition Envelope stage 8 definition Lowest MIDI pitch at which this operator will be used (range: 0 - 127) Highest MIDI pitch at which this operator will be used (range: 0 - 127) Lowest velocity at which this operator will be used (range: 0 - 127) Highest velocity at which this operator will be used (range: 0 - 127) Expands an octave pitch range over more than one octave of MIDI notes. 0 = no scaling 1 = 1:2 octave scaling 2 = 1:4 octave scaling 3 = 1:8 octave scaling |

OPERATOR DEFINITION - Version 2 (continued)

| Name | Byte Offset | Size (in bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| Wave Count Wave Index 1 Wave Block Count 1 Wave Loop Flag and Base Pitch 1 Wave Low Pitch 1 Wave High Pitch 1 ¦ ¦ Wave Index n Wave Block Count n Wave Loop/Base n Wave Low Pitch n Wave High Pitch n | 29 30 32 33 34 35 | 1 2 1 1 1 1 2 1 1 1 1 | BYTE WORD BYTE BYTE BYTE BYTE WORD BYTE BYTE BYTE BYTE | Number of Wave Definitions (n) that follow (range: 0 - 8) Waveform Table in ROM to index (range: 0 - 6143) Number of 256 byte wave blocks forming table (range: 0 - 255) LS 7 bits MIDI pitch at which this wave table will play when output at 22.05 KHz (range: 0 - 127) MS 1 bit 0 = multiple loop waveform 1 = single loop waveform Lowest MIDI pitch at which this wave table will be used (range: 0 - 127) Highest MIDI pitch at which this wave table will be used (range: 0 - 127) Wave information repeated for n wave tables. Low/High Pitch may not overlap for this version. |

XE "Condensed Patch Bank Format" \r "CondensedFormat"§XE "Patch Bank Format" \r "PatchBankFormat"§