DISK TECHNICIAN(R)

FACTORY TEST FOR SEAGATE(R) DRIVES

USER GUIDE

Version 1.22

Disk Technician Factory Test For Seagate Drives

is a Disk Technician Corporation product,

distributed by Seagate Technology, Inc.

License Agreement:

Ownership of the Disk Technician(R) Factory Test For Seagate

Drives programs remains exclusively with Disk Technician

Corporation, which provides the Disk Technician Factory Test For

Seagate Drives programs. License to use these programs is granted

only to Seagate Technology(R) Inc. authorized OEMs, system

integrators, distributors and their dealers.

WARNING! Unauthorized or uninformed use of this product may result

in irretrievable data loss, or other damage.

You may use these programs and make copies thereof. All copies

must be full and complete and must include the copyright notice.

You may not rent, sell, assign, lend, give, or otherwise transfer

the Disk Technician Factory Test For Seagate Drives programs to

any person. Any attempt to transfer the Disk Technician Factory

Test For Seagate Drives programs or any rights under this License

is void.

YOU MAY NOT ALTER, DIVIDE, BREAK, MODIFY, ADAPT, TRANSLATE,

DECOMPILE, DISASSEMBLE, OR REVERSE ENGINEER THE DISK TECHNICIAN

FACTORY TEST FOR SEAGATE DRIVES PROGRAMS IN ANY MANNER FOR ANY

PURPOSE WHATSOEVER. YOU MAY NOT USE THE PROGRAMS OR ANY PORTIONS

THEREOF IN ANY DERIVATIVE WORKS.

This License is effective until terminated. You may terminate it

by destroying the programs and supporting materials together with

all copies in any form. This License will also terminate if you

fail to comply with any terms or condition of this License. You

agree upon such termination to destroy the programs and all

supporting materials together with all copies.

This License is governed by the laws of the State of California.

Disclaimer of Warranty:

The Disk Technician Factory Test For Seagate Drives programs are

furnished to you by Disk Technician Corporation. There is no

warranty provided. If the programs or any materials provided

therewith are defective in any way, notify Disk Technician

Corporation immediately. Disk Technician Corporation does not

warrant that the functions contained in the programs will meet

your requirements or that the operation of the programs will be

uninterrupted or error-free. THE PROGRAMS CONTAINED IN THIS

PACKAGE ARE PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND, EITHER

EXPRESSED OR IMPLIED, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE. THE ENTIRE RISK RELATED TO THE QUALITY AND PERFORMANCE OF

THE PROGRAMS IS ON YOU. IN THE EVENT THERE IS ANY DEFECT, YOU

ASSUME THE ENTIRE COST OF ALL NECESSARY SERVICING, REPAIR, OR

CORRECTION.

i

This agreement constitutes the complete and exclusive statement of

the terms of the agreement between you and Disk Technician

Corporation. It supersedes and replaces any previous written or

oral agreements and communications relating to the software. No

oral or written information or advice given by Disk Technician

Corporation, its dealers, distributors, agents or employees will

create any warranty or in any way increase the scope of the

warranty provided in this agreement, and you may not rely on any

such information or advice.

Patents, Servicemarks, Trademarks, Copyrights:

U.S. and foreign patent(s) pending.

Disk Technician(R) is a registered trademark of Disk Technician

Corporation. Finds And Fixes Hard Disk Problems While You Use Your

Other Software(SM), and We Make Technology Easy and Affordable(SM)

are servicemarks of Disk Technician Corporation. Seagate(R),

Seagate Technology(R), and the Seagate(R) logo are registered

trademarks of Seagate Technology Incorporated. All other

trademarks and servicemarks are acknowledged.

(C)1991-93 DISK TECHNICIAN CORPORATION. All rights reserved.

ii

*****************************************************************

Table of Contents:

*****************************************************************

Legal...........................................................i

Introduction....................................................1

Requirements....................................................2

Setup...........................................................3

Operating Instructions..........................................4

Using The Test Results..........................................6

Product Support Services........................................6

Appendix A: Eligible Seagate Drives

And Matching Controllers/Adapters...................7

Appendix B: Installing Jumpers And Terminators.................11

Appendix C: White Paper On Disk Technician Factory Test

For Seagate Drives.................................12

Appendix D: Disk Technician Gold And The Rest

of The Hard Drive Reliability Story................17

Appendix E: White Paper On Disk Technician Gold................18

iii

Disk Technician Factory Test for Seagate Drives Manual

*****************************************************************

Introduction:

*****************************************************************

Disk Technician Factory Test For Seagate Drives puts the power

of Seagate factory testing in your hands. It requires no special

hardware and is fast and easy to use. Its two tests assure the

reliability of Seagate drives for you and your customers:

* A Quick test rapidly identifies gross failures.

* Media certification creates a new, updated media defect map

and recertifies the drive for normal use.

Disk Technician Gold(R), the retail end-user version, puts the

power of Seagate factory testing in your customers' hands. It

makes hard discs continuously self-repairing. It's the first hard

disc preventive maintenance software that works all of the time

-- in the background.

Disk Technician Factory Test For Seagate Drives was developed

especially for you. That makes you our most highly-valued source

for how to make it better. We need your suggestions. Please let

us hear from you.

1

Disk Technician Factory Test for Seagate Drives Manual

*****************************************************************

Requirements:

*****************************************************************

* AT-286, -386, or -486 class machine with at least 640K of

RAM and a 1.2 megabyte 5.25" or 1.44 megabyte 3.5" floppy

drive.

NOTE: Some older AT-286 clones do not support AT (IDE)

drives. If you are using one of these early computers as

a test system, you may find that you are unable to test

certain drives unless you update the computer's BIOS.

Also, make certain that your test system is set-up so

that, if a diskette is loaded into the floppy drive, it

will boot from the floppy drive.

* The recommended controller/adapter for the hard drive to be

tested. See "Appendix A" for details.

NOTE: To precisely match Seagate factory testing and

assure your customers that their hard drives have been

recertified on that basis, it is best use the recommended

controller or adapter.

Disk Technician Factory Test For Seagate Drives may

operate with controllers and adapters not listed in the

README.DOC. However, these have not been tested or

certified with Disk Technician Factory Test For Seagate

Drives and may give invalid results.

* A printer.

* A MS-DOS 5.0 or 6.0 bootable 1.2 megabyte 5.25" or 1.44

megabyte 3.5" diskette.

2

Disk Technician Factory Test for Seagate Drives Manual

*****************************************************************

Setup:

*****************************************************************

Copy all files from the Disk Technician Factory Test For Seagate

Drives distribution diskette to the DOS 5.0 or 6.0 bootable

diskette.  This will be your work copy.

3

Disk Technician Factory Test for Seagate Drives Manual

*****************************************************************

Operating Instructions:

*****************************************************************

Because Disk Technician Factory Test For Seagate Drives

automatically configures itself to each Seagate drive, you will

NOT need to:

* Change any CMOS settings when setting-up or testing

different drives.

* DOS format or FDISK any drive prior to testing.

WARNING!!! WARNING!!! WARNING!!! WARNING!!! WARNING!!! WARNING!!!

All data on the drive being tested will be destroyed.

For nondestructive testing, use Disk Technician Gold.

WARNING!!! WARNING!!! WARNING!!! WARNING!!! WARNING!!! WARNING!!!

1.  Make certain that the power is off, then install the

Seagate hard drive and recommended controller/adapter in

the test system. See "Appendix A" for details. Please be

certain that:

* You have correctly installed all jumpers and

terminators on the drive to be tested. See "Appendix B"

for details.

* You are using the recommended controller or adapter.

See "Appendix A" for details.

* For MFM and RLL (ST412-type) drives, confirm that the

interleave is set at 1 prior to testing. If it is not,

low-level format with an interleave value of 1.

2.  Be sure the printer is on and connected.

3.  Turn on the computer and boot from the Disk Technician

Factory Test For Seagate Drives work copy diskette.

The opening screen will appear with a flashing message that

Disk Technician Factory Test For Seagate Drives is

"Configuring To The Test Drive. Please Wait." When

configuration is complete, you will see another flashing

message, "Press any key to continue." Press any key to go

to the "Tests" menu.

NOTE: Disk Technician Factory Test For Seagate Drives may

not automatically recognize all drives. In these cases,

you will see a list of eligible drives on screen.

Highlight the correct drive and then press ENTER to

continue.

4

Disk Technician Factory Test for Seagate Drives Manual

4.  Press ENTER to start the "Quick test." The "Quick test"

screen will appear and testing will start.

Or use the up/down arrow keys to select "Media

certification." (If you select "Media certification," go to

step 7 now.)

The "Quick test" rapidly identifies gross failures,

including the inability to seek and/or to perform

read/write operations. The "Quick test" does the following

operations on the drive:

* Seek testing.

* Quick read and write testing of the entire drive for

soft and hard errors.

5.  When the test is finished, you will see the "Quick test

report," with a pass/fail result. Type P for a printed

report.

NOTE: To remove from service all new defect locations

detected by Disk Technician Factory Test For Seagate

Drives, simply perform a high-level format under

the appropriate operating system (DOS, UNIX, etc.).

6.  Press the Escape key to return to the "Tests" menu. Or

press F10 to quit.

NOTE: If you choose to quit now, skip instructions 7-9

and go to the last section, "Using The Test Results. "To

test a different drive now, turn off the computer and

begin again from step 1, above."

7.  Use the up/down arrow keys to select "Media certification,"

then press ENTER to start testing. The "Media

certification" screen will appear and testing will start.

Media certification creates a new, updated media defect map

and recertifies the drive for normal use. It performs the

following operations on the entire drive:

* Seek testing.

* Comprehensive read and write testing of the entire

drive for soft and hard errors.

* Media defect identification.

5

Disk Technician Factory Test for Seagate Drives Manual

8.  When the test is finished, you will see the "Media

certification report," with a pass/fail result. Type P for

a printed report.

Save the printed report and return it along with the

customer's drive.

NOTE: To remove from service all new defect locations

detected by Disk Technician Factory Test For Seagate

Drives, simply perform a high-level format under

the appropriate operating system (DOS, UNIX, etc.).

9.  Press the Escape key to return to the "Tests" menu. Or

press F10 to quit.

NOTE: To test a different drive now, turn off the

computer and begin again from step 1, above.

*****************************************************************

Using The Test Results:

*****************************************************************

1.  For optimal customer satisfaction: if the test drive passes

the "Quick test," it is best to perform "Media

certification" to assure that the drive is recertified for

normal use.

2.  Include a copy of the test report with the hard drive when

you return it to your customer. This will provide assurance

that the drive has been recertified for normal service by

Seagate factory testing.

*****************************************************************

Product Support Services:

*****************************************************************

For product support and troubleshooting, please call Disk

Technician Technical Support at 619 272 4000. Technical Support

is available Monday through Friday, 8 a.m. to 5 p.m., Pacific

Time.

Support is also available on the Disk Technician Corporation

electronic Bulletin Board System (BBS). It can be reached by

modem 24 hours a day, 7 days a week, at 619 272 9240.

Disk Technician Factory Test For Seagate Drives is continuously

updated. New revisions can be obtained from the Seagate

Technology Bulletin Board System (BBS). Seagate's BBS can be

reached by modem 24 hours a day, 7 days a week, at 408 438 8771.

6

Disk Technician Factory Test for Seagate Drives Manual

*****************************************************************

Appendix A:

Eligible Seagate Drives And Matching Controllers/Adapters

*****************************************************************

Disk Technician Factory Test for Seagate Drives V1.22.

The following are Seagate drives supported by version 1.22.  Also

listed are the controllers and adapters we recommend you use.

Controller and adapter numbers prefaced with ST are products of

Seagate, those prefaced with TMC are products of Future Domain.

NOTE: To precisely match Seagate factory testing and assure

your customers that their hard drives have been recertified on

that basis, it is best use the recommended controller or

adapter. Disk Technician Factory Test For Seagate Drives may

operate with controllers and adapters not listed in the

README.DOC. However, these have not been tested or certified

with Disk Technician Factory Test For Seagate Drives and may

give invalid results.

=================================================================

Model Number   Controller/Adapter

------------   ------------------

ST1057A        ST08

ST1090A        ST08

ST1090N        TMC1670/TMC1680

ST1096N        TMC1670/TMC1680

ST1100         ST22M

ST1102A        ST08

ST1106R        ST22R

ST1111A        ST08

ST1111N        TMC1670/TMC1680

ST11200N       TMC1670/TMC1680

ST11201N       TMC1670/TMC1680

ST1126A        ST08

ST1126N        TMC1670/TMC1680

ST1133A        ST08

ST1133N        TMC1670/TMC1680

ST1144A        ST08

ST1150R        ST22R

ST1156A        ST08

ST1156N        TMC1670/TMC1680

ST1162A        ST08

ST1162N        TMC1670/TMC1680

ST11700N       TMC1670/TMC1680

ST11701N       TMC1670/TMC1680

ST11750N       TMC1670/TMC1680

ST11751N       TMC1670/TMC1680

ST1186A        ST08

ST1186N        TMC1670/TMC1680

ST1201A        ST08

ST1201N        TMC1670/TMC1680

ST1239A        ST08

ST1239N        TMC1670/TMC1680

ST124          ST22M

ST12400N       TMC1670/TMC1680

ST12401N       TMC1670/TMC1680

ST125          ST22M

7

Disk Technician Factory Test for Seagate Drives Manual

ST12550N       TMC1670/TMC1680

ST12551N       TMC1670/TMC1680

ST125A         ST08

ST125N         ST02/TMC1670/TMC1680

ST138          ST22M

ST138A         ST08

ST138N         ST02/TMC1670/TMC1680

ST138R         ST22R

ST1400A        ST08

ST1400N        TMC1670/TMC1680

ST1401A        ST08

ST1401N        TMC1670/TMC1680

ST1480A        ST08

ST1480N        TMC1670/TMC1680

ST1481N        TMC1670/TMC1680

ST151          ST22M

ST157A         ST08

ST157N         TMC1670/TMC1680

ST157R         ST22R

ST1581N        TMC1670/TMC1680

ST177N         ST02/TMC1670/TMC1680

ST1980N        TMC1670/TMC1680

ST2106N        TMC1670/TMC1680

ST212          ST22M

ST2125N        TMC1670/TMC1680

ST213          ST22M

ST2209N        TMC1670/TMC1680

ST225          ST22M

ST225N         ST02/TMC1670/TMC1680

ST225R         ST22R

ST2274A        ST08

ST2383A        ST08

ST2383N        TMC1670/TMC1680

ST238R         ST22R

ST2502N        TMC1670/TMC1680

ST250R         ST22R

ST251          ST22M

ST251N         ST02/TMC1670/TMC1680

ST252          ST22M

ST253          ST22M

ST274A         ST08

ST277N         ST02/TMC1670/TMC1680

ST277R         ST22R

ST278R         ST22R

ST279R         ST22R

ST280A         ST08

ST296N         ST02/TMC1670/TMC1680

ST3051A        ST08

ST3096A        ST08

ST31200N       TMC1670/TMC1680

ST3120A        ST08

ST3123A        ST08

ST3144A        ST08

ST3145A        ST08

ST3243A        ST08

ST325A/X       ST08

ST325N         TMC1670/TMC1680

ST3283A        ST08

8

Disk Technician Factory Test for Seagate Drives Manual

ST3283N        TMC1670/TMC1680

ST3290A        ST08

ST3385         ST08

ST3390A        ST08

ST3390N        TMC1670/TMC1680

ST3500         ST08

ST3500N        TMC1670/TMC1680

ST351A/X       ST08

ST3550         ST08

ST3550N        TMC1670/TMC1680

ST3600         ST08

ST3600N        TMC1670/TMC1680

ST3610N        TMC1670/TMC1680

ST3655A        ST08

ST3655N        TMC1670/TMC1680

ST4026         ST22M

ST4038         ST22M

ST4038M        ST22M

ST4051         ST22M

ST4053         ST22M

ST406          ST22M

ST4085         ST22M

ST4086         ST22M

ST4096         ST22M

ST4097         ST22M

ST412          ST22M

ST41200        TMC1670/TMC1680

ST4135R        ST22R

ST4144R        ST22R

ST41520N       TMC1670/TMC1680

ST41600N       TMC1670/TMC1680

ST41601N       TMC1670/TMC1680

ST41650N       TMC1670/TMC1680

ST41651N       TMC1670/TMC1680

ST4182N        TMC1670/TMC1680

ST419          ST22M

ST42000N       TMC1670/TMC1680

ST42100N       TMC1670/TMC1680

ST42101N       TMC1670/TMC1680

ST42400N       TMC1670/TMC1680

ST43400N       TMC1670/TMC1680

ST43401N       TMC1670/TMC1680

ST43402N       TMC1670/TMC1680

ST4350N        TMC1670/TMC1680

ST4376N        TMC1670/TMC1680

ST4385N        TMC1670/TMC1680

ST4702N        TMC1670/TMC1680

ST4766N        TMC1670/TMC1680

ST4767N        TMC1670/TMC1680

ST506          ST22M

ST9051A        ST08

ST9052A        ST08

ST9077A        ST08

ST9080A        ST08

ST9096A        ST08

ST9100AG       ST08

ST9144A        ST08

ST9145AG       ST08

9

Disk Technician Factory Test for Seagate Drives Manual

ST9235A        ST08

ST9235AG       ST08

ST9235N        TMC1670/TMC1680

ST9295AG       ST08

ST9295N        TMC1670/TMC1680

10

Disk Technician Factory Test for Seagate Drives Manual

*****************************************************************

Appendix B:

Installing Jumpers And Terminators

*****************************************************************

Your Disk Technician Factory Test For Seagate Drives work copy

diskette contains a hypertext file called the "Seagate Technical

Support Desk Reference." This reference contains jumper and

terminator settings for Seagate drives, controllers and adapters.

To use this reference:

1.  From the system prompt, type the following:

SETTINGS and then press ENTER.

2.  Follow the instructions on your screen.

11

Disk Technician Factory Test for Seagate Drives Manual

*****************************************************************

Appendix C: WHITE PAPER

Disk Technician Factory Test For Seagate Drives

And The Elimination of No Problem Found Drive Returns

*****************************************************************

The Problem:

On average, 35% of all drives returned to Seagate Customer

Service Repair at Oklahoma City for repair (in or out of

warranty) have absolutely nothing wrong with them.

These "No Problem Found" (NPF) drives not only waste both Seagate

and reseller resources, but they also cause customer

dissatisfaction and damage to Seagate's valuable reputation for

product quality.

Seagate's Objective:

Seagate's objective was to provide a simple solution to No

Problem Found drive returns by empowering its resellers with the

ability to perform on-site drive testing with factory-level

accuracy. In order to be readily adopted and remain in use, any

proposed solution would have to:

* Be fast. Resellers must clearly perceive the potential for

greater profits as a result of quick drive turnaround and

immediate customer satisfaction.

* Be easy to learn and operate. Training time must be minimal.

Test operators selected by resellers are likely to be non-

technical.

* Precisely match Seagate factory hardware test results. Both

Seagate and the resellers must be assured that results

obtained from the field NPF solution will  correlate to those

of Customer Service Repair (CSR).

* Be affordable. Resellers would be unlikely to participate if

the solution required an investment in special equipment,

periodic recalibration or maintenance.

The Solution:

Disk Technician Factory Test For Seagate Drives is software that

eliminates No Problem Found drive returns from Seagate's direct

accounts (and, eventually, from distributors' dealers). It puts

the power of Seagate factory testing in the resellers' hands.

It is purposefully designed as a quick and easy to use Go/No-Go

tool for non-technical operators. As such, it does not perform

failure analysis. Instead, Factory Test completely exercises and

tests the drive's electronics and mechanics, tests the media,

then produces an authoritative "Pass" or "Fail" report with a

list of any new grown defect locations detected.

12

Disk Technician Factory Test for Seagate Drives Manual

A "Pass" result allows Seagate resellers to immediately return a

No Problem Found drive to their customer with the assurance that

it has been recertified by Seagate factory testing for normal use.

A "Fail" result indicates that the drive can be returned to

Seagate with certainty that the drive requires further analysis

and repair.

Factory Test has been extensively researched and examined by

Seagate Technology and has been found to be an effective method

of testing and recertifying Seagate disc drives. It was developed

in close consultation with Seagate Engineering and is designed to

factory certify Seagate disc drives and avoid needless returns.

It is the most effective substitute available for Seagate factory

testing available.

Disk Technician Factory Test For Seagate Drives achieves all of

Seagate's objectives for eliminating NPFs at the reseller site:

* It is fast.  On average it takes only 2 minutes to test a 40

megabyte IDE drive, although testing time will vary depending

on drive specifications and the number of defects found.

* It is easy to learn and use. Factory Test was designed for

non-technical operators. It automatically recognizes and

configures itself to Seagate drives.

* It has been tested against Seagate factory test hardware on

a large number of hard disc drives to assure that results

precisely match.

* No special equipment is needed.  Any AT-class machine is all

that is required.

* It is provided free to Seagate's direct accounts (and,

eventually, to distributors' dealers).

Requirements:

* An AT-286/386/486 machine with at least 640K of RAM and a

1.2 megabyte 5.25" or 1.44 megabyte 3.5" floppy disc drive.

* The recommended controller or adapter for the hard drive to

be tested (these are listed in the README.DOC file on the

Factory Test distribution diskette).

* A printer.

* A DOS 5.0 bootable 1.2 megabyte 5.25" or 1.44 megabyte 3.5"

diskette.

Setup:

All files from the Factory Test distribution diskette are copied

to the DOS 5.0 bootable diskette. This will become the work copy

diskette.

13

Disk Technician Factory Test for Seagate Drives Manual

Operation:

Factory Test only recognizes and tests Seagate drives. It

automatically configures itself to any Seagate drive in the test

program (these are listed in the README.DOC file on the diskette.

Additional drives may be added at the determination of Seagate).

Because of this, there is no need to verify or change any CMOS

settings, DOS format or FDISK any drive to be tested.

Additionally, because Factory Test overwrites all data on the

drive being tested -- including sectors marked bad, it will test

drives formatted under any operating system (i.e. DOS, UNIX,

Macintosh, etc.).

The operator simply installs the Seagate drive to be tested with

the recommended controller or adapter. (A listing of the correct

jumper and terminator settings for each drive can be found in the

Seagate Technical Support Desk Reference file on the work copy

diskette.)

After making certain that the printer is connected and on, the

operator inserts the work copy diskette into the computer's

floppy drive and turns on the system.

Important! Factory Test overwrites (destroys) all data on the

drive being tested. This destructive test method allows drives to

be tested regardless of the operating system under which they

were formatted (UNIX, DOS, etc.). It also makes testing time much

quicker.

How Factory Test Eliminates NPFs:

Disk Technician Factory Test For Seagate Drives performs

comprehensive NPF testing of the entire hard disc drive subsystem

(including electronics, mechanics and media) in the following

order:

1. When Factory Test begins, it tests host-to-drive

communications. It then issues a drive identification

command to verify that it is operating on a Seagate drive.

2. Factory Test next obtains geometry for the specific drive

from its own internal tables and sets the system parameters

for the drive to be tested.

Then the operator selects either the Quick Test (to rapidly

eliminate gross failures) or Media Certification (to recertify

the drive for normal service).

3. Next, Factory Test does random and step seek tests.

These check the drive electronics as well as the positioning

mechanics (including motor, stepper and positioning system).

4. The guaranteed good sector 0/0 is tested multiple times

with all Seagate test patterns and their inversions.

14

Disk Technician Factory Test for Seagate Drives Manual

5. If the Quick Test is selected, the media is tested for

soft, ECC and hard error locations by reading and writing

to all sectors with a single Seagate test pattern.

6. If Media Certification is selected, the media is tested

for soft, ECC and hard error locations by reading and

writing to all sectors using all Seagate test patterns and

their inversions.

7. In the final stage of both the Quick Test and Media

Certification, defect locations are globally analyzed by an

expert system for patterns indicating electronic, mechanical

and media failures.

The "Pass/Fail" Report

Upon completion of testing, a report appears on-screen with a

"Pass" or "Fail" finding, along with a list of media defect

locations. This report should be printed so that it can accompany

the drive.

The Pass/Fail standard for each Seagate drive has been

predetermined by specifications set by Seagate Engineering.

If the drive passes the Quick Test, it is recommended to perform

Media Certification to assure the customer that the drive is

recertified for normal use.

If the drive passes Media Certification, a copy of the test

report containing the updated list of the new media defect

locations should be included with the drive when it is returned

to the customer. This provides assurance that the drive has been

recertified by Seagate factory testing.

NOTE: To remove from service all new defect locations

detected by Disk Technician Factory Test For Seagate

Drives, simply perform a high-level format under

the appropriate operating system (DOS, UNIX, etc.).

If the drive fails either the Quick Test or Media Certification,

a copy of the test report should be included with the drive when

it is sent to Seagate for analysis and repair. This will

significantly reduce both diagnostic and turnaround time.

Benefits:

* Disk Technician Factory Test is backed by the Seagate

name.

* Seagate resellers now have a resource when "failures"

occur.

* Seagate resellers enjoy more profit, less handling, less

work.

* No Problem Found drives are eliminated by Seagate resellers.

15

Disk Technician Factory Test for Seagate Drives Manual

* Rapid turnaround creates happier, more loyal customers.

* By eliminating NPFs, Seagate's reputation for product

quality is enhanced.

Product Support Services:

For product support and troubleshooting, please call Disk

Technician Technical Support at (619) 272 4000. Technical Support

is available Monday through Friday, 8 a.m. to 5 p.m. Pacific

Time.

Support is also available on the Disk Technician Corporation

Bulletin Board System (BBS). It can be reached by modem 24 hours

a day, 7 days a week, at (619) 272-9240.

Additional drives may be included in the Factory Test program at

the determination of Seagate.

16

Disk Technician Factory Test for Seagate Drives Manual

*****************************************************************

Appendix D:

Disk Technician Gold

And The Rest of

The Hard Drive Reliability Story

*****************************************************************

Seagate drives are born perfect. Before they arrive at your

door, each one must pass a rigorous quality control process

that is the acknowledged world standard. And each drive is

factory certified for long and reliable service.

So why do you get returns? Call it physics, or call it the law

of nature. But, as soon as hard drives begin their long life of

service, a host of microscopic, natural forces also begin a

relentless attack on the various parts. These cause "grown

defects," spots on the media surface that can no longer store

data reliably, and that "grow" after factory certification. As

they evolve from subtle, to marginal, to bad, grown defects wreak

havoc with your customers' data. When that happens, you get a

return. A preventable return.

Disk Technician Gold makes your customers' hard drives

continuously self-repairing. It stops grown defects before they

get a chance to grow up. Operating as a memory-resident device

driver, Disk Technician Gold goes to work automatically every

time the computer is switched on. And it stays on the job

continuously to prevent the very problems that cause drives to be

returned to you.

Disk Technician -- either the Factory Test For Seagate Drives

or the Gold version -- is the only software that uses the same

soft error test standards as factory quality control to monitor

every hard disc access -- only tolerating perfect performance.

However, since Disk Technician Gold is designed to work on "live"

drives loaded with data, its testing is nondestructive. And, when

a problem is detected, Disk Technician Gold not only reports it,

it fixes it. On-the-fly. In the background.

So how can you do fast turnarounds on returns -- and make your

customers happy? Just keep Disk Technician Factory Test For

Seagate Drives on your test bench. And how can you prevent

returns in the first place -- and keep your customers happy? Just

recommend Disk Technician Gold. It's the rest of the hard drive

reliability story.

17

Disk Technician Factory Test for Seagate Drives Manual

*****************************************************************

Appendix E: White Paper On Disk Technician Gold:

Comprehensive Preventive Maintenance

For The Hard Disc Drive and Its Subsystems

*****************************************************************

The Problem of No Problem Found Drive Returns:

Before leaving the factory, each Seagate hard disc drive

undergoes rigorous quality control and is certified for long and

reliable service. Yet, once these drives are installed in end

user computer systems, an array of natural microscopic forces

begins a relentless, continuous attack on the media, electronics

and mechanics. While these forces can be effectively managed in a

laboratory environment through scheduled, proactive diagnostics

and repair, end users can only react to the catastrophic problems

that these forces will eventually cause if left unchecked.

However, problems inherent to the hard disc drive itself are only

a small part of the picture. Failures in other computer

components such as the power supply also affect data reliability

and therefore the end user's perception of hard drive quality.

End users seeing a DOS error message such as DRIVE NOT READY

cannot distinguish between a problem caused by worn or dirty

connectors, intermittent ribbon cables, an intermittent power

supply -- or within the hard drive itself.

Still other causes of perceived hard drive reliability problems

that happen after the drive has left Seagate's reseller channel

include such things as improper drive installation, incorrect

CMOS setup, viruses, CHKDSK errors, improper software

installation, and physical abuse.

In any case, end users are not concerned with inter-industry

engineering issues and intra-industry finger pointing. Fairly or

unfairly, when any problem affects their precious data -- no

matter what the actual cause may be the finger of blame is

invariably pointed at Seagate and the hard drive is returned for

repair or replacement.

It should come as no surprise then, that on average, 35% of all

drives returned to Seagate Customer Service Repair in Oklahoma

City for repair -- in or out of warranty have absolutely nothing

wrong with them. Or that, depending on the reseller, this figure

can go as high as 80%. It is easy to deduce that servicing these

No Problem Found (NPF) drives not only wastes both Seagate and

reseller resources, but also cause customer dissatisfaction and

damage to Seagate's valuable reputation for product reliability

and quality.

End User Resistance to Hard Drive Preventive Maintenance:

The benefits of hard drive preventive maintenance are many,

particularly to business and professional users. Because of the

value of the data stored on their hard drives, these users

typically own one or more hard disc utility software packages.

18

Disk Technician Factory Test for Seagate Drives Manual

Unfortunately, users rarely remember to run these utilities until

after a severe problem has occurred, too late for preventive

measures. Nor are these utilities preventive in the proper sense

of the word, in that they cannot detect or measure soft errors.

Further, the sole function these utilities provide is media

surface scanning, which does not address the many other (and much

more prevalent) causes of hard drive reliability problems.

In addition, these utilities took from 1-1/2 to 5-1/2 hours to

run on a 20 megabyte drive, which discouraged regularly scheduled

maintenance. However, today's desktop computer is much more

likely to have a 120 megabyte drive, and many new systems are

shipped with 200 megabyte or greater drives. The effect of this

is that critically necessary hard drive maintenance time has now

eclipsed potential computer production time.

With the decreasing cost of hardware and increasing

sophistication of software applications, the PC has assumed many

corporate mission-critical tasks that were previously in the

domain of mainframes and minis, including things such as

accounting, vendor and customer records, engineering and

manufacturing process control. For business users, however, PC

hard drive data reliability has remained the single, unresolved

issue.

Seagate's Objective:

While Disk Technician Factory Test provides Seagate's resellers

with a go/no-go software solution to eliminate NPFs once drives

have arrived back in the channel (for more information, see the

Factory Test White Paper), no one would disagree that it would be

far preferable and mutually profitable to prevent these drives

from being returned in the first place. Further, suppose a

comprehensive tool could be developed which could detect and

eliminate all of the previously mentioned problems affecting data

reliability? Suppose that it could be installed right in the

computer operating system, such that hard drive preventive

maintenance would not require end user involvement? One result

would surely be that unnecessary drive returns would plummet to

virtually zero. Everyone -- from end users, to dealers, VARs or

VMRs, to distributors, to System Integrators, to OEMs, to Seagate

itself -- would enjoy the benefits of self-repairing hard drives.

Disk Technician Corporation's Objective:

Disk Technician Corporation wanted to develop a user-independent

solution to both inherent and environmentally induced hard drive

problems, as well as non-hard drive problems that affect data

reliability and hence are perceived to be hard drive-related. In

order to be an acceptable end user desktop solution, such a

product would need to:

* Detect and eliminate grown defects. Media defects develop

continuously throughout a hard drives service life. These

must be eliminated in their soft stage, before they cause

lost and corrupted data, or precipitate a crash.

19

Disk Technician Factory Test for Seagate Drives Manual

* Detect imminent electronic, mechanical and media problems in

the hard drive. If undetected, these can cause sudden,

catastrophic failures.

* Detect non-hard drive computer component problems affecting

data reliability. These can cause lost and corrupted data as

well as sudden, catastrophic failure.

* Detect other non-hard drive issues affecting data reliability

(including CHKDSK errors, viruses, improper software

installation, improper drive installation and setup, etc.).

* Precisely match Seagate factory hardware test results.

Seagate, resellers and end users must be assured that testing

results obtained from the end user preventive maintenance

product correlate to Seagate Customer Service Repair's.

* Be fast. End users will not adopt a product that ties-up

their systems for any significant period of time.

* Be easy to learn and operate. A pragmatic hard drive

preventive maintenance product must assume that the majority

of its users will be non-technical and have no desire to

learn the intricacies of hard drives or complex preventive

maintenance, repair and recovery procedures.

* Be affordable. Even "must-have" desktop solutions must be

inexpensive in order to become widely adopted.

The Solution? Build Both Prevention and Cure Right Into The

System:

Disk Technician Gold is a retail product (suggested list price,

US$149.95) designed to be installed on end user hard disc drives.

It works constantly in the background to prevent media errors and

provide advance warning of electronic and mechanical errors as

the drive functions. Once Disk Technician Gold is installed, no

further user interaction is required. It replaces traditional

hard disc maintenance utilities where the user must remember to

run the software. Disk Technician Gold begins to work the moment

the computer is turned on and can actually eliminate the types of

problems that necessitate disc drive returns. It will help ensure

that disc drives remain where they belong -- at the customer site.

Seagate Technology, Inc., has tested and approved Disk Technician

Gold. In their endorsement, Seagate stated that:

"Disk Technician Gold sets the standard for hard drive

preventive maintenance software. Seagate recommends the use of

Disk Technician Gold to ensure error-free data storage

throughout the service life of your hard disc drive."

Disk Technician Gold's key component is a 14K memory-resident

device driver (not a TSR) that becomes part of the DOS operating

system and Windows 3.1, continuously and automatically providing:

20

Disk Technician Factory Test for Seagate Drives Manual

* Error detection, diagnostics and repair -- on the fly, in the

background, with no system performance degradation.

* Crash prevention and hard error data recovery and relocation

-- on the fly.

* Advance warning of electronic, mechanical and media failure

for all hard drive and computer components affecting data

reliability -- on the fly, in the background.

* Passive system, disc and data diagnostics.

* Test results that match those of Seagate factory hardware.

A separate, pull-down menu program provides interleave testing

and resetting, nondestructive low-level formatting,

defragmenting, manual drive testing, reports and interface

customization. An optionally installed virus prevention

device driver is also included.

Software Installation, Configuration and Setup:

Installation of Disk Technician Gold is both simple and

straightforward. End users either allow the program to make

installation decisions, or they make their own step-by-step

choices. Once files are installed, Disk Technician Gold

configures itself to the host system by measuring and testing the

specific hardware and software parameters, including hard disc

drive geometry. It then performs a system integrity test,

followed by media certification (see the (see the System

Integrity Test Hard Disc Media Certification sections in this

white paper for more information). Once these steps have been

completed, no further user interaction is required.

How Real-Time Error Detection Works Without Performance

Degradation:

Disk Technician Gold detects errors by testing all hard drive

accesses while working transparently in the background of other

programs, without interrupting normal activity, and without

system performance degradation. In fact, because it handles

block data transfers more efficiently than DOS, there is a net

gain of about 1% in average data transfer rate with Disk

Technician Gold installed. (The rate will be slightly slower on

very large data transfers; faster on small ones.)

When a soft error is detected, its location is recorded in the

databank (a special file created during installation).

The next time the computer is booted, Disk Technician Gold

automatically performs a Bootup System Check (which includes a

series of test routines designed to quickly check the major

components of the hard drive and its subsystems). The databank is

scanned for any new error locations recorded since the last

bootup. Any data in these locations is copied into RAM, then each

location is tested by writing, then reading back, worst case test

patterns and their inversions.

21

Disk Technician Factory Test for Seagate Drives Manual

Disk Technician Gold is the only software able to measure the

exact number of attempts the hard drive makes while accessing a

particular sector. If testing determines that the soft error is

normal (non-repeating), the data is returned to its original

location. This is because a non-repeating soft error is a

harmless, random and normal event.

However, an abnormal (repeating) soft error is dangerous and may

continue to recur at the same location unless corrective action

is taken. It is a strong indicator that the sector is in a

failure mode (i.e., the soft error rate is likely to escalate

rapidly to hard, or unreadable, status). If testing determines

that the number of soft errors is in excess of the threshold for

reliable data storage set by Seagate engineering, the data is

relocated to a new, safe location. The unsafe location is then

blocked from further use by marking it bad in the FAT, all

automatically without operator intervention.

After the data is safely rewritten, Disk Technician Gold records

the location and type of error in the databank, then uses the

information for two separate purposes:

1. Intermittent error detection and elimination. Disk

Technician Gold compares past test results with current

results to determine if a repeating soft error hit

(occurrence) is intermittent. This is when an area produced

a repeating soft error hit in the past, was repaired, and

has now produced another such hit.

If Disk Technician Gold's expert system rules that the hit

is recurring intermittently, any data in the area affected

is moved to a good area and the unsafe area is marked bad

in the FAT.

2. Hardware failure advance warning. The final step in the

Bootup System Check is an expert system analysis of the

databank to determine if the error is part of a developing

pattern indicating imminent hardware failure. The importance

of this step to data reliability cannot be overemphasized,

since the majority of perceived hard drive failures are not

actually caused by the hard drive itself. Patterns

recognized by the expert system must not only encompass the

hard disc drives electronics, mechanics and media, the

controller or adapter -- but also ribbon cables and

connectors, as well as non-hard drive computer components

that affect data reliability, such as the power supply.

Each of the many causes of hardware failure has its own, unique

signature. For example, as the disc platter spindle bearing

begins to fail, the platters begin to wobble microscopically. The

further away from the platter spindle bearing, the greater the

wobble. This may cause variations in the flying height of the

read/write heads with respect to the platters. This, in turn,

results in an intermittent high soft error rate, greatest on the

outer edges of the discs and gradually decreasing towards the

spindle.

22

Disk Technician Factory Test for Seagate Drives Manual

When such a failure pattern is recognized, Disk Technician Gold

displays a message directing users to immediately back-up their

data and consult a hardware service technician.

However, people responsible for maintaining data integrity and/or

hard drive systems on multiple computers (MIS management, PC

managers and coordinators, service maintenance departments, VARs,

consultants, etc.) were concerned that, despite the seriousness

of the warning message, it might still be ignored. For these

people, then, the warning message can be self-modified to direct

end users to a specific course of action.

Depending on the type of failure, the user typically receives

from a few days' to a few weeks' advance warning. As a diagnostic

aid to service technicians, the users guide contains a list

ranking the most likely reasons the warning message was

generated.

The entire Bootup System Check, including testing and repairs,

typically takes only a few seconds, and may be bypassed (as a

convenience for users who must reboot their systems often). Note

that testing, repair, relocation of data and updating of the File

Allocation Table (FAT) all occur as part of the bootup process.

The net result is that all hard drive and associated data

reliability issues are handled automatically and transparently as

operating system background tasks.

Real-Time Recovery From Critical And Hard Errors:

Most of Disk Technician Gold's real-time error detection and

preventive maintenance functions are transparent. The single

exception is when it detects a critical error (defined as one

that would result in data loss or a drive crash). There are three

types:

1. Excessive soft errors. These indicate imminent failure,

because on the next access attempt the soft error rate may

have escalated to such a degree that the area may be totally

unreadable (a hard error). For this same reason, any hard

disc location exhibiting excessive soft errors during

Seagate final quality control testing is marked in a "Bad

Track Report" or "Microdefect Map" for removal from use.

2. Excessive soft errors and Error Correction Code (ECC) was

invoked to reconstruct data. (ECC is defined as a

proprietary series of algorithms, specific to an individual

controller or hard drive, used to reconstruct data after it

has exhausted its preset number of attempts to read data

from a track or sector. This should not be confused with

on-the-fly ECC.) On the next disc access, the soft error

rate may have escalated such that ECC may not be able to

reconstruct the data.

Note that Disk Technician Gold does not use ECC alone as a

criterion for determining media reliability. Here are three

of many reasons:

23

Disk Technician Factory Test for Seagate Drives Manual

A. Individual controller and hard drive designs vary

widely in the number of retries they will make to get

data before invoking ECC. Many older XT-class

controllers allow 30 and more retries (soft errors)

before invoking ECC, an unacceptably dangerous

situation for data.

B. Conversely, technology now being introduced in small

form drives do not allow any retries whatsoever before

invoking ECC.

Because soft errors are both commonplace and typically

non-repeating in the normal PC hard drive environment,

ECC-dependent error detection could result in an

untimely and unwarranted condemnation of a perfectly

good drive.

C. Until recently, most drives and controllers reported

ECC information, which was ignored by DOS, but

available to diagnostic software. Today, however, most

block transfer drives (IDE and SCSI) and advanced

controllers do not return ECC status at all, such that

diagnostic software cannot detect any ECC activity

whatsoever.

3. Hard error (the area was completely unreadable). In this

case, ECC may or may not have been invoked. If it was

invoked, it failed to reconstruct the data. If not invoked,

it was because the data involved did not meet ECC criteria

for potential reconstruction.

Imagine someone working in an application such as Lotus 1-2-3 or

WordPerfect. The person has just opened a file that is in the

process of being read from the hard drive into RAM. Suddenly,

without warning, DOS generates an error message such as "Sector

not found," or "Disk error reading FAT," followed by "Abort,

Retry, Fail?". Until the introduction of Disk Technician Gold,

the only options at this point were rarely successful, after-the

-fact recovery and reconstruction attempts.

However, when Disk Technician Gold encounters a critical error,

it immediately begins a series of emergency actions:

1. It recovers the data from the affected area, using a

combination of seek-profiling, ECC and other proprietary

techniques.

2. The recovered data is recorded in the databank, along with

the error location.

3. Disk Technician Gold sounds an audio warning signal and

displays an on-screen message to alert users that a critical

error has been encountered. The message prompts users to

save their work, exit the program they are using, and reboot

their systems:

24

Disk Technician Factory Test for Seagate Drives Manual

Users working in some graphic environments are alerted to

critical errors by the audio signal only. A special decal

(called the Audio CueCard) that is usually affixed to the

keyboard or monitor frame, is included to provide the same

step-by-step instructions displayed on-screen:

4. When the system is rebooted, Disk Technician Gold tests the

area, repairs it if possible, or, if the area is unrepairable,

relocates the data to a new, safe location and blocks the

unsafe location from further use.

Once the problem has been resolved, Disk Technician Gold updates

the databank and FAT, and returns to the system prompt. The user

may then safely resume working in the interrupted program.

Real-Time Virus Prevention:

Disk Technician Gold includes an optionally installed, 7K memory-

resident Virus Detector to help protect systems and data. Like

the Error Detector, the Virus Detector is a device driver that

loads into RAM during bootup and works transparently in the

background of other programs.

The Virus Detector helps to protect hard drives against the most

common form of viral infection, where viruses attach themselves

to other files and begin mutating and reproducing.

Disk Technician Gold acts to stop viruses before they can attach

themselves to critical files by continuously monitoring write

activity to registered file types. Whenever an attempt is made to

write one of these files, Disk Technician Gold sounds an audio

warning signal and displays an on-screen message.

Of course, not all write attempts to protected files indicate the

presence of a virus. However, Disk Technician Gold does make

users aware of suspicious write attempts and provides the option

to abort them.

Menu-Driven Tests:

A key benefit of Disk Technician Gold is that, once installed, no

user interaction is needed to maintain the hard drive system in

factory-perfect condition. However, should users wish to perform

comprehensive drive tests, Disk Technician Gold includes several

on a separate, pull-down menu program:

System Integrity Test. This battery of tests is similar to, but

more comprehensive than, the Bootup System Test. Tests include:

* A self-examination of Disk Technician Gold for evidence of

virus contamination, and for the integrity of the critical

databank file.

* Checking the hard drives seek mechanisms for gross failure.

* Confirming the ability of DOS to create, open, write, read,

close and delete a file on the drive being tested.

25

Disk Technician Factory Test for Seagate Drives Manual

* Verifying the integrity of the boot record, directory, and

FATs.

* DOS CHKDSK-type routines to identify lost clusters,

crosslinked chains, allocation errors, and subdirectory

errors.

Like the Bootup System Test, the System Integrity Test also tests

and repairs, detects intermittent errors, and checks for signs of

hardware failure. The System Integrity Test normally takes about

one minute for most drives, depending on the size and speed of

the drive, the computer, and the nature and number of problems

found.

Hard Disc Media Certification. This is Disk Technician Gold's

most comprehensive test. Media Certification thoroughly tests

every area on the hard drive for soft, ECC and hard errors. It

either repairs those areas that produced errors; or, if an area

is unrepairable, it relocates any data in the area to a new, safe

location and marks the area bad in the FAT.

In addition to test and repair, Media Certification initializes

(or updates) the databank, recreates an updated media defect map,

and recertifies the drive for reliable data storage and normal

use.

Media Certification typically takes from 20 seconds to five

minutes per megabyte, depending upon the speed of the computer

and hard drive. An interrupt/resume feature allows users to leave

the operation and return to other applications. Then, when they

return to Media Certification, they will be asked if they wish to

resume from where they stopped.

Interleave Test/Low-Level Format. Interleaving matches the way

data is arranged on hard disc recording tracks to the speed at

which the controller can read the data sequentially. Disk

Technician Gold tests various interleave settings, then

determines and displays the optimal setting for maximum data

throughput speed. Users may then perform a nondestructive low-

level format and reset the interleave to the determined setting,

or choose their own.

For drives that allow low-level formatting and resetting of the

interleave, choosing the correct interleave can make a

significant difference in data access time. Drives that do not

allow low-level formatting and resetting of the interleave are

quickly identified as ineligible by Disk Technician Gold

(typically those using the SCSI interface, IDE drives, drives

using translating controllers, and even some MFM/RLL drives).

However, low-level formatting and resetting of the interleave on

"ineligible" drives are unnecessary, since these procedures only

need to be performed once in the lifetime of a drive and have

already been done by Seagate.

An Important Improvement In Failsafe Architecture:

26

Disk Technician Factory Test for Seagate Drives Manual

Some of the operations performed by Disk Technician Gold require

relocating data, and at certain times the data being relocated

only exists electronically in RAM. Should a power failure occur

at precisely this moment, some data could be lost. (It should be

noted that, since data relocation occurs rapidly, there is little

risk of losing data during Disk Technician Gold operations as a

result of power failure.)

However, for battery-powered computer users, or those who require

the ultimate assurance against data loss, Disk Technician Gold

provides an optional safe mode (also called failsafe

architecture) for media certification, testing and resetting

interleave, nondestructive low-level formatting, and

defragmenting.

The safe mode guards against data loss by requiring that a copy

of data undergoing relocation remains recorded on the hard drive

at all times, either in its original location or in a special

duplicate location. By eliminating instances when data exists

solely in RAM, the possibility of data loss due to power problems

is eliminated.

Disk Technician Gold's failsafe architecture offers a significant

improvement over previous methods. The problem was one of limited

effectiveness: data was protected only in the event that power

failed while the data file itself was being accessed. However, if

the FAT or a directory was being accessed when power failed, all

data on the affected drive could be lost. With Disk Technician

Gold, failsafe architecture works even if the FATs and directory

are being updated at the time of power failure.

Additional Features:

Media Defect Report. This report looks much like the Defect Map

affixed to ST506/412 drives, although it is more detailed because

of the additional types of errors detected by Disk Technician

Gold's expert system and continuous testing.

The Media Defect Report shows the location and number of bad

sectors on the drive, as well as the reason why they have been

marked out. (A bad sector is defined as one that cannot store

data reliably, as determined by Seagate engineering standards.

Such sectors are blocked from use by marking them bad in the

FAT.)

Monitored/Soft Error Report. This report shows the sectors on the

hard drive that produced a repeating soft error hit during

testing, but were successfully repaired. These sectors are

monitored during subsequent tests in order to determine whether

the hits were random or are part of a repeating error pattern

indicating an intermittent error or impending hardware failure.

If subsequent testing determines the sector to be unreliable, it

will automatically be blocked from use by marking it bad in the

FAT. Then, test information for this sector will be deleted from

the "Monitored/Soft Error Report" and added to the "Media Defect

Report."

27

Disk Technician Factory Test for Seagate Drives Manual

Command Line Operation. Those responsible for maintaining

multiple systems may wish to set-up a user-transparent hard drive

optimization program. The option of command line operations

provides the ability to bypass menus, set-up time-specific, self-

executing batch files, etc.

Additionally, users who normally leave their systems on

continuously can setup a command line to run Disk Technician

Gold's Bootup System Check occasionally, in order to test and

repair error locations detected by the device driver program.

Still other users may prefer to schedule automatic drive

defragmenting by setting-up a self-executing batch file.

System Requirements:

* IBM PC, XT, AT, -286, -386, -486, PS/1, PS/2 or 100 percent

compatible with 640K of memory.

* About 14K of RAM for the Error Detector device driver (high

memory loadable).

* MS-DOS 3.1 - 6.0; DR DOS 3.41, 5.0 and 6.0.

* Compatible with Windows 3.1. However, Windows is not a

requirement.

* Hard disc drive.

Hard Drive Specifications:

* Interfaces and Drives:  IDE, SCSI-1, SCSI-2, Fast SCSI-2, ESDI,

ST506/412.

* Controllers and Encoding Schemes:  MFM, RLL, ARLL, ERLL,

translating, ZBR (Zone Bit Recording).

* Partitions:  Supports any size DOS partition.

Optional Requirements:

* 7K of RAM for the Virus Prevention device driver (high memory

loadable).

* A Microsoft mouse or 100 percent compatible.

28

Disk Technician Factory Test for Seagate Drives Manual

Other:

* Works with most on-the-fly data compression software, including

DoubleSpace, Stacker, SuperStor, SuperStor Pro.

* Works with QEMM 386, 386 MAX and most other third-party memory

managers.

* Works with both hardware and software caching.

* Works with programs that keep a duplicate version of the FAT in

RAM (such as the DOS program FASTOPEN) or on the hard drive.

* Not copy protected.

* Free, unlimited technical support by telephone, fax and BBS.

Conclusion:

Disk Technician Gold software ensures PC hard disc drive data

reliability by providing continuous, real-time background

diagnostics, repair and recovery -- coupled with advance warning

of imminent failure for hard drive electronics, mechanics and

media, as well as other hard drive-related computer components

that affect data. Once installed, Disk Technician Gold becomes

part of the DOS operating system and requires no further user

involvement.

Disk Technician Gold is manufactured by Disk Technician

Corporation, of San Diego, California. It is sold worldwide

through its authorized distributors and is available at dealers

everywhere. Development of Disk Technician technology has been

continuous since 1980, paralleling the introduction of personal

computer hard drives. The first commercial version was shipped in

March, 1987. Since that time, newer versions have kept pace with

advancements in hard drive, controller, operating system and CPU

development, culminating with Disk Technician Gold, which

prevents data loss by making hard drives continuously self

-repairing.

In 1992, Seagate Technology Inc. determined that test results

from Disk Technician software precisely matched those of its

factory test hardware, and began distributing the Seagate version

of Disk Technician to its distributors for field testing and

recertification of its drives. For more information, call Steve

Luster at Disk Technician Corporation, (619) 274-5000.

Disk Technician Corporation

1940 Garnet Avenue

San Diego, California 92109 USA

Corporate Telephone 619 274 5000

Technical Support 619 272 4000

-END-

29