NetWare Driver Installation               JULY 1995

===========================

This diskette is labeled as LAN_DRV_SYS and the files are organized as follows:

Directory                    Contents

-----------------------  -------------------------------------------------------

\NETWARE\CLIENT\IPX      A pre-generated IN220P.COM

\NETWARE\CLIENT\V215C    NetWare v2.15c, v2.2 or NetWare 386 workstation drivers

\NETWARE\CLIENT\V215B    NetWare v2.15b, v2.2 or NetWare 386 workstation drivers

\NETWARE\CLIENT\ODI.DOS  ODI workstation driver for DOS (support NetWare v4.x)

\NETWARE\CLIENT\ODI.OS2  ODI workstation driver for OS2

\NETWARE\SERVER\NW311    NetWare 386 server or bridge drivers

( Support NetWare v.3.12 )

\NETWARE\SERVER\NW4X     NetWare 4.0 server driver

( Support NetWare v.4.01 )

\NETWARE\TURBO\NW311     Turbo LAN driver for IN-220P SERIES

( Support NetWare 3.11 or 3.12 )

\NETWARE\TURBO\ODI.DOS    Turbo LAN driver for IN-220P SERIES

( Support NetWare 3.11 or 3.12 )

*********************************

*  QUICK SETUP FOR WORKSTATION  *

*********************************

A pre-generated workstation driver IN220P.COM is included in this disk.

You can simply run the IN220P on a workstation to access the Novell network:

>IN220P        --Supports NetWare Lite, 2.1x, 2.2 or NetWare 386

>NETx         --This program is provided by Novell

***********************

*  WORKSTATION SETUP  *

***********************

If you want to generate your own workstation driver instead of using the

provided IN220P.COM, please do the following:

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_SYS.  You can use the DOS "Label" command as follows:

>Label  A:LAN_DRV_SYS

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy  A:\NETWARE\V215c\*.*  A:\   --For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides in the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-2 or WSGEN diskette.

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedures as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After installation completed, modify the STARTNET.BAT file as follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------------------

LSL

IN220P

IPXODI A

CLIENT

3) Copy \NETWARE\CLIENT\ODI.DOS\NET.CFG file to NWLITE sub-directory which

was created when you installed NetWare Lite.

4) You may run STARTNET to access the Novell network.

*****************************

*  SERVER AND BRIDGE SETUP  *

*****************************

NetWare 386 Server/Bridge Setup

=================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled SYSTEM-1.

>Copy  A:\NETWARE\SERVER\NW311\*.*  B:\  --SYSTEM-1 disk must be in drive B

2)  Execute Novell's SERVER program, which resides in the SYSTEM-1 diskette.

After loading the hard disk driver, you can load the network driver as

follows:

:Load  A:\IN220P.LAN        --Select your network card settings

:Bind ipx to IN220P

:load install              --Follow the NetWare installation instructions

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

IN-220P SERIES ODI Driver program allows you to run multiple protocol stacks.

It  also allows you to install up to two network adapters in a workstation.

IN-220P SERIES ODI driver supports "port, node address, protocol and frame"

options. You can use these options to create your own NET.CFG file.  For

detailed info on Keywords, please refer to Novell manual "NetWare ODI shell

for DOS - NET.CFG options".

1)  Copy the IN220P.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the IN220P.COM whenever a reference is made to the Hardware Specific

Module.

3)  A typical startup for ODI workstation is as follows:

>LSL               --Link Support Layer Module provided by Novell

>IN220P             --Hardware Specific Module

>IPXODI            --IPX Protocol Stack Module provided by Novell

>NETx              --DOS Support Module provided by Novell

>LOGIN

ODI Workstation for OS/2

========================

1)  You must first install the Requester on the workstation.  For detailed

information, refer to Novell manual "NetWare Requester for OS/2".

2)  Copy IN220P.SYS (from this diskette) to your C:\NETWARE directory.

Afterwards, add the following device driver statement to your CONFIG.SYS

file:

DEVICE=C:\NETWARE\CLIENT\ODI.OS2\IN220P.SYS

3)  IN-220P SERIES ODI driver supports "port, node address, protocol and frame"

options.  You can use these options to create your own NET.CFG file. For

detailed information on Keywords, please refer to Novell manual

"NetWare Requester for OS/2 - Reconfiguring the NetWare Requester".

*********************************************

* NetWare 4.0 SERVER and ODI CLIENT DRIVERS *

*********************************************

NetWare 4.0 Server Setup

==========================

1. Install the CDROM properly.

2. Prepare a INFOTEL LAN driver diskette that includes the following files

IN220P.LAN        (in IN-220P SERIES driver diskette)

IN220P.LDI        (in IN-220P SERIES driver diskette)

ETHERTSM.NLM     (in Netware 4.0 server)

MSM.NLM          (in Netware 4.0 server)

Insert INFOTEL driver diskette in A driver

Insert INFOTEL LAN driver diskette in B driver

1) Copy IN220P.LAN, IN220P.LDI from INFOTEL driver programs diskette

as follows,

copy A:\NETWARE\SERVER\MW4X\IN220P.LAN B:

copy A:\NETWARE\SERVER\NW4X\IN220P.LDI B:

2) Copy ETHERTSM.NLM, MSM.NLM from NETWARE 4.0 CD ROM or Diskette

copy L:\NETWARE.40\________\LANDRV\ETHERTSM.NLM B:

copy L:\NETWARE.40\________\LANDRV\MSM.NLM      B:

3. Install NetWare 4.0 SERVER

L: (CD ROM drive)

L:\NETWARE.40\ENGLISH\install

4. Loading the LAN driver

Choose a LAN driver that corresponds to the network interface board in

the server, repeat this step for any additional drivers.

1) Press <INS> to load a LAN driver not listed.

2) Press <F3> to specify a different path,

> B:\

( Put INFOTEL LAN driver into the diskette drive which contains

ETHERTSM.NLM and MSM.NLM )

3) Netware will display the list including the additional new driver.

Select the new LAN driver to install

IN220P.LAN

IN220P LAN Driver v2.10 (940815)

4) Set driver IN-220P SERIES parameters such as following :

Port value : 300H

Node address :

Frame type = Ethernet_802.2

Retries = 5

5) Bind the interface card to a protocol

assign a unique IP Network address to bind IPX to IN220P

(frame Ethernet_802.2)

6) Proceed and finish with installation

ODI Client for DOS

===================

IN-220P SERIES ODI Driver allows you to load multiple protocol stacks. It also

allows you to install up to two network adapters in a workstation. IN-220P

ODI driver supports "port, node address and frame" options. You can use these

options to customize the NET.CFG file.  For detailed info on Keywords, please

refer to Novell manual "NetWare ODI shell for DOS - NET.CFG options".

Please install the NetWare ODI client driver files following the procedures

as described in the NetWare installation guide.

If you want to generate your own NetWare ODI client driver, please do the

following:

1)  Make a backup of WSDRV_1 diskette.

2)  Copy the appropriate IN220P.COM, ETHDRVS.DOS and IN220P.INS driver

files to DOS directory of WSDRV_1 diskette (or backup):

Insert INFOTEL driver diskette in A driver.

Insert NetWare WSDOS_1 diskette in B: driver.

>Copy A:\NETWARE\CLINET\ODI.DOS\*.* B:\DOS --For NetWare client workstation

Append contents of ETHDRVS.DOS to DRIVERS.DOS

Contents of ETHDRVS.DOS is as follows:

" IN-220P SERIES Ethernet for ISA, IN220P.COM,IN220P.INS,IE0 "

3)  Execute Novell's INSTALL program, which resides in the WSDOS_1 diskette.

When you are asked to install the driver for your network board, press

the INS key.  You will be asked to insert a WSDRV_1 diskette.  Insert

this diskette (or backup) which contains IN220P.COM, DRIVERS.DOS and

IN220P.INS in drive A.

4)  Follow the NetWare Client Install normal installation procedures, Which

resides in the WSDOS_1 diskette. When asked to select a network driver,

simply choose " IN-220P SERIES Ethernet for ISA " for selecting parameter

like as Base I/O Port, Media Frame Type(s) and Optional Node Address.

Upon completion, a new NET.CFG file will be created in your DOS directory.

A typical of NET.CFG is as follows:

Link Driver IN220P

PORT 300

FRAME Ethernet_802.2

NetWare DOS Requester

FIRST NETWORK DRIVE = F

:

:

5)  A typical startnet.bat for ODI workstation is as follows:

SET NWLANGUAGE=ENGLISH  --Set NetWare 4.0 to English language

LSL                     --Link Support Layer Module provided by Novell

IN220P                   --Hardware Specific Module provided by IN-220P

IPXODI                  --IPX Protocol Stack Module provided by Novell

VLM                     --DOS Support Module provided by Novell

F:                      --Change to connected device

LOGIN .ADMIN.NV40SRV    --Login to file server NV40SRV as user ADMIN

6) You may run STARTNET to access the Novell network.

***************************************************************

* Turbo Driver for NetWare 3.11 SERVER and ODI CLIENT DRIVERS *

***************************************************************

Turbo Driver (IN220P.COM) follows the format of ODI driver,

so it is an ODI driver.

Turbo Driver for Server

=======================

The usage of Turbo Driver on the server is as following :

( In the NetWare 3.11 Server )

1. Added "set maximum physical receive packet size = 4202"

in STARTUP.NCF file

2. Load IN220P.LAN

3. Bind IPX to IN220P

Note:

The Turbo Driver IN220P.LAN only support NetWare 3.11 & 3.12

Turbo Driver for Workstation

============================

The usage of Turbo Driver on the workstations is as following :

LSL

IN220P.COM

IPXODI

NETX

F:

LOGIN John

Note 1:

Don't need to load IPX driver (IN220P) again.

It will not take additional memory than ODI driver. It can

replace the ODI driver.

Note 2:

You must load IN220P.LAN on the NetWare server and

run IN220P.COM on the workstations.

Then the effect of the Turbo Driver will work out.