STANDARD MICROSYSTEMS CORPORATION

DEC PATHWORKS v4.1 for DOS

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Use EZStart to copy the DEC PATHWORKS version 4.1 for DOS

NDIS driver files from the SuperDisk to your PC.

o  Prepare your operating system so you can attach to the network.

Note: This driver supports all 8216 (Ultra), 80xx (Elite),

80xx (PLUS), and 8416 (EtherEZ) Series ISA, EISA, and Micro

Channel Ethernet adapters.

COPYING THE DRIVER FILES

From the Driver Installation screen, select 'Copy Files' to copy all

the files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that

you specify.

PREPARING YOUR OPERATING SYSTEM

1. Insert the DEC Pathworks Client Setup diskette #1 into your

computer's disk drive.

2. Type: NETSETUP

3. Press the Enter key.

4. When prompted about whether you have an LK250 keyboard, specify

'N(o)' if you do not have this type of keyboard.

5. Select the DECNET option and press Enter.

6. Insert the DNET TRNSPRT 1 diskette into your computer's disk drive.

7. Specify the destination drive letter.

8. When the prompt 'Are you creating an initial workstation

diskette' appears, select N(o).

9. Type the node name and address.

10. Press Enter.

11. Type the server node name and number.

12. When prompted to add LK250 keyboard setup to AUTOEXEC.BAT file,

select N(o).

13. Select 'Ethernet' and press Enter.

14. Select 'LAN Manager-Supplied NDIS'.

15. Press the Enter key twice.

16. Select 'Western Digital EtherCard - WD8003E' option and press

Enter.

17. Press F9 to write the key disk. Then press Enter.

18. Press Enter to accept the default PROTOCOL.INI.

19. Follow the screen prompts.

20. After the Write Key Disk procedure is done, press F10 from the

Configuration screen to exit to DOS.

21. Locate PROTOCOL.INI file in the directory where EZStart

copied the driver (default is C:\DECNET). Then rename this file

to PROTOCOL.DEC. Then rename the DEC41DOS.INI (located in the

same subdirectory as the file you just renamed) to PROTOCOL.INI.

22. Using any text editor, edit the CONFIG.SYS file by replacing

the line:

device=\DECNET\MACWD.DOS

with this line:

device=\DECNET\SMC8000.DOS

23. Make sure the ni_irq value in the [DATALINK] section matches

the irq value of the network adapter specified in the

PROTOCOL.INI file.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.