{

This file is maded for midiprogrammers.

If any bugs/more information,

please report via

SBC-Connection

BulletinBoardSystems

MIDICTRL.DOC

by

Jari Kaija

(c)1991-93

Last edition date: 11,July,1993

-----------------------------------------------------------------------

If you made changes to this document, and those changes are

not correctly tested (non-standard midicommands)

=>

commands will not work properly with some machines.

SO,

!!!       DON'T EDIT THIS FILE       !!!

Commands tested with: Korg 707, Casio CZ-1000, Yamaha DX-7s

! Casio sends noteoff commands without NOTEOFF byte !

! Example:  Noteon = 144 60 64, Noteoff = 60 0      !

-----------------------------------------------------------------------

Channel commands    Statusbyte        Databyte 1       Databyte 2

(voice / few channel-command)

Note off            80 Hex + x        Notevalue        Keyoffpressure

Note on             90 Hex + x        Notevalue        Keyonpressure

Poly key pressure   A0 Hex + x        Notevalue        Pressure

Control chance      B0 Hex + x        Control number   New value

Local control       B0 Hex + x        7A Hex           On/Off

Program change      C0 Hex + x        Soundnumber

Channel pressure    D0 Hex + x        Pressure

Pitch bend wheel    E0 Hex + x        LSB              MSB

(play mode)

All notes off       B0 Hex + x        7B Hex           00 Hex

Omni on             B0 Hex + x        7C Hex           00 Hex

Omni off            B0 Hex + x        7D Hex           00 Hex

Mono on             B0 Hex + x        7E Hex           How many channels

Poly on             B0 Hex + x        7F Hex           00 Hex

x = Channel number 00-0F Hex (channels 1-16)

System commands     Statusbyte        Databyte 1       Databyte 2

(common)

Exlusive commands   F0 Hex            *Made by...*     Values...

Song pos. pointer   F2 Hex            LSB              MSB

Song select         F3 Hex            Song number

Tune request        F6 Hex

EOX                 F7 Hex

(real time)

Timing clock        F8 Hex

Start               FA Hex

Continue            FB Hex

Stop                FC Hex

Active sensing      FE Hex

System reset        FF Hex

-----------------------------------------------------------------------

*Made by...* (values by DECIMAL):

1    Sequential Circuits

2    Kurzweil

15   Ensonig

16   Oberheim

64   Kawai

65   Roland

66   Korg

67   Yamaha

68   Casio

69   Akai

-----------------------------------------------------------------------

Control chance values

Control number Hex          Name

00                     -

01                    Modulation wheel

02                    Breath

03                     -

04                    Foot pedal

05                    Portamento time

06                    Data-entry Hibyte

07                    Volume

08                    ? (Ctrl-release or) Balance ?

09                    Parameter number

0A                    Pan

0B                    Expression

0C - 0F                -

10 - 13               General-purpose controllers 1-4

14 - 1F                ??

20 - 3F               values for previous (??) commands

40                    Damper pedal

41                    Portamento switch

42                    Sustenuto

43                    Soft pedal

44                     -

45                    Hold 2

46 - 4F                ???

50 - 53               General purpose controllers 5-8

54 - 5B                -

5C                    Tremolo depth

5D                    Chorus depth

5E                    Detune depth

5F                    Phaser depth

60                    Sound increment, (value increment)

61                    Sound decrement, (value decrement)

62                    Not registered LSB

63                    Not registered MSB

64                    Registered LSB

65                    Registered MSB

66 - 7F               Reserved for channel mode commands

-----------------------------------------------------------------------

Midi clock timing for notes:

Note            Tick(s)

1/1     -       96

1/2     -       48

1/2.    -       32

1/4     -       24

1/4.    -       16

1/8     -       12

1/8.    -       8

1/16    -       6

1/16.   -       4

1/32    -       3

1/32.   -       2

1/64.   -       1

-----------------------------------------------------------------------

Song position:

Song position valuelimit = 0 - 16382 ticks

1 tick = 1/16 note

-----------------------------------------------------------------------

Few programs which uses MIDI...

Amiga:

Pro MIDI Studio (Mimetics)      USEFULL, BUT 'slow'

Deluxe Music    (Elec.Arts)                         *

Instant Music   (Electronic Arts)                   *

Sonix           (Aegis)                             *

Atari ST:

KCS             (Dr T's)                            *

4-OP Deluxe     (Dr T's)                            *

Creator         (C-Lab)                             *

Notator         (C-Lab)         IT'S GREAT !!!!

X-Alyser        (C-Lab)                             *

Smpte Trak      (Hybr.Arts)                         *

C-64:

KCS             (Dr T's)                            *

Instant Music   (Electronic Arts)                   *

Super Trak      (C-Lab)                             *

Score Trak      (C-Lab)                             *

DX-7 Support    (C-Lab)   Yamaha DX-7 editor        *

C-128:

KCS             (Dr T's)                            *

(and c-64 programs)

PC:

Cakewalk      (windows) SLOW!

Voyetra       (DOS)     Junior & Gold, versions available

Winjammer     (windows)

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Just waiting a couple of months (years?-):

BeatComposer MidiSequencer edition v2.0

SB1.0 - PRO compatible.

Running under DOS & OS/2 DOS

(MPU included later, when I get one...)

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

* = Information taked from 'C-Commodore (Finland) 2/88' magazine

-----------------------------------------------------------------------

RGDS

Jari Kaija

FINLAND, Helsinki

(1991 - 1993)

}

BEGIN

END.