----------------------------------------------------------------

-----------------------  Power Monitor  ------------------------

----------------------------------------------------------------

---  Version 1.2   ---

---   8 May 1995   ---

---                ---

---  Introduction  ---

---                ---

PwrMon is an HP-100LX system-manager-compliant application for

monitoring battery voltage levels. In addition to providing

current information on the state of the main and backup

batteries, it provides trend information by showing a history of

voltage readings.

This application *should* work with the HP-200LX as well as the

HP-100LX, but as it has not been tested on an HP-200LX, no

guarantee is made. (For that matter, no guarantee is made about

operating on the HP-100 either -- but it works on mine, and it

should work on yours.)

Please note the following points:

* Do not disregard the battery warnings issued by the HP-100

built-in software!

* Refer to Appendix A of the HP-100LX User's Guide for

information about battery usage with the HP-100LX.

* Don't wait for the backup battery voltage to drop low

before replacing it! The backup battery is only under

load when the main batteries are exhausted. If the backup

is old, it may exhaust rapidly under load.

---              ---

---  Background  ---

---              ---

Primary power for the HP-100LX is provided by two 1.5 volt AA

cells in series, thus providing a nominal 3.0 volts. "Backup"

power is provided by a single CR2032 lithium cell, itself

providing a nominal 3.0 volts. The backup battery provides power

to maintain memory during conditions when the main batteries are

not supplying power -- when they are being changed, or when they

reach a level lower than 1.8 volts.

Under normal conditions, power is drawn from the main batteries.

When the main batteries reach a level of 2.2 volts (alkaline) or

2.3 volts (NiCd), warning messages begin to appear indicating a

low battery condition.

If the main battery voltage level reaches 2.0 volts, the system

enters "software shutdown" mode. This mode is intended to

prevent a "warm-start" (i.e. reboot). In software shutdown mode,

whenever the system is turned on, a message is displayed, two

high-pitched beeps are sounded, and the system turns itself off.

System memory and RAM disk memory is maintained. When the AC

adapter is plugged in, or the batteries are changed, the system

returns to normal operation.

If the voltage drops to 1.8 volts, the system goes into backup

mode. The system is shut down, and the RAM disk is maintained by

power from the backup battery until main power is restored, or

until the backup battery is exhausted. When backup mode is

exited, a "warm-start" (reboot) always occurs. System memory is

reset, but RAM disk is maintained -- assuming that power was

restored before the backup battery was exhausted.

Whenever the AC adapter is supplying power, the batteries are

not used.

---            ---

---  Manifest  ---

---            ---

The files included in this archive are:

pwrmon.doc      This documentation file

pwrmon.exm      The pwrmon executable

pwrmon.icn      An Application Manager icon for pwrmon

---                     ---

---  Installing PwrMon  ---

---                     ---

---  Step 1: Load the files  ---

Copy the following files to your HP-100LX (you may want to put

them in a subdirectory):

file          description

------------  ------------------------------

pwrmon.exm    PwrMon executable

pwrmon.icn    App Manager icon for PwrMon

---  Step 2: Define PwrMon to Application Manager  ---

Open Application Manager by pressing the [&...] key. Press F2

(Add), and enter the following values:

Name:     Power Monitor

Path:     (drive):\(subdirectory)\pwrmon.exm

Comments:

In the Path field, (drive) and (subdirectory) should be replaced

with the drive letter (a or c) and the subdirectory to which you

copied the PwrMon files.

Now tab to the Icon field, and the PwrMon icon should appear.

If it does not, press the down-arrow key until you see the

PwrMon icon.

Press F10 (OK).

Now press the key-combination you want to use as the hot-key for

PwrMon (I use Alt+Filer; if you want to use something else, the

dialog which is dispayed at this point shows you what your

options are).

---  Step 3: Start PwrMon  ---

Back at the Application Manager screen, highlight the PwrMon

icon and press Enter. To end PwrMon, press MENU, Q.

---                ---

---  Using PwrMon  ---

---                ---

PwrMon samples battery voltage when it is started, and when the

"Sample" F-key (F3) or the space-bar is pressed. PwrMon can also

be configured to automatically sample battery voltage at one of

three fixed intervals. To do this, Press MENU, I(nterval), and

select an interval. When automatic sampling is active, it will

continue until stopped by pressing the "Cancel" F-key (F6) or by

selecting "Disable Sampling" from the "Interval" menu. Automatic

sampling will continue even if you switch to another applica-

tion or turn off the HP-100. Automatic sampling will be stopped

when PwrMon is terminated.

The controls for PwrMon are as follows:

MENU/File

/New        -- Resets to zero the voltage sampling history.

/Exit       -- Closes the PwrMon application and terminates

automatic voltage sampling.

MENU/Interval

/Disable Sampling -- Disables automatic voltage sampling.

/15 Minutes -- Enables automatic voltage sampling with an

interval of 15 minutes.

/30 Minutes -- Enables automatic voltage sampling with an

interval of 30 minutes.

/60 Minutes -- Enables automatic voltage sampling with an

interval of 1 hour.

/2 Hours    -- Enables automatic voltage sampling with an

interval of 2 hours.

/4 Hours    -- Enables automatic voltage sampling with an

interval of 4 hours.

MENU/Quit     -- Closes the PwrMon application and terminates

automatic voltage sampling.

MENU/Help

/About PwrMon...  -- Shows the version and copyright notice.

F1 (Help)     -- Describes the PwrMon display.

F3 (Sample)   -- Samples the battery voltage. If automatic

sampling is enabled, the interval is restarted.

F5 (Auto)     -- Enables automatic voltage sampling with an

interval of 1 hour. This Fkey is only active

when automatic sampling is disabled.

F6 (Cancel)   -- Disables automatic voltage sampling. This FKey

is only active when automatic sampling is

enabled.

F8 (Sleep)    -- Causes PwrMon to be sent into the background

(i.e. it is still open, but it will not

display unless you explicitly switch to it.

See "Notes".)

F9 (NiCd)     -- Sets the battery type to NiCd, and enables

battery charging.

F10 (Alkalin) -- Sets the battery type to alkaline and disables

battery charging.

---               ---

---  The Display  ---

---               ---

Power source     This can be "battery" or "AC". If the AC

adapter is plugged in and providing power to

the HP-100, then "AC" is shown; otherwise,

"battery" is shown.

Charging status  This may be one of the following:

Charging enabled

Charging disabled

Fast charging hh:mm

Slow charging in progress

For fast charging, the elapsed time in fast

charge mode is shown, in hours and minutes.

This time is only updated when a sample is

taken.

Main batteries   The type of batteries selected (by the user) is

shown. The current voltage is indicated on the

bar, and the warning (V-sub-w) and backup

(V-sub-b) thresholds are shown (the warning

threshold is different for NiCd v. Alkaline

batteries). The bounds of the bar indicate the

measurement range, NOT the normal operating

range. The histogram shows the level of the

last 64 voltage samples.

The threshold voltage values are:

Alkaline   NiCd

--------   ----

Warning mode   2.2v     2.3v

Software shutdown mode   2.0v     2.0v

Backup mode   1.8v     1.8v

Note that the voltage under load will differ

substantially from voltage when not. This can

be observed by plugging in the AC adapter. Main

battery voltage will suddenly rise, even if the

batteries are not being charged.

The histogram data can be zeroed by pressing

MENU, F(ile), N(ew).

Backup battery   The same information is provided for the backup

battery, except for threshold voltages, which

do not apply.

---         ---

---  Notes  ---

---         ---

When PwrMon is set for automatic sampling, if it is not the

foreground (i.e. visible) application when the time comes to

take a sample, then PwrMon will momentarily come to the

foreground, take its sample, and disappear. This produces two

effects which may be considered unsettling, until you become

used to them. The first is the obvious and disconcerting "popup"

over the active application. The second is more subtle: once

PwrMon has taken a sample from the "background," it apparently

"disappears", that is, if you close all other open applications,

PwrMon does *not* appear. It seems to have closed itself. But in

fact it has not. If you open the Application Manager and switch

to list mode, you will find that the status of PwrMon is listed

as "Asleep." What has happened is that in order to revert back

to the active application after momentarily coming to the

foreground to take a sample, it issues a system call which

causes PwrMon to be sent to the bottom of the application

queue - below even the Topcard application. So, PwrMon is

still open, but you will not see it again until you explicitly

switch to it (or until it takes another sample).

The current setting and the last 64 voltage samples are saved

in a file named pwrmon.env, in the subdirectory c:\_dat,

whenever PwrMon is closed (by MENU, Q(uit) or MENU, F(ile),

e(X)it); and whenever a sample is taken.

---                   ---

---  Version History  ---

---                   ---

Version 1.2 - dated 8 May 1995

* Added "Sleep" Fkey to explicitly send PwrMon to the

background.

* Changed link options, significantly reducing memory

requirements.

Version 1.1 - dated 6 March 1995

* Added capability to automatically sample voltage at fixed

intervals even when PwrMon is not the foreground

application.

* Only "Auto" or "Cancel" Fkey is labelled, depending on

whether or not auto sampling is active.

* Changed interval set by "Auto" Fkey to 1 hour.

* Added 2-hour and 4-hour sampling intervals.

Version 1.0 - dated 25 February 1995

The initial release of PwrMon.

Eugene Dorr

Internet: dorre@well.com

CompuServe: 75156.3132