BRIEF DOCUMENTATION FOR

SETVOL.COM & VOL_IS.COM

by

Ed Keefe

(c)July 22, 1993

VOL_IS.COM

----------

PURPOSE : Determine the current volume (sound level) on the

HP 100LX.

RETURNS : The program, when run, returns an errorlevel in the

range 0 to 3, where 0 is no sound and 3 is the loudest

sound currently attainable on the HP 100LX

USAGE   : To see a number representing the current sound level,

use the following batch file.

@ECHO OFF

REM WHATVOL.BAT

VOL_IS

FOR %%I IN (0 1 2 3) DO IF ERRORLEVEL %%I SET VOL=%%I

ECHO %VOL%

The WHATVOL.BAT program runs VOL_IS and uses the errorlevel to

set an environment variable (VOL) to one of the values 0 to 3. The

ECHO command displays this value. (You could also use SET to  see

what else is in your environment space.)

SETVOL.COM

----------

PURPOSE : Set the volume level, from DOS.

USAGE   : SETVOL n

where n is a number in the range 0 to 3 inclusive.

If SETVOL is run, without any commandline value, the

volume defaults to 1 (the soft sound).

If n is greater than 3, the BIOS function used, in

SETVOL.COM, forces the value to 3, the loudest sound.

LIMITATIONS: This command changes the volume level. It does not

set the volume histogram in the SETUP program to the

corresponding value.

EXAMPLE :

@ECHO OFF

REM TESTVOL.BAT

FOR %%I IN (3 0 2 0 1) DO CALL RING %%I

@ECHO OFF

REM RING.BAT

SETVOL %1

ECHO ^G^G^G^G^G^G^G^G

REM ^G is the bell sound

REM entered by pressing CTRL+G

DISCLAIMER

These two small programs are copyrighted by Ed Keefe, and, hereby

offered, as is, as freeware. No claims are made about their

usefulness. Use them at your own risk. They appear to function

properly but I will accept any bug reports graciously and try to

fix them.

ADDENDA

DEBUG SCRIPT FILES USED:

N VOL_IS.COM

A 100

MOV AL,1E         ;set offset

OUT 22,AL         ;point to offset

IN  AL,23         ;read in the value from the port

MOV CL,6          ;put 6 in CL

SHR AL,CL         ;shift AL 6 bits to the right

AND AL,3          ;mask AL with 0011b

MOV AH,4C         ;quit

INT 21            ;return errorlevel in AL

RCX

10

W

Q

N SETVOL.COM

A 100

MOV BX,80          ;point at the commandline

MOV CX,0031        ;stuff 31h in CL as default value (1 dec)

INC BX             ;move into the commandline

MOV DL, BYTE [BX]  ;put the byte in DL

MOV CH,CL          ;put CL in CH

MOV CL,DL          ;put DL in CL

CMP DL,D           ;is DL the ENTER keystroke?

JNE 0106           ;NO: goto INC BX

MOV AH,48          ;else, put 48h in AH

MOV AL,CH          ;put CH in AL

SUB AL,30          ;subtract 30h from AL

INT 15             ;call the BIOS

MOV AH,4C          ;

INT 21             ;quit

R CX

1E

W

Q