******************************************************************************

===== Brief HP 95LX Overview =====

The HP 95LX is very PC compatible except in areas which have been customized

to obtain its small size or to support large amounts of memory, particularly

in the following areas:

DISPLAY -- The physical display is 40 characters by 16 lines i text mode.  This

is a window into an MDA standard 80 character by 25 line display RAM. There is

provision for windowing the physical display around the larger display.  Also

included is a non-standard graphics mode which has pixel dimensions of 240

columns by 128 rows.

CHARACTER FONT -- The font is stored in ROM.  Codepage 850 was used since it

contains necessary international characters needed for product localization.

The standard PC codepage is 437 which contains some additional line drawing

characters and some other differences in the upper 128 characters.

KEYBOARD -- There are some differences, additions, and deletions from a stand-

ard PC keyboard.

MEMORY MAPPING -- The HP 95LX contains 512K of RAM and 1M of ROM, as well as

up to 2M of plug-in card memory space.  Much of the ROM and plug-in card

memory space is accessed by mapping it into banks C000, D000, and E000 of the

CPU address space, using non-PC standard mapping calls.

SYSTEM MANAGER -- The SysMgr is a COMMAND.COM replacement, which implements the

task-swapping used in the HP 95LX, as well as supplying a large number of

function calls that SysMgr compliant applications must use in order to avoid

resource conflicts and/or to reduce the size of the application.

DOS -- The HP 95LX uses DOS 3.22, which is very similar to DOS 3.20, but which

executes directly out of ROM, thus freeing valuable RAM.

******************************************************************************

===== Development Tools ======

The HP 95LX SysMgr requires that executable programs follow a particular

structure.  All HP and Lotus development was done using Microsoft C and/or

Microsoft MASM.  Other languages may work, but you'll be blazing your own

trail.  Once an .EXE file has been generated using these tools (and following

certain guidelines), the E2M.EXE tool is used to transform the .EXE file into

a .EXM file, which is what the SysMgr requires.

TSRGRAPH is a PC TSR that can be used for early phase development to run

graphics based applications on a PC.  It provides HP 95LX compatible graphics

mode and all the low-level graphics routines that are supported in the HP 95LX.

Successive executions of TSRGRAPH alternate between installing and de-install-

ing the TSR.

******************************************************************************

===== Structure of an .EXM file =====

The special C-language program startup situations for SysMgr compliant applic-

ations are:

1) applications must be linked with a special version of the C-language

startup code, crt0.obj.  This version is needed because the SysMgr

launch mechanism has already performed the tasks performed by the

normal C-language startup code.

2) command line and environment variables (argc, argv, and envp) are

not available.

Add-in SysMgr compliant applications must be small memory model programs, less

than 64K of code and less than 64K of preallocated data (including the stack).

Additional data space, up to available memory, can be dynamically allocated

using the SysMgr m_alloc or m_alloc_large functions.

DATA PTR CONSIDERATIONS -- The SysMgr may move an application's data space

under certain circumstances and in particular when an application is inactive.

If only DS relative NEAR pointers are used, then this is not a problem since

the SysMgr will sete up DS properly prior to activating the application.  If

FAR data pointers are used, then the application needs to fix these pointers

each time the SysMgr moves its data segment.

COMPILING/LINKING CONVENTIONS -- When compiling and linking SysMgr apps:

1) Applications must be compiled with the /Gs option to eliminate

stack checking.

2) There must be at least 256 bytes of application stack space avail-

able for SysMgr use whenever a SysMgr service is called.

3) Applications must link with the SysMgr services interface module,

CSVC.OBJ.  This module contains the c_service function that performs

the software interrupt to transfer control to the SysMgr services

jump table.

If the SysMgr provides a given service, that service should be used instead of

using a C-library function.  This ensures compatability and reduces the size of

the application's code.  Some specific points to note are:

1) Keyboard input MUST be obtained only by the use of the SysMgr

m_event or m_nevent functions, or task swapping will NOT work.

2) Dynamic memory allocations MUST be done only by the use of the

SysMgr m_alloc or m_alloc_large functions, or by using DOS services

directly (preferably the SysMgr functions).  Standard C library memory

management functions should NOT be used.

3) Elementary C library rouintes such as strlen and atoi can be used.

4) SysMgr file I/O uses a FILE structure that is not compatible with

the FILE structure defined in Microsoft C STDIO.H.  Use care with

including STDIO.H and, in particular, don't include both FILEIO.H and

STDIO.H in the same module.

5) The standard C library startup code is NOT used with SysMgr compli-

ant apps.  This may affect the use of some types of C library functions.

Avoid initializing data if at all possible.  Initialized data takes up space

in the .EXE/.EXM file, whereas un-initialized data takes up no space in the

.EXE/.EXM file and is allocated/zeroed automatically by the SysMgr at startup

time.

APPLICATION INITIALIZATION and TERMINATION --- The SysMgr implements non-pre-

emptive task swapping for SysMgr compliant applications.  Hence, compliant

applications are event driven and must make timely calls to the m_event or

m_nevent function to receive keyboard input and other events.

The SysMgr reports key strokes for nomal keys, but reports a deactivate event

when another application's hot key is pressed.  In response to a deactivate

event, the application is expected to do any necessary housekeeping to prepare

for suspension and then request the next event.  The next event, which will be

an activate event, will not be returned until this application is activated

again.  Activation can occur for several reasons; for example, the applications

hot key is pressed, all subsequently activated applications are quit, or the

application has been selected for termination from the low memory close out

menu.

In response to an activate event, the application must redraw its screen.  For

RAM efficiency, it is recommended that applications have a means of redisplay-

ing the screen from primary data, rather than by saving a copy of display

memory.

The SysMgr may also return a termination event at any time.  The application

is required to respond to this as if the user issued a quit command, with

any user-interface variations needed to make clear what is happening.  For

example, if an editor is being terminated and its buffer has been modified,

the user should be prompted to save the file.

If possible, when an application terminates, it should save its state inform-

ation in a file.  When it is subsequently relaunched, it can inspect the file

and reconstruct its state prior to termination.

INTERRUPTIBLE PROCESSES --- An "interruptible process" within a program is one

which the user can interrupt by pressing a key.  An example of an interruptible

process is function plotting in the built-in calculator ap.

During an interruptible process, the program must periodically check the key-

board to see if the user ahs requested interruption.  The HP 95LX BIOS uses

keyboard checks as an occassion to go into light sleep.  This is done as a

power saving feature and is only done when running on batteries.  In many cases

keyboard checks are only done when a program is otherwise idle.  In these cases

going into light sleep does not affect performance and is an effective way to

conserve batteries.  However, during an interruptible process, going to light

sleep can substantially slow down the process.

The BIOS provides a service (INT 15h, function 4Eh) which controls whether

light sleep will be entered during a key test.  This function should be used to

disable going to light sleep during an interruptible process and then to re-

enable light sleep for power conservation reasons.  The sample application

BOXES.C contains an example of an interruptible process.

ACCESSING SYSMGR SERVICES FROM C -- Each source file containing services calls

must include the header file interfac.h which contains macro defintions for

each SysMgr service.  The macro expands the call to be a call to a common

System Service request function.  In additon, the macro adds a service number

to the argument list and casts near pointers to far pointers as appropriate.

For example, the m_disp call in the source:

#include	"interfac.h"

int	row, col, style, ostyle;

char	*str;

.

.

.

m_disp(x, y, str, strlen(str), style, ostyle);

expands to:

c_service(F_M_DISP, x, y, (void far *)str, strlen(str), style, ostyle);

where F_M_DISP is the function code for m_disp and is defined in interfac.h.

The function c_service is provided in the object module csvc.obj and converts

the call into a software interrupt that transfers control to the SysMgr

dispatch table.

ACCESSING SYSMGR SERVICES FROM ASM --- Assembler programs access the services by

pushing any required arguments on the stack and then using the SMCALL macro to

"call" the service.  For example, usage of m_disp might appear as:

include	interfac.mac

...

push	dx	; ostyle (not actually used)

push	dx	; style

push	cx	; string length

push	ds	; string segment

push	si	; string offset

push	ax	; column

push	bx	; row

SMCALL F_M_DISP	; display the string

add	sp,14	; remove arguments from the stack

There are three things to note in this example:

1) The arguments are pushed in left to right order as is done by C.

2) Pointers are passed as far pointers.  See interfac.h for argument

specifics where, in general, pointers are cast as far pointers.

3) The function numbers such as F_M_DISP and the SMCALL macro can be

found in interfac.mac.

SYSTEM MANAGER AND TASK MANAGEMENT -- First, consider the case where an applic-

ation is being selected and no applications are currently active (ie, the

owner-information, or business card, screen is showing).

When the SysMgr starts an application, it loads the application into memory,

sets up the segment registers, and transfers control to application's entry

point.  For built-in appliactions (that run from ROM), the load step involves

allocating enough system RAM for the application's data and copying the intial-

ized data from ROM to RAM.   For loading external applications, the SysMgr must

also allocate system RAM for the application's code.  In this case, both the

code and intialized data are read from disk into memory.

Once started, an application enters an event loop where it calls the SysMgr

m_event or m_nevent function to get the next key stroke or other event.  Once

started, an application will be called open until it calls the SysMgr m_fini

function, which is normally when the user quits the application.  An open ap-

plication will be called active if it has control and inactive if another ap-

plication has control.

Now, consider the case where an application is being selected but a different

application is currently active.  In this case, the SysMgr not only needs to

load and launch the new application, but also needs to deactivate the previous

application.

When switching to a new application, the SysMgr sends a deactivate event to the

current appliaction, which changes its status from active to inactive, and then

starts the new one.  If there is insufficient memory to load the new applicat-

ion, the SysMgr displays a low memory close down screen and gives the user the

chance to terminate one of the open applications.  Once there is sufficient

memory, the new application is loaded and launched, while the data for the

previously open applications remain in memory.

Special handling of the the code space is required for external applications.

Only one external application's code is kept in memory at any one time, so if

the new application is an external application, and another external application

is opened, this code space (after possible expansion) is reused for the code of

the new external ap.

NOTE: Whenever there is an external application open, the external application

code space is NOT reduced in size because there must always be enough code

space to restart any of the open (but inactive) external applications.

MEMORY MANAGEMENT --- The SysMgr is repsonsible for efficiently managing

memory for SysMgr compliant applications.

For example, to prevent memory fragmentation, an inactive application's data

space may be moved in memory as other applications are launched and exited.

This means that applications should not save the DS value in memory unless the

application is prepared to modify the DS value in case its data segment has been

moved.

As another example, the code space is overlaid for all external applications.

This means that the code for an inactive external application will not be in

memory if any other external application has been subsequently activated.  This

implies that variable data should not be stored in the code segment.

TSRs, INTERRUPT VECTORS, and THE SYSTEM MANAGER --- There are two ways that DOS

can be accessed on the HP 95LX.  One way is to change the DOS shell to

COMMAND.COM by creating a CONFIG.SYS file with the line SHELL=COMMAND /P.  The

other way is to run a DOS command from the Filer or from the DOS command line

accessed via the Filer's System function.  Thus, there are two environments in

which to run TSRs.

Running TSRs from the Filer has two drawbacks.  One is memory fragmentation

due to the hole left by COMMAND's data when the TSR terminates.  This memory

may or may not be usable by the SysMgr.  A more important drawback is that the

SysMgr may not be able to run an external application after a TSR has been

installed from the Filer.  This is because the SysMgr needs to expand a memory

block in order to load the external application's code and the TSR may block

this expansion.

Consequently, we generally recommend that TSRs be run BEFORE starting the

SysMgr.  The TSRs can be started from an AUTOEXEC.BAT file which, if terminated

with the $SYSMGR command, will start the SysMgr after the TSR loads.  This

technique permanently ties up memory for COMMAND's data;  but, since this is

below the SysMgr, it does not cause memory fragmentation.

TSRs usually make use of some interrupt vectors.  The SysMgr takes over some

interrupt vectors without "chaining" onto the previous owner.  This means that

a TSR that is loaded first may not get control when it expects to if the SysMgr

has taken the TSR's interrupt.  Thus, it is necessary to know which interrupts

are used by the SysMgr.  The interrupt taken by the SysMgr are:

INT 05h	Print Screen

INT 06h	HP 95LX specific BIOS service used to signal entering and

leaving sleep modes.

INT 0Ch	COM1 serial port interrupts

INT 1Bh	CTRL-Break interrupt.  The SysMgr saves the orignal value and

restores it before accessing DOS from the Filer.

INT 4Ah	User alarm.  Called by BIOS when real-time clock alarm goes off.

INT 60h	Used for calls to SysMgr services.

INT 61h	Used by the SysMgr to load its DS register.

In addition, the SysMgr chains into the INT 1Ch, the user timer tick interrupt.

Finally, INT 62h is used by the HP 95LX's Calculator application.

HEADER FILES --- Header files are used for macro and structure definitions.

All functional areas require the inclusion of interfac.h for C modules or

interfac.mac for assembler modules.

Some functional areas also require the inclusion of additional header files:

Service class	C Program	Assembler Program

-------------	------------	-----------------

Event		event.h		event.mac

Menu		menu.h		menu.mac

File Menu	fmenu.h		fmenu.mac

Editing		edit.h		smedit.mac

Clipboard	cbcodes.h	cbcodes.mac

File I/O	fileio.h	filio.mac

Date/Time	smtime.h	smtime.mac

Configuration	settings.h	settings.mac

Communication	comio.h		comio.mac

Bridge		bridge.h	bridge.mac

Misc		m_error.h	m_error.mac

A NOTE ON C-LANGUAGE FUNCTION PROTOTYPES --- The header files do not contain

function prototypes for the services.  Instead, interfac.h contains macro

definitions that expand SysMgr service calls into calls to a single service

dispatch routine.

Hence, if a service is accidentally called with the incorrect number of

parameters, it will not match the macro and the compiler will issue a warning

about too few or not enough actual parameters for the macro.

However, if a service is accidentally called with an incorrect argument type

but with the correct number of parameters, this may not be caught by the comp-

iler due to the lack of a function prototype.

The following functions in interfac.h are NOT USABLE BY APPLICATIONS:

com_timer_addr		com_timer_count_addr

InitCom			m_appcount

m_app_name		m_common_open

m_day_trigger		m_disable_macros

m_enable_macros		m_fall_printer

m_get_settings		m_ram_iv_info

m_reboot		m_set_daterule

m_set_settings		m_spawnarg

m_sys_rsrc_addr

All pointers in SysMgr service calls are FAR pointers.  However, the service

macros contain casts of pointer arguments to far pointers.  Consequently, near

pointers can be used for data in cases where compiler can supply the segment

value due to the cast.