Commchk.doc	Version 1.8

Documentation for the Serial protocol analizer

Commchk is used to analyze information on a serial line by

intercepting the information and allowing you to view the

captured information in either a HEX format or in an ASCII format.

The information captured may also be saved in files stored on disk.

Commchk performs this function by routing any information read from

com1 to com2 and any information read from com2 to com1, while

capturing the information for display and or saving to a file.

Commchk will split the screen into 2 windows, an upper window

and a lower window. the upper window will display the information

captured from com1 and sent to com2. The lower window will

display the information captured from com2 and sent to com1.

This program was developed to allow analyzing information being

send from a host computer to a slave computer to determine the

source of errors that were getting into the system.

Alternatively, Commchk can be used to allow a computer that does

not have a modem to access the modem of a second computer.

Commchk command line options are summarized as follows.

-1, -2, -3, -4	Setup ports to use. The first entry becomes

the port designated com1 at the top of the

screen. The second entry becomes the bottom

half of the screen.

-I 0/1/2	all commands up to the next "I" command

are for the identified port.

0 = both ports

1 = com1

2 = com2

-B baud		baud rates from 300 baud to 57600

baud are supported.

-C		Turn on TSC Computer CRC checking

This will then generate an additional file

with a "C.FIL" appended when disk capture

is turned on.

-F file name	the file name must not have a length of

greater than 7 characters, not including

the path name. It is used to create create

2 files, one for information read from com1,

and one for information read from com2 by

appending the communications port number

and .fil to the end of the file name.

IE: -f test

generates test1.fil - com1 info

test2.fil - com2 info

-f c:\commchk\test

generates c:\commchk\test1.fil

c:\commchk\test2.fil

-H Help off	Turn off the initial help screen

-M		Turn Modem/Line status checking on/off

-N 		Set the port name to display on the upper

line of the port display screen

-P parity	Allowable paritys are

EVEN, ODD, NONE

-W word length	7 or 8 bits

-R Resident	As soon as the program gets the serial ports

initialized it will imiadiately shell out to

dos. Information will continue to pass thru

the serial ports from one to the other.

-S stop bits	1 or 2 stop bits

Example startup command:

COMMCHK -I 1 -B 1200 -P E -W 7 -S 2 -I 2 -B 4800 -P N -W 8

THIS SETS COM1 TO 1200 BAUD, 7 BIT, EVEN PARITY, 2 STOP BITS

AND COM2 TO 4800 BAUD, NO PARITY, 8 BIT.

Commchk uses an initalization file named "COMMCHK.INI" to initialize

the serial port parameters.

If no initialization file is found then the ports will default

to the default PC serial port values. Values for all ports do

not need to be supplied.

Only values for the ports you want to change from the default.

You may also place command line parameters in the commchk.ini file.

The first character on a line containing command line parameters

"MUST" be a "-" character or the line is interpeted as a serial port

setup line as described in Section "A" below.

Section A:

The format of the ini file is as follows:

port,base address,interrupt vector		<== serial port setup

port,base address,interrupt vector

-M -B 4800								<== command line parameters.

-P

The allowable ports are 1 - 6.

The commchk display is as follows:

PORT1 BUFFER     0.0   SYSTEM BUFFER      0.0   RTS DTR DSR CTS      0     0

^ SIZE OF THE BUFFER FOR PORT 1

^ SIZE OF THE BUFFER USED FOR DISPLAY

TOTAL NUMBER OF SERIAL PORT ERRORS  ^

LAST RECIEVED SERIAL PORT ERROR CODE      ^

Commchk keyboard commands are summarized as follows.

F1 key - Help

F4 key - Allow changing the serial port characteristics online.

O  key - Open the currently active capture files

C  key - Close the currently active capture files

D  key - Toggle video display on and off

F  key - Flush out all current serial port buffers

of all characters.

A  key - place the display in ASCII mode

H  key - place the display in HEX mode

I  key - Reinitialize the port, also clears all buffers

Q  key - Quit

R  key - Toggle the RTS line

S  key - Shell out to dos - interrups and information flow from

one port to the other continues to occur

T  key - Toggle the DTR line

L  key - Toggle the line display status line

M  key - Toggle the Line and Modem status interrupt on/off

if you are on a system where the unused lines are allowed to

float then they may cause numerous interrupts to occur and

appear to lock your system. so turning these interrupts off

may fix this problem.

Commchk Startup defaults are as follows

2400 Baud, 8 Data bits, 1 Stop bit, No parity

No capture file, com1 and com2 ports

Revision History:

11/5/90  - 	Fixed serial protocol data errors.

11/13/90 - 	Modified help routine to not interrupt serial data flow.

Added Flush buffer command.

Modified asynchronous character send routine

errors that caused loss of characters.

Added code to allow setting of the serial

ports to different parameters.

Removed SHAREWARE message.

VERSION 1.3

11/27/90 - 	Modified code to use only 8088/8086 code.

Added a line at the bottom of the status windows

that displays statistics about the serial port

Added code to allow toggling the RTS and DTR lines

Modified the asyncronous lines to have all serial port

IO completely asyncronously driven to assist in

performance on slower computers.

Added code to allow changing the serial port

characteristics while the program is running (F4 key).

VERSION 1.4/1.5

04/26/92 - 	Added the -H Startup command to turn off the initial

help screen.

Added the -R Command to perform an imiadiate shell to

Dos.

VERSION 1.6

11/06/92	Added Port names to display for identifying the port

Added CRC Checking for testing

Added "C" command to turn CRC testing on

VERSION 1.7

01/29/93 - Modified to allow any baud rate from 50 to 115200 baud to

be used.

- Added comm ports 5 & 6.

- Added "COMMCHK.INI" initialization file.

- Added ability to use 5 and 6 bit word length.

VERSION 1.8

02/07/93 - Fixed command line parsing for 5 and 6 bit word length

- Added ability to turn off Line/Modem status interrupts

- Added ability to place command line commands in the initialization

file "COMMCHK.INI"

Written using Turbo C++, Borland C++ by:

Timothy L. Garrison

3003 W. Marquette Woods Rd.

Stevensville Mi. 49127