NOBEEP

Version 1.60                      3/30/89

A PCjr BIOS replacement that fixes many of

the problems caused by use of NMI for the

keyboard on the PCjr.

Written by:     Gary Strait

Features:

- Eliminate keyboard beep in most cases (including disk busy) (for disk reads

only when used with Impulse / Alpha / Micro Products memory sidecars).

- Fix an interrupt handling bug in the PCjr BIOS that allows interrupts

to be enabled when they're not supposed to be (can happen any time

you hit a key).

- Generate 'break' scan codes for function keys.

PCjr BIOS should do this, but doesn't.

- NMI protector (similar to the one in JRHOTKEY).

- Fix to INT 09 similar to PCINT09 and JRHOTKEY.

- Allow Fn-PrtSc to print screen while in Fn-Pause (without un-pausing).

- Expand keyboard buffer (optional, NOBEEP.SYS version only)

- Typamatic speedup (optional)

- Add 9600 baud support (optional)

- Set 80 column mode (NOBEEP.SYS only)

- Reset the disk drive for quieter operation with DOS 3.x

- Turn on the keyboard clicker (optional)

- Turn off the system unit beeper (optional)

This package installs replacements for the BIOS keyboard and diskette

code.  These need to be installed in "fast" sidecar memory.  They will

not work in the 128k of "slow" memory in the system unit.  This

complicates the installation process somewhat, since DOS always installs

device drivers and TSRs in the lowest free memory.

I have provided 2 install options.  In most cases, either one will work

equally well.  In some cases, one may be better suited than the other.

Once installed, they are functionally equivalent.  The two options are:

NOBEEP.SYS is a device driver that will steal the top 4k of memory

from DOS and install itself there.  It is designed to work

with PCJRMEM.COM, which is the driver software for the IBM

memory sidecar.  It also works with the memory drivers for

most other types of expansion memory, but possibly not

all.  It uses an unconventional method of altering

interrupt vectors, which may make it incompatible with

some software (especially multi-tasking software, such as

Microsoft Windows or DesqView (I haven't tried either

one)).  Installed early in the boot process before other

device drivers and TSRs, it may work better than

NOBEEP.COM when used with other software that hooks into

the interrupts that NOBEEP takes over.  NOBEEP takes over

the disk interrupts 13H and 0Eh, the PCjr-unique keyboard

interrupts 02h (NMI) and 48h, and the serial port

interrupt 14h.

NOBEEP.COM is a TSR, a "Terminate and Stay Resident" program.  When

run, it installs itself permanently in memory taking over

the keyboard and disk drive functions while returning

control to DOS so other programs may be run.  It must be

installed in memory above 128k.  To install it, you must

first fill the lower 128k with other device drivers or

TSRs (a ramdisk will do nicely) so DOS will install it

above 128k.  NOBEEP.COM cannot be used if the lower 128k

must be left free for use by other programs.

Installation instructions:

NOBEEP has been used successfully with every DOS version from 2.1 to

4.0.  If using DOS 2.1, the DOS patches from the DOS21FIX package on

PCTOOLS should be used.  This is a must when running Prodigy.

Prodigy may hang if DOS 2.1 is used without the patches.

Copy the selected version to your DOS boot disk and modify the

CONFIG.SYS or AUTOEXEC.BAT file as follows:

NOBEEP.SYS: DEVICE=NOBEEP.SYS must be added to your CONFIG.SYS file

before the DEVICE=PCJRMEM.COM statement.  It takes over

the memory size adjustment and reboot functions of

PCJRMEM.COM.  PCJRMEM.COM or other memory driver is

still required to relocate the video buffer.  Other

memory drivers may be confused by having the DOS memory

size already adjusted before they start, and may not

work.  Do not press any keys while the system is booting

up.  NOBEEP.SYS may fail if keys are pressed before DOS

is completely installed.

NOBEEP.COM: This may be placed in your AUTOEXEC.BAT or entered at

the DOS prompt.  You must have already filled the lower

128k of memory with device drivers, ramdisks, or other

TSRs, or the installation will fail with an error

message.  Simply add NOBEEP (one word) as a line in the

AUTOEXEC.BAT file on your DOS disk.  To minimize

interference with other TSRs, it should be placed before

other TSRs in the AUTOEXEC.BAT file.  If you need other

TSRs to fill the lower 128k, it will of course have to

go after them.  Some experimentation may be required to

determine what order TSRs should be installed to get

everything to work.  If you do not have an AUTOEXEC.BAT

file, create one with the following 3 lines:

NOBEEP

DATE

TIME

NOBEEP and JRHOTKEY (from PCTOOLS) both contain NMI protectors.  If

both NMI protectors are installed, NOBEEP's must be installed first

or NOBEEP and JRHOTKEY will fight each other, 'protecting' different

NMI handlers.  At least one of the following must be observed if you

want to use JRHOTKEY with NOBEEP:

- Install JRHOTKEY with option "H" ("Hands off NMI").  This is

preferred since having two NMI protectors installed at the same

time is redundant and unnecessary.  NOBEEP will provide the only

NMI protector.

- Install NOBEEP.COM before JRHOTKEY (not a concern with NOBEEP.SYS,

since device drivers get loaded before TSRs).  This will result in

two NMI protectors 'protecting' the same NMI handler.  This will

work, but is not as efficient, since each NMI protector causes a

very slight loss in processing speed.

NOBEEP and JRHOTKEY both contain the "PCINT09" fix.  While this will

not cause problems, it is not necessary.  If using JRHOTKEY or

another PCINT09-like fix, use NOBEEP option /P to eliminate the

extra copy of this fix.

Options:

Some functions are optional.  The following switches are provided

for controlling these functions:

-  Search for memory above 640k (NOBEEP.SYS only)

NOBEEP.SYS will, by default, search for memory between 640k and

736k, and make it available to DOS if found.  Since NOBEEP handles

the memory size adjustment, PCJRMEM.COM can be used even if you

have 736k.  Options:

/X     Do not search for or give DOS memory above 640k.

/Mnnn  Do not search for memory.  Set memory size to nnn k bytes.

(nnn must be equal to or less than what you have installed)

nnn is a decimal number (example: /M640).  4k bytes will be

deducted from the specified size to allow room for NOBEEP's

code to reside above DOS.

/Mnnn may be useful for:

- Determining minimum software memory requirements

- Reserving memory fur special non-DOS functions

- Forcing option /A to test broken unrecognized memory

-  Test for memory errors (NOBEEP.SYS only)

Verifies that memory is working correctly.  This is important

because the PCjr's memory is not parity checked.  The POST test of

memory below 640k and NOBEEP's test of memory above 640k provide the

only protection against memory errors.  The default is for NOBEEP to

test memory if the memory size is changed from the size set by POST.

NOBEEP's test can be disabled if desired to save time (I strongly

recommend letting this test run at least once when first installing

new memory above 640k).  The test can also be enabled to include all

memory above 128k (the test is more effective than POST, since it

can detect addressing faults, but is a bit slower as a result).

Options:

/N  Do not test memory

/A  Test all memory above 128k.

Memory above 640k can be used without being tested; just specify /N.

A bargraph is drawn (darkened in in 4 passes) as the test runs.

Successful completion is indicated if this completes with no error

messages.  In the event of a failure, failing address and data are

displayed.

-  Expanded keyboard buffer

This will let you type ahead farther before getting a beep due

to keyboard buffer full.  Options:

/K  103 key buffer.  Takes an additional 256 bytes from DOS

(available with NOBEEP.SYS only).

/S  15 key buffer (default)

Some software will not work if the buffer is moved.  According to

the KEYBUF package on PCTOOLS: YTERM, I3101, COLORKEY, DOSAID,

QUICKEYS, and maybe even ANSI.SYS, fall into this category.

I have found that some of these will work in some cases depending

on the DOS version being used and other resident software installed.

-  PCINT09 fix

This duplicates the fix in Max Maurer's PCINT09 package.  This was

developed to allow Microsoft QuickBasic 3.0 to work on the PCjr.

Option:

/P  causes this fix to not be installed.

JRHOTKEY also includes the PCINT09 fix.  If you are using JRHOTKEY,

/P may be used to prevent multiple copies of the PCINT09 fix from

being installed.

-  Impulse / Alpha / Micro Products memory sidecar

NOBEEP cannot write to disk successfully when it is running in Impulse

memory (the problem is hardware, not the Impulse driver software).

This version of NOBEEP will recognize that the write has failed, and

will revert to ROM BIOS to retry the write operation.  This prevents

the corrupted disks that could occur with some earlier versions of

NOBEEP.  Retries double the time required for writing to disk, though.

To prevent the increased write time caused by this, NOBEEP can be told

to go directly to ROM BIOS for disk writes.  ROM BIOS will cause beeps

if you type while the disk is running, though, so the beeps will not be

fixed when writing to disk with Impulse memory.  The following options

determine whether NOBEEP or ROM BIOS will handle disk reads and writes:

/I  This is the default if you have an Impulse 100 sidecar with the

Impulse EPROM.  If you have a different type of Impulse sidecar,

option /I must be specified, as NOBEEP can only recognize

Impulse memory if the EPROM is present.  If option /I is

specified or assumed, message "Impulse patch applied" is

displayed when NOBEEP is installed.  /I causes NOBEEP's

replacement disk code to be used for disk reads only.  BIOS will

be used for writes (beeps will occur if you type while writing

to disk).

/W  This is the default if no Impulse memory is found.  /W causes

NOBEEP to handle all reads and writes.  With a clock speedup,

NOBEEP may be able to write to disk successfully with Impulse

memory (this has not been tested).  If this is true, specifying

/W will override the default of /I for Impulse.

/B  causes BIOS to be used for all disk accesses.  The 'beeps' fix

will not be installed.

-  Keyboard speedup

Options:

/0  typamatic disabled

/H  typamatic slowdown (half the normal rate)

/1  normal typamatic rate (default)

/2  2x typamatic speedup

/3  3x typamatic speedup

/4  4x typamatic speedup

Note that some software may have a problem if the keyboard is

sped up excessively.  The BASIC interpreter is a good example.

With BASIC, for example, keys are put into the keyboard buffer

faster than basic can remove them.  This can be damaging if it

happens with the backspace key, since after you release the key

BASIC will continue taking backspaces from the buffer, erasing

more text until the buffer empties.

- Serial port patch to allow 9600 baud (always installed by NOBEEP.SYS,

not provided unless requested with NOBEEP.COM).  Option:

/9  installs this patch. Takes an additional 112 bytes from DOS.

- Keyboard clicker

/C  will turn the keyboard clicker on and off (equivalent to

pressing Ctrl-Alt-CapsLock).

- Beeper

/Q  will quiet the internal (system unit) beeper.  Audio will be

output through the monitor speaker and rear panel audio jack

only.  If the monitor volume is fully turned down, there will

be no sound.  Note that this change may not be permanent - it

is possible for a program to turn the beeper back on.

JRHOTKEY is one program that turns the beeper back on.

Note:

Disabling the beeper also disables the cassette motor relay.

- Change number of floppy disks in BIOS equipment flags (NOBEEP.SYS only)

This is provided as a way of enabling additional floppy drives that

you have added (this is a replacement for the patches some people

had applied to PCJRMEM.COM - those patches will no longer work).

This can also be used in place of patched DOS boot records.  If you

are using some other method to get DOS to recognize additional disk

drives, this option may not be required.  Drives enabled with

DRIVER.SYS should NOT be included here.  This defines the drives

that are known before defining additional drives with DRIVER.SYS.

It may be possible to use 3.5 inch drives without DRIVER.SYS by

using a combination of this option and SET720.COM from PCTOOLS.

Options:

/D1  sets 1 floppy disk drive

/D2  sets 2 floppy disk drives

/D3  sets 3 floppy disk drives

/D4  sets 4 floppy disk drives

The following are included by default and do not need to be selected:

- Eliminate keyboard beep (except with Impulse memory sidecars).

- Not enable interrupts when keys are pressed.

- Generate 'break' scan codes for function keys.

- Allow Fn-PrtSc to print screen while in Fn-Pause.

- NMI protector.

- PCINT09 fix.

Options can be placed in any order.  Invalid options will be ignored.

Examples:

DEVICE=NOBEEP.SYS /K /3 /C  (in CONFIG.SYS, for NOBEEP.SYS)

NOBEEP /2 /9                (in AUTOEXEC.BAT, for NOBEEP.COM)

After installation:

NOBEEP cannot be removed, but some options can be modified.  The

typamatic rate can be changed, the clicker turned on and off, the

beeper turned off, and the default 15 key keyboard buffer can be

restored (if the 103 key buffer was installed by NOBEEP.SYS) by

(re)running NOBEEP.COM, specifying the new options.  This works

regardless of whether you installed NOBEEP.SYS or NOBEEP.COM.

Example:

NOBEEP /1

will return to the normal typamatic rate.

Memory usage:

NOBEEP.SYS takes 4096 bytes of memory (4352 bytes if /K is used).

NOBEEP.COM takes 3504 bytes of memory (3616 bytes if /9 is used,

2112 bytes if /B is used alone without /9) *

* The memory used by NOBEEP.COM may vary depending on your environment

size.

Interrupts used by NOBEEP:

The following interrupts are replaced by NOBEEP.  Any other device

drivers or TSRs that want to hook into these interrupts must be

installed after NOBEEP.

- INT 02 (NMI) - keyboard interrupt

- INT 48 - PCjr to PC keyboard translation

- INT 13 - diskette interrupt (some requests may be passed along to the

previous INT 13 owner:  ROM BIOS or possibly a disk device driver).

- INT 0E - diskette timeout

- INT 14 - serial port (may not be relaced by NOBEEP.COM - user option)

The following interrupts are hooked into by NOBEEP.  Any other device

drivers or TSRs that replace these interrupts must be installed before

NOBEEP.

- INT 08 - timer tick *

- INT 09 - keyboard *

* NOBEEP.COM hooks into these interrupts immediately when it is loaded.

NOBEEP.SYS does not hook into these interrupts until the first key is

pressed (they cannot be hooked into when NOBEEP.SYS is loaded, since

DOS has not set up these interrupts yet).

Special instructions for users of my older PCJR9600 package:

PCJR9600 and NOBEEP must not be used together  (they are both

replacement keyboard NMI handlers - only one NMI handler can

be active at one time).

If you use NOBEEP.COM (the TSR), use option /9 to get the same

9600 baud fix that was provided by PCJR9600.

If you were using the PCJR9600 S or M options to receive the

XPRESS or XCHANGE cable news service, the interrupt handling

changes provided by this package provide the equivalent function.

No special options are required for use with XPRESS or XCHANGE,

just load NOBEEP (with the /9 option if you use NOBEEP.COM), and

go.  As before, if you don't have the internal modem, you will

have to use "MODE COM1: 9600,N,8,1", or run a COM port swapping

program before startup and select the "secondary" communications

port in the XPRESS/XCHANGE PC setup menu.  You also need to set

80 column mode before starting XPRESS or XCHANGE.

Request my PCJR9600 package for more comments on using XPRESS and

XCHANGE on the PCjr.

Some possible problems:

-  Several keystrokes "disappear" then all appear at once when

another key is hit.

Cause:  Some other program has stolen INT 08.

-  Beeps still occur while accessing diskette.

Cause:  1. INT 02 (NMI) or INT 13 (diskette) has been stolen.

If you are also using JRHOTKEY, be sure you have followed

the special instructions in the text above.  2.  Option /I or

/B has been specified or assumed (see description of these

options above).  Look for message "Impulse Patch Applied"

when NOBEEP is loaded.  If present, your machine appears to

have an Impulse 100 sidecar.

-  Beeps occur at random times when the disk drive is not running.

Cause:  Some clock speedups seem to cause this.  I do not have a

solution, but if someone would like to try to work with me on

this problem, please contact me.

I also have one report of beeps with NOBEEP.COM without a clock

speedup.  Switching to NOBEEP.SYS solved the problem.  I do not

know the cause, but interference with other resident software is

suspected.

-  The disk keeps spinning and programs stop if keys are held down.

Cause:  The PCjr cannot handle the disk drive and keyboard at

the same time.  PCjr BIOS was written to favor the disk drive,

causing "the beeps".  I have changed it to favor the keyboard.

The PCjr now cannot finish a disk access while keys are held down.

Each time you hit a key while the disk drive is running, you

are delaying the disk access.  If you hold a key down (even a

key that "does nothing", like a shift key), the disk access will

never finish.  You can type while the disk is spinning, but each

time you do, you are delaying the disk access.

-  "ERROR D" or "ERROR E" during POST, when connected to XPRESS or

XCHANGE.

Cause:  POST interprets the data coming from your XPRESS/XCHANGE

receiver as an error.  POST does not expect the serial port to be

receiving data while it is testing the port.  Press enter to

continue, ignoring the error.

-  /K does not provide an expanded keyboard buffer.

Cause:  You have other device drivers ahead of NOBEEP.SYS that

consume too much memory.  The keyboard buffer (which is contained

in memory owned by the device driver) must be somewhere

between 1k and 65k in memory.  If NOBEEP.SYS gets installed

above 65k, it can't contain the keyboard buffer, so it leaves

the default buffer installed.  It may be possible that later

releases of DOS may be large enough to exceed 65k by themselves.

If this occurs, /K will not work.

-  The DOS clock is not as accurate with NOBEEP installed.

Cause:  This is an unfortunate side effect of this change.

Normally, the PCjr uses some of the keyboard hardware to keep

track of the time of day while it is accessing the diskette.

NOBEEP keeps the keyboard enabled during diskette accesses, so

this hardware cannot be used to keep track of the time of day.

NOBEEP adds a fixed value to the time of day each time the disk

is accessed to try to keep the time as accurate as possible.

- PCjr locks up if keys are pressed while booting.

Do not do this if you are using NOBEEP.SYS.  Part of the install

process cannot be done at the time NOBEEP.SYS is loaded.  It must

be done later, after DOS has set up interrupt vectors.  NOBEEP

assumes this has been done when you press the first key, and

finishes installing itself at that time.

- Disk accesses take longer with NOBEEP installed.

Cause: NOBEEP may be getting errors on each disk access and

going to BIOS for retries.  Try option /I or /B (see instructions

for these options).

Problem reports:

Please indicate the version number of the program and what options

you selected when you installed NOBEEP.  Also include the name of

your memory expansion driver (PCJRMEM.COM or other) and type of

memory expansion (IBM, Microsoft, Impulse, etc).  Limited support

will be available, as my time permits.