D    TECHNICAL PARAMETERS

For most people, two speed parameters and the

Inboard 386/PC environment selected by the SETUP

program will handle all their programs.

Occasionally, however, you may need a different

speed, or you may want to change the Inboard

386/PC environment.  This appendix describes the

additional speed parameters and describes

manually installing the Inboard 386/PC software.

The ISPEEDPC parameters

Because of timing constraints, some application

programs can't run at very fast speed (4).  You

can slow down the computer's operating speed by

adding wait states -- speed decreases as the

number of wait states increases.  Table D-1

shows the number of wait states Inboard 386/PC

adds for each speed.

Table D-1    Wait states

________________________________________

Speed     Number of additional wait states

_________ _________________________

1         30

_________ _________________________

2         16

_________ _________________________

3         8

_________ _________________________

4         0

___________________________________

Manually installing the InboardTM 386/PC software

Manually installing the Inboard 386/PC software

is a two-part process:  copying the software to

the root directory and adding commands to the

CONFIG.SYS and AUTOEXEC.BAT files.  If you'd

rather have the software do this for you, turn

to Chapter 7.

TECHNICAL PARAMETERS                               D-1

Copying the InboardTM 386/PC software

Before you can use the Inboard 386/PC software,

you must copy the files you need from the

Inboard 386/PC diskette to your hard disk's root

directory or to your system diskette.  Copy only

the files you need.

Chapter 7 describes the files on the Inboard

386/PC diskette.

Setting up the InboardTM 386/PC software

NOTE___________________________________________________

The only Inboard 386/PC software you must set up

is INBRDPC.SYS (described in step 1).  All other

Inboard 386/PC software is optional.

1    Add the following command to the first line of

the CONFIG.SYS file in the hard disk's root

directory or on the system diskette.

DEVICE=INBRDPC.SYS  [SPEED=n] [EGACACHE]

[NOCACHE] [NODIAGS]

[NOPAUSE] [NOSCRUB]

[SYSWAIT=n] [FDWAIT=n]

[HDWAIT=n]

The parameters in brackets ([ ]) are optional.

You may include none or one or more on the

command line.  Don't type the brackets.  Table

D-2 describes these parameters.

Table D-2    INBRDPC.SYS optional parameters

_____________________________________________________

Parameter      Description

______________ ___________________________________

SPEED=n        Specifies the initial operating

speed of the

Inboard 386/PC.  The variable n

can be 1

(very slow) or 4 (very fast).  If

you don't

specify SPEED=n on the command

line, the

computer starts in very fast

speed.  Almost

all application programs run fine

in very fast

speed (SPEED=4).

D-2                               TECHNICAL PARAMETERS

If you're like most users, very

fast speed and

very slow speed are all you'll

ever need.  But

for the rare situation where

neither is right,

Intel provides two additional

speeds (refer to

Table D-1).

_____________________________________________________

TECHNICAL PARAMETERS                               D-3

Table D-2    INBRDPC.SYS optional parameters (cont.)

_______________________________________________________

Parameter      Description

______________ ___________________________________

EGACACHE       Specifies that the EGA ROM BIOS

(if

present) will run in the Inboard

386/PC's

32-bit memory.  This means the

EGA display

will update faster.  Use EGACACHE

only if

you have an EGA monitor.  Don't

use

EGACACHE if you have an EGA board

set

to AUTOSWITCH mode.  The auto-

switch

EGA BIOS can't be cached.

Depending on the size of the ROM

BIOS, the

EGACACHE parameter reserves up to

32K

bytes of Inboard 386/PC 32-bit

conventional

memory for caching the EGA ROM

BIOS.

______________ ___________________________________

NOCACHE        By default, the system BIOS is

executed

from the Inboard 386/PC 32-bit

RAM.

NOCACHE lets you switch system

BIOS

execution back to the computer's

slower

ROM.

In some cases, you can't warm-

boot the

computer (by pressing

Ctrl/Alt/Del) if you

specify the NOCACHE parameter.

______________ ___________________________________

NODIAGS        By default, extended memory

diagnostics

are run when you turn on or

restart the

computer.  NODIAGS lets you turn

off the

D-4                               TECHNICAL PARAMETERS

extended memory diagnostics.  If

extended

memory parity errors are detected

when you

turn on or restart the computer,

the

diagnostics will be run even with

NODIAGS

on the command line.

Intel recommends you don't use

NODIAGS

-- the diagnostics are fast and

give you an

extra measure of protection.

______________ ___________________________________

NOPAUSE        The Inboard 386/PC software

pauses after it

installs -- you have to press a

key to

continue.  The NOPAUSE parameter

eliminates this pause unless an

error occurs.

The software always pauses for

error

conditions.

_____________________________________________________

TECHNICAL PARAMETERS                               D-5

Table D-2    INBRDPC.SYS optional parameters (cont.)

_______________________________________________________

Parameter      Description

______________ ___________________________________

NOSCRUB        By default, conventional memory

is initialized

when you turn on or restart the

computer.

NOSCRUB lets you specify that

conventional

memory not be initialized.

Be aware that not initializing

conventional

memory may result in

the computer's getting parity

errors.

______________ ___________________________________

SYSWAIT=n      By default, no wait states are

added for the

computer's operation (except for

floppy disk

I/O).  SYSWAIT lets you specify a

number of

wait states added for the

computer's

operation.  (These wait states

don't affect

floppy disk or hard disk I/O.

Refer to the

FDWAIT and HDWAIT parameters for

floppy

disk and hard disk I/O wait state

parameters.)

n is an even number from 0 to 30

(inclusive).

Don't put both SYSWAIT and

SPEED=n on

the DEVICE=INBRDPC.SYS command

line.

You can use one or the other but

not both.

______________ ___________________________________

FDWAIT=n       System BIOS code executes much

faster

when it runs from Inboard 386/PC

32-bit

RAM.  Use FDWAIT to slow down

timing-

D-6                               TECHNICAL PARAMETERS

sensitive BIOS routines (such as

floppy disk

code).

By default, 16 wait states are

added for

floppy disk operations.  FDWAIT

lets you

specify a number of wait states

added for

floppy disk operations.

n is an even number from 0 to 30

(inclusive).

TECHNICAL PARAMETERS                               D-7

Table D-2    INBRDPC.SYS optional parameters (cont.)

_______________________________________________________

Parameter      Description

______________ ___________________________________

HDWAIT=n       By default, no wait states are

added for hard

disk operations.  HDWAIT lets you

specify a

number of wait states added for

hard disk

operations.  Because most hard

disk BIOSs

can run at very fast speed, most

users won't

need to use HDWAIT.

n is an even number from 0 to 30

(inclusive).

_____________________________________________________

CAUTION________________________________________________

Don't ignore error messages, even if they don't

prevent you from using the computer.  Correct

any error conditions as soon as you receive an

error message.

The following are four sample INBRDPC.SYS

command lines.

o The command line should look like this if:

- you don't have an EGA monitor

- you want the Inboard 386/PC to run in very

fast speed (the

default)

- you don't want to pause after the Inboard

386/PC software

installs.

DEVICE=INBRDPC.SYS  NOPAUSE

o The command line should look like this if you

have the same situation as sample 1 plus you

don't want the system BIOS to run from Inboard

386/PC 32-bit RAM.

DEVICE=INBRDPC.SYS  NOCACHE  NOPAUSE

o The command line should look like this if:

- you have an EGA monitor which you want cached

in 32-bit

memory

D-8                               TECHNICAL PARAMETERS

- you want the Inboard 386/PC to run in very

fast speed (the

default)

- you want 30 wait states added for floppy disk

operations.

DEVICE=INBRDPC.SYS  EGACACHE  FDWAIT=30

TECHNICAL PARAMETERS                               D-9

The command line should look like this if:

- you don't have an EGA monitor

- you don't want diagnostics

- you want the Inboard 386/PC to run in fast

speed.

DEVICE=INBRDPC.SYS  NODIAGS  SPEED=3

2    ILIM386.SYS turns extended memory into expanded

memory.  If you already have an expanded memory

board (such as an Intel Above Board) in the

computer, you can either continue to use the

expanded memory board or turn extended memory on

the Inboard 386/PC into expanded memory.  You

can't do both.  Intel recommends you continue to

use the expanded memory board.

ILIM386.SYS doesn't support expanded memory on

Above Boards -- use EMM.SYS.  If you install

ILIM386.SYS, you must disable expanded memory on

Above Boards.

NOTE___________________________________________________

The DEVICE=ILIM386.SYS command must come after

the DEVICE=INBRDPC.SYS command and before any

other device drivers in the CONFIG.SYS file.

NOTE___________________________________________________

Programs that put the computer in virtual 8086

mode can't run with other programs that put the

computer in virtual 8086 mode.  For example,

Windows 386 and ILIM386.SYS can't run together.

This is the nature of these progams and has

nothing to do with the Inboard 386/PC.

To use ILIM386.SYS, add the following command

line to the CONFIG.SYS file:

DEVICE=ILIM386.SYS  [EMS=n or EXT=n]  [FRAME=x]

The brackets ([ ]) specify an optional

parameter.  Don't type the brackets.  Table D-3

describes these parameters.

NOTE___________________________________________________

ILIM386.SYS uses approximately 56K bytes of

extended memory.  This reduces the total amount

of extended memory available.

D-10                              TECHNICAL PARAMETERS

Table D-3    ILIM386.SYS optional parameters

_______________________________________________________

Parameter      Description

______________ ___________________________________

EMS=n          Specifies the amount of extended

memory to

be turned into expanded memory.

The

values for n range from 0K bytes

to the

total amount of extended memory

on the

Inboard 386/PC and the optional

Piggyback

Memory boards.**  n must be a

multiple of

16.

(Type just the number; don't type

the K.)

OR*

EXT=n          Specifies the amount of extended

memory

not turned into expanded memory

(that is,

the amount of extended memory

left as

extended memory).  The values for

n range

from 0K bytes to the total amount

of

extended memory on the Inboard

386/PC

and the optional Piggyback Memory

board.

If n isn't a multiple of 4,

ILIM386 will round

the value of n up to the next

multiple of 4.

(Type just the number; don't type

the K.)

Be sure n equals or exceeds the

amount of

extended memory needed by all

extended

memory device drivers.

_____________________________________________________

TECHNICAL PARAMETERS                              D-11

*You can specify the EMS=n parameter or the

EXT=n parameter, but not both.  Use EXT=n when

you need a specific amount of extended memory,

such as when caching the hard disk with

ICACHE.COM (described in step 3).  When all you

care about is expanded memory, and the amount of

extended memory left over doesn't matter, use

the EMS=n parameter.  Or don't specify either

EMS=n or EXT=n and use the default value of

turning all available extended memory into

expanded memory.

**ILIM386.SYS resides in approximately 56K bytes

of extended memory.  So the maximum amount of

expanded memory available when ILIM386.SYS is

installed is the total amount of extended memory

minus 56K.  Consider using the EXT parameter

instead of the EMS parameter.  Any memory not

specifically assigned as extended memory by the

EXT parameter and not used by ILIM386.SYS is

turned into expanded memory.

D-12                              TECHNICAL PARAMETERS

Table D-3    ILIM386.SYS optional parameters (cont.)

_______________________________________________________

Parameter      Description

______________ ___________________________________

FRAME=x        Specifies the address (in

hexadecimal) of the

64K-byte page frame through which

expanded memory is mapped.  If

you don't

include the FRAME parameter, the

default

address of E000 is used.

Intel recommends you not use the

FRAME

parameter.  That way, if the

default address

of E000 isn't available, the

ILIM386.SYS

program will automatically select

a non-

conflicting address.

If you want to choose the page

frame

address, include the FRAME

parameter,

replacing x with one of the

following

addresses:

Hex addresses

E000      D400      C800

DC00      D000      C400

D800      CC00      C000

Try E000 first.  If this causes a

conflict with

other boards in the computer, try

DC00 and

so on.

_____________________________________________________

The following are two sample ILIM386.SYS command

lines.

o The command line should look like this if you

want to turn 1024K bytes of extended memory into

expanded memory.

ILIM386.SYS  EMS=1024

TECHNICAL PARAMETERS                              D-13

o The command line should look like this if you

want to reserve 512K bytes of extended memory

and turn the rest into expanded memory.

ILIM386.SYS  EXT=512

D-14                              TECHNICAL PARAMETERS

3    ICACHE.COM speeds disk reads by storing the

information from the computer's hard disk in

Inboard 386/PC 32-bit extended memory.

To use ICACHE.COM (only if the computer has a

hard disk), add the following command line to

the AUTOEXEC.BAT file:

ICACHE  [/S:n]  [/B-]  [/T-]  [/-x]

Table D-4 describes these parameters.

Table D-4    The ICACHE.COM parameters

_______________________________________________________

Parameter      Description

______________ ___________________________________

/S:n           Specifies the amount of extended

memory

(in K bytes) used for caching the

hard disk.

Values for n range from 48 to

1024.  If you

don't specify the S parameter,

the default

value of all available extended

memory up to

1024K bytes is used.

______________ ___________________________________

/B-            By default, data is copied to and

from the

disk cache in batches of sectors.

This

increases performance, but batch

copying

can increase the time interrupts

are turned

off.

If you have problems during high-

speed data

communications (for example,

using a

modem at 4800 bps or faster),

specify the

/B- parameter.

_____________________________________________________

TECHNICAL PARAMETERS                              D-15

Table D-4    The ICACHE.COM parameters (cont.)

_______________________________________________________

Parameter      Description

______________ ___________________________________

/T-            By default, when a program

requests a disk

read, the rest of the track is

also read into

cache memory.  This lets ICACHE

read

more sectors from the hard disk

at one time.

Because DOS often requests

information

from nearby sectors soon after

requesting a

given sector, reading the rest of

the track in

advance usually increases

performance.

The T- parameter disables whole-

track

reads.  Consider using the T-

parameter in

the following situations:

- You have little conventional

memory to

spare (the software uses 12.5K

bytes of

conventional memory).

- Your programs access many small

files

scattered over the disk.

Reading the rest of

the track will give little

performance gain

and may actually bump sectors

in the

cache that are read frequently,

decreasing

performance slightly.

- Your disk has many bad sectors

that have

been mapped out.  If the files

you are

reading are on the same tracks

as these

bad sectors, reading the rest

of a track can

cause an error.  While ICACHE

automatically backs off and

reads only the

D-16                              TECHNICAL PARAMETERS

data originally requested, this

can waste

time.

______________ ___________________________________

/-x            Specifies that drive x not be

cached.  You

can specify as many drives as you

wish, as

long as each is preceded by "/-."

If you

specify a drive that is a

partition, both drives

won't be cached.

x is the letter of the disk drive

you don't want

cached.

_____________________________________________________

TECHNICAL PARAMETERS                              D-17

The following are three sample ICACHE command

lines.

o The command line should look like this if:

- you want a 256K-byte disk cache

- you don't want batching because you have a

modem running at

9600 bps.

ICACHE  /S:256  /B-

o The command line should look like this if:

- you want a 512K-byte disk cache

- you have two hard drives (C and D) and don't

want to cache

drive D.

ICACHE  /S:512  /-D

o The command line should look like this if:

- you want the disk cache to be the size of all

available extended

memory up to 1024K

bytes (the default)

- you don't want the entire track read into disk

cache memory.

ICACHE  /T-

You can enter the following command after

ICACHE.COM is installed (the parameters are

described in Table D-5).

ICACHE  [/M]  [/F]  [/P]  [/U]

D-18                              TECHNICAL PARAMETERS

Table D-5    The ICACHE.COM post-installation

parameters

_______________________________________________________

Parameter      Description

______________ ___________________________________

/M             Displays the number of disk

transfer

requests, the number oftransfers

from the

disk, the number of disk

transfers saved by

the disk cache, and the

percentage of

transfer requests saved by the

disk cache.

______________ ___________________________________

/F             Flushes the cache and resets its

measurements to zero.  This

resets the disk

cache to its initial state, which

can be useful

when performing benchmarks.

______________ ___________________________________

/P             Displays all the parameters

specified.

______________ ___________________________________

/U             Un-installs the disk cache.  This

is useful

when you want to un-install the

disk cache

and re-install it with new

parameters.

_____________________________________________________

4    Restart the computer (by pressing Ctrl/Alt/Del).

The commands you added to the CONFIG.SYS and

AUTOEXEC.BAT files won't take effect until you

do.

Sample InboardTM 386/PC software set-ups

This section shows five sample Inboard 386/PC

software set-ups.

1    Your files should look like this if:

- you don't have an EGA monitor

- you don't want to turn Inboard 386/PC extended

memory into

expanded memory

TECHNICAL PARAMETERS                              D-19

- you don't want to speed up disk reads by

caching the hard disk.

CONFIG.SYS                         AUTOEXEC.BAT

DEVICE=INBRDPC.SYS                 add nothing

non-Intel device drivers

2    Your files should look like this if you have the

same situation as sample 1 except you have an

EGA monitor that you want cached in 32-bit

memory.

CONFIG.SYS                         AUTOEXEC.BAT

DEVICE=INBRDPC.SYS  EGACACHE       add nothing

non-Intel device drivers

3    Your files should look like this if:

- you don't have an EGA monitor

- you want to set up a 128K-byte VDISK in

Inboard 386/PC

extended memory

- you want to set up a 128K-byte hard disk

cache.

CONFIG.SYS                         AUTOEXEC.BAT

DEVICE=INBRDPC.SYS                 ICACHE /S:128

DEVICE=VDISK.SYS 128 512 64 /E:8    other

commands

non-Intel device drivers

4    Your files should look like this if you have the

same situation as sample 1 and you want to

continue using an Above Board in the computer.

CONFIG.SYS                         AUTOEXEC.BAT

DEVICE=INBRDPC.SYS                 add nothing

DEVICE=EMM.SYS PC D000 258

non-Intel device drivers

Note that the EMM.SYS command line parameters

changed with EMM 4.0.  Refer to Appendix C for

information on using expanded memory on Above

Boards when the Inboard 386/PC is installed.

5    Your files should look like this if you have the

same situation as sample 1 plus:

- you want to turn extended memory into expanded

memory,

while reserving 256K bytes for extended memory

- you want to set up a 256K-byte hard disk

cache.

CONFIG.SYS                         AUTOEXEC.BAT

D-20                              TECHNICAL PARAMETERS

DEVICE=INBRDPC.SYS                 ICACHE /S:256

DEVICE=ILIM386.SYS EXT=256          other

commands

non-Intel device drivers

Where to go from here

This completes the Inboard 386/PC software

installation.  Chapter 7 explains how to use the

software.

TECHNICAL PARAMETERS                              D-21