UNLOAD86.COM is a program that converts or "unloads" a file into it's INTeL

hex representation.  Alternately, it will also output a BNPF, BHLF, OR B10F

type file for those users that require these formats.  This program was

written in response to the need to have INTeL hex files for downloading to

an EPROM programmer.  As MS-DOS programs don't support INTeL hex, the need

to convert binary files to hex arose.  Most development systems, INTeL, CP/M,

& others do support INTeL hex, and most EPROM programmers expect it.

Use of the program is simple:

UNLOAD86  <RETURN>

prints a brief help screen.

The main form of the unload86 invocation is:

UNLOAD86 FILENAME.TYP [/OPTIONS]

B  output a BNPF file instead of INTeL hex.  This option overrides any other

selections (Except Z).  If the B option is chosen, no other options will work,

and the program will output a straight 8-bit BNPF file, uless it has been

modified for BHLF, or B10F.

H  outputs a raw ASCII hex file (non-Intel hex) without any address information,

checksums, address records, etc.  Overrides all other options except B (which

overrides this) and Z.  Simply outputs 32 ASCII hex bytes on a line without

intervening spaces.  Is compatable with the TRS-80 ASCII hex scheme sometimes

used for file transfers.

Z  Simply adds a Control-Z end-of-file mark (1A Hex) to the end of the

output file, be it INTeL hex or BNPF.  Most well-behaved programs ignore

this, some expect it, but some can't tolerate it.

L  Selects the new "extended linear address record".  See the discussion

at the end of this document for details.  If you don't know what this is,

you probably don't need it.  Interacts with /E, /O, & /P.  Automaticly

turns on /O, thus /O shouldn't be used with /L as /O will turn off /L.

/L also turns off /E & /P.

O Override extended addressing.  Any output file representative of 64K or

greater normally uses extended address records.  This is any file created

with /PC, /PD, /PE, or /PF, as well as any time the input file is longer

than 64K.  This inhibits the extended address records for the rare cases

where this is needed.  Interacts with /E.  Whichever is last is the effective

switch, normaly only one would be used at a time.  This option also turns

off /L

E forces the use of extended address records for file smaller than 64K.  Files

larger than 64K already automaticly select extended addressing.  Interacts

with /O and /L.  Whichever is last is the effective switch, normaly only one

would be used at a time.

T forces the output to CR-only TRS-80 or MACintosh style line termination.

U forces the output to LF-only UNIX style line termination

If neither T or U is specified, output is standard CR-LF MS-DOS or CP/M

style line termination.  If both are specified, the last one is in effect.

P allows selection of an EPROM size so that all unused locations are filled

with the user specified data (defaults to FF)  This option interacts with

/L.  Whichever is last is the effective switch, normaly only one would be

used at a time.  This option is followed immediately by a parameter number

0-F.  This parameter is:

0  32 bytes.  Creates a file to fill a 32 byte ROM.  All unused locations

are filled with the user defined data.  As with all these options, if the

file is already longer than 32 bytes, an error message will be displayed

on the system console, and the operation will be aborted.  The file left in

this case is a useful INTeL hex file, but isn't padded up to size as it is

already too big for the device specified.  32 byte devices are generally

little bipolar ROMs often used as memory decoders

1  64 bytes.  This is also generally a bipolar device.

2  128 bytes.  Bipolar.

3  256 bytes.  1702A EPROM, some bipolars

4  512 bytes.  2704 EPROM.

5  1k bytes.  2708 EPROM.

6  2k bytes.  2716 EPROM.

7  4k bytes.  2732 EPROM.

8  8K bytes.  2764 EPROM.

9  16K bytes.  27128 EPROM.

A  32K bytes.  27256 EPROM.

B  64K bytes.  27512 EPROM.

C  *128K bytes.  27010 EPROM.

D  *256K bytes.

E  *512K bytes.

F  *1M bytes.

* These options force the generation of extended addressing and thus

do not require the use of the 'E' option.  This may be overridden

with the 'O' or 'L' options.

It was felt un-necessary to provide padding when the /L switch is used as

an extended linear address is normally used only for files larger than 1

megabyte.  If you are reading this in 1999 and you're still stuck with a

computer that can run MS-DOS programs, need to pad a 4 megabyte PROM, you're

getting what you deserve!  Get a real computer!

It should be noted that all INTeL hex output, regardless of options selected,

is padded up to the next 32 byte increment.  This makes the /P0 parameter

redundant, but what else am I going to do with it?

ERRORLEVEL support.  UNLOAD86 now supports ERRORLEVEL.  It exits with 0

for no errors, and 255 if there was some kind of error.

EXAMPLES:

UNLOAD86 FOOBAR.COM/B    <-- produces a BNPF file called FOOBAR.BNP

UNLOAD86 FOOBAR.COM/EB   <-- produces a BNPF file called FOOBAR.BNP

UNLOAD86 FOOBAR.COM/E    <-- produces a hex file called FOOBAR.HEX

w/extended addressing

UNLOAD86 FOOBAR.COM/EZ   <-- produces a hex file called FOOBAR.HEX

w/extended addressing & CNTRL-Z at the

end of the output file

UNLOAD86 FOOBAR.COM/L    <-- products a hex file called FOOBAR.HEX

w/extended linear addressing

UNLOAD86 FOOBAR.COM/LE   <-- produces a hex file called FOOBAR.HEX

w/extended addressing

UNLOAD86 FOOBAR.COM/EL   <-- products a hex file called FOOBAR.HEX

w/extended linear addressing

UNLOAD86 FOOBAR.COM      <-- produces a hex file called FOOBAR.HEX

w/extended addressing if input >64K

UNLOAD86 FOOBAR.COM/P0   <-- produces a hex file called FOOBAR.HEX

padded to 32 bytes.

UNLOAD86 FOOBAR.COM/PF   <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, w/ extended addressing

UNLOAD86 FOOBAR.COM/PFO  <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, WITHOUT extended addressing

UNLOAD86 FOOBAR.COM/PFE  <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, w/ extended addressing

UNLOAD86 FOOBAR.COM/EPF  <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, w/ extended addressing

UNLOAD86 FOOBAR.COM/EPFB <-- produces a BNPF file called FOOBAR.BNP

UNLOAD86 FOOBAR.COM/H    <-- produces a raw ASCII hex file called FOOBAR.HEX

Notes, patches, etc.:

UNLOAD86 cannot realisticly handle binary input larger than a megabyte as

the extended addressing scheme runs out of addresses.  If you unload a file

larger than 1 megabyte, the extended addressing will wrap around if it has

been selected with the /E or the /Px options.  This limitation does not apply

if you use the /L switch to select the extended linear address records.

There are several user patchable options that can be patched with DEBUG.

They are as follows:

102:  FF   This is the data that is padded when using the /Px option.

Change to meet your requirements.

103:  'PN' Change to 'HL' for BHLF output instead of BNPF, or '10' for B10F.

Or any other two ASCII characters for other variations of this.

105:  '.'  This is the dot for the hex filename.  Must not be changed...

106:  'HEX' Output file type for INTeL hex.  Change to meet your needs

109:   00  This is the termination for the hex filename.  Don't change...

10A:  '.'  This is the dot for the BNPF filename.  Must not be changed...

10B:  'BNP' Output file type for BNPF.  Change to 'BHL' for BHLF, or

'B10' for B10F, or to whatever filetype you prefer for this

format.

10E:   00  This is the termination for the BNPF filename.  Don't change...

10F:   00  Change to FF to add a control-Z to the end of file by default.

In this case, the Z switch turns control-Z OFF instead of ON.

This program was written by Mark D. Pickerill on the Pro-Log ABL-2 CP/M

microcomputer. (what else?)  It bears no resemblance to UNLOAD 2.2 or

2.3 for CP/M by the same author.

THE USER BEARS ALL RESPONSIBILITY FOR TESTING AND USE!  IF THE PROGRAM

MANGLES YOUR DATA, BLOWS YOUR HARD DRIVE, MAKES YOUR CAT BARF ON YOUR

KEYBOARD, OR HAS OTHER UNDESIRED EFFECTS, YOUR SOLE RECOURSE IS TO

DISCONTINUE USE OF THE PROGRAM!!

As with all my works, AE is dedicated to my patron deity, the Goddess Athena.

If you are a member of a fanatical monotheistic religion and cannot deal with

this, oh well...

UPDATES:

Ver. 1.6.

I suddenly found myself needing to send a raw ASCII hex file with

CR-only line termination to a TRS-80.  So I plugged in 3 new options into

the program.

/H for raw ASCII hex output, no load addresses, no checksums, no address

records.

/U to force output file to UNIX style LF only line termination.

/T to force output file to TRS-80 or MACintosh CR only line termination.

If neither /U or /T is specified, lines are still terminated with the standard

CR-LF MS-DOS or CP/M line termination.

04/01/94

Ver. 1.5b. Even more minor cleanup.  Found an un-needed instruction,

deleted it.  No functional change, just the new 1.5 that will be distributed.

10/28/92

Ver. 1.5a.  Very minor cleanups to source code and documentation.  Made

the /Z option a toggle rather than a set.  No real functional changes, so

this is just the new 1.5 that will be distributed. 06/04/92

Ver. 1.5  After considerable work, and three tries, I have finally hung

a buffered disk I/O routine onto the program.  I've learned a good deal

about 80x86 programming in general, and disk I/O in particular in the four

years this program has been under development.  As the original disk routines

were closely tied to the actual UNLOAD routine, an attempt to increase

the size of data read by these routines was scrapped as un-workable.  Instead,

I borrowed a technique from my CP/M BIOS hacking days called BLOCK/DEBLOCK.

The original routines see no difference in their I/O requests, but instead

calls a couple of new routines pass the data back and forth, and maintain

a rather large set of disk I/O buffers.  The result?  As you might expect,

a huge increase in speed.  A test file, 271,634 bytes long was used.  Ver.

1.4 took 1:42 to UNLOAD this into Intel hex on a 25MHZ '286.

The new ver. 1.5 only took 0:16.  A test of BNPF resulted in 3:15 vs. 0:36

for the new version.  Plus it no-longer bangs the disk heads reading and

writing all those little blocks of data.  This was all done with a slight

DECREASE in size from 1.4 due to more intellegent buffer overlaying of the

help screen.  No operational differences from 1.4.  Released 04/23/92,

one year to the day that 1.3 was released!

Ver. 1.4  After receiving multiple complaints, I found and fixed a bug

that wrote 6 garbage bytes after the EOF mark.  Well-behaved programs

were not affected, but as MS-DOS is *full* of ill-behaved programs....

09/25/91  Also added the Z switch to turn on the CNTRL-Z EOF mark.

Ver. 1.3  It has come to my attention that Intel has finally patched

their specification to accomodate files larger than 1 Megabyte with the

addition of the "Extended Linear Address Record".  Support for this has

been added.  Also changed the help screen such that if the user patches

the file type for .HEX or .BNP, that the help screen will reflect this

change.  Added Intel hex discussion to the documentation.  04/23/91

Ver. 1.2  After receiving multiple complaints, I changed the command line

parser so that spaces are now allowed between the filename and the switch

character.  Added - and [ as alternate switch characters for UNIX and CP/M

types.  Input files larger than 64K now automaticly generate extended

address records unless overridden with the /O switch.  /O also overrides

forced extended addressing on the larger /Px options.  Added ERRORLEVEL

support. 01/18/91

Ver. 1.1  Corrects a bug that dis-allowed zeros in the output filename.

An input file of ROM0.BIN would be unloaded to ROM.HEX instead of

ROM0.HEX.  Spurious bugs possibly could have also appeared if the input

filename had no extension. 10/3/88

Ver. 1.0  Initial release. 8/10/88

DISCUSSION OF INTEL HEX:

Intel hex is a method of representing binary data as ASCII data.  This

has several advantages.  First, it can be viewed and changed with ordinary

text editors.  Second, the data has a checksum so that transmission errors

can be detected.  Third, it is an excellent way to transfer binary data through

a 7 bit port or with software that only has ASCII transfer capability.  Fourth,

and lastly, additional information such as load addresses and execution start

addresses are incoded in the file.  It has one drawback however.  The Intel

hex file is always at least slightly more than twice the size of the binary

file it represents.

Intel hex is represented in records.  All records have the same format.

They start with a colon, this is followed by a two digit byte count which tells

how many data bytes are in the data field.  This is followed by a four digit

load address which tells where the data loads in memory.  Then comes a two

digit record type specifier (discussed below) which tells what kind of record

this is.  Then comes 1 or more bytes (two or more digits) of actual data.  Last

comes a one byte (two digit) two's complement checksum (modulo-256) of

everything that came before, except of course, the colon.  Anything that comes

after this checksum, such as carriage return - line feed pairs, etc. is

ignored until the next colon.

:020000020000FC

This is a typical Intel hex record.  We will add spaces for clarity:

: 02 0000 02 0000 FC --Checksum

|  |   |  |    |

|  |   |  |    --Data bytes, two in this example

|  |   |  --Record type

|  |   --Load address for data

|  --Byte count of data record

-- Leading colon

RECORD TYPES:

There are currently 6 types of records.  The Intel hex specification has

undergone 3 different revisions over the years.  The original specification

only defined two record types, then the next revision added one more, and

the most recent has added three more to that.  These revisions were made such

that software that processes Intel hex need only write one routine to process

all different types.  Unfortunately people cannot seem to realize this and

use 3 different routines to process data that was intended to be backwards

compatable.  The exact same problem exists with the XMODEM file transfer

protocol.  XMODEM checksum, XMODEM CRC, and 1K XMODEM are all backwards

compatable, yet most file transfer software makes you select between the three.

00 - Data record

This record has a 00 in the record type field.  The byte count is typicaly

10 or 20 (hex), although it can be any number from 01 to FF.  The "wider"

the record (larger the byte count), the less overhead there is in the file,

but the checksum is less reliable.  Some software cannot handle byte counts

larger than 40.

01 - End record

This record has a 01 in the record type field.  This is the only record

type which can have (and always does have) a byte count of zero.  The most

common end record is :00000001FF.  This is a byte count of zero, a load

address of zero, a record type of 1, and the checksum of FF.  What most

people don't know is that the start execution address of the file can be

stored in the address field, with appropiate alterations to the checksum.

However, a lot of software will barf if it encounters a end record other than

the one shown here.

The above two types are the original Intel hex specification

02 - Extended address record

This record has a 02 in the record type field.  Its byte count is always 02.

It has a load address of zero, and the extended address is in the two data

bytes.  Why the address isn't stored in the address field for this, and all

succeeding record types is unknown.  This record type was added by Intel when

they introduced the 8086.  What Intel didn't make clear at the time was the

fact that this two byte extended address is actually a *segment* address.

It represents address bits 4 through 19.  So this address is the tired old

segment addition:

0000  -- "Extended address"

0000 -- "Load address"

00000 -- "Physical address"

Some software, especially software written to do Intel hex for other processors

such as the 68000 family, use the extended address record to represent address

bits 16-31.  This is incorrect!  When Intel (finaly) introducted a processor

capable of doing flat addressing, they added record type 4 for this purpose.

When UNLOAD86 uses this type of record, it starts at zero and increments it

in blocks of 64K which adds 1000 hex to this segment address.

The above three types are the "Rev 2" Intel hex specification

03 - Extended start address

This record has a 03 in the record type field.  Its byte count is always 02.

It is otherwise identical to record type 02, and is used to indicate the

segment start execution address of the file.  As UNLOAD86 has no way of knowing

this information, this record type is not supported.

04 - Extended linear address record

This record has a 04 in the record type field.  Its byte count is always 02.

The data field contains the extended linear load address, address bits 16-31.

This is the much more sensable extended address record.  When UNLOAD86 uses

this type of record, it starts at zero and increments it in blocks of 64K

which merely adds one to this address.  A lot of software cannot handle any

of these new record types.  It is recomended that this type of addressing be

used if your application and your software permit it, rather than record

type 02.  Although it is perfectly legal to use both types, UNLOAD86 only

uses one at a time as it is converting a contiguous binary file to Intel hex,

thus it doesn't make a lot of sense to use both.  Both types might be used

in the case of a compiler or assembler output where the data is non-contiguous.

05 - Extended linear start address

This record has a 05 in the record type field.  Its byte count is always 02.

It is otherwise identical to record type 04, and is used to indicate the

extended linear start execution address of the file.  As UNLOAD86 has no

way of knowing this information, this record type is not supported.

Discussion of BNPF:

BNPF is an old paper tape format that still crops up from time to

time.  It has a couple of varients, BHLF, and B10F.  They all work the

same way.

BPNPNNPNPF  ... Represents one byte.  The B stands for Begin.  The

following 8 Ns and Ps stand for Negative or Positive.  The F stands for

Finish.  Thus the byte pictured is A5 hex.  BHLF uses H & L for High

and Low instead of N & P.  B10F uses 1 and 0.  These are all ASCII characters,

although I suppose they could be coded in Baudot as well.  You'd think

from the name that BNPF would really be called BPNF, but it never is.

SUPPORT:

The "home" BBS for UNLOAD86.COM is the Hacker Heaven BBS at (408) 375-5455.

We currently support baud rates from 300 to 14.4K+v.42bis.

In this day of media hype, it is necessary to point out that the word

"Hacker" means computer programmer, NOT computer criminal.  We do not

tolerate illegal activities of any kind.

OTHER PROGRAMS:

The following is a list of currently supported programs written by

Mark D. Pickerill, the author of UNLOAD86.  These may be found on the Hacker

Heaven BBS, or a fine BBS near you.  Most have been uploaded to EXEC-PC as

well.

AE:  Asm Edit.  A source code editor for assembly language programmers.

(At last!)  Supports auto-colons on labels, auto-comments, much, much more.

Removes the mundane chores of creating well-documented assembly language

source code.  A must for any serious assembly language programmer.

Distributed as AE*.ZIP where * is the version number.  AE22.ZIP was the

latest at this writing.

UNLOAD86:  This is the end-all and be-all of binary to Intel Hex conversion

programs.  Many options, supports fill to end of ROM, the new "Linear extended

address records", more.  Includes Source.  Distributed as UNLD86*.ZIP where

* is the version number.  UNLD8616.ZIP was the latest at this writing.

A couple of the initial versions were distributed as UNLOAD86.ZIP.  These

are severely downrev, and should be deleted.

BAUDASC:  Converts ASCII <---> Baudot.  Allows use of the ancient Baudot

5 bit code.  Includes Source.  Distrubuted as BAUDASC*.ZIP where * is the

version number.  BAUDASC1.ZIP was the latest at this writing.

CURSOR:  Very tiny TSR forces your IBM PC compatable's cursor to a block

at all times.  Much smaller than other programs of its type.  Includes source.

Distributed as CURSOR*.ZIP, where * is the version number.  CURSOR14.ZIP

was the latest at this writing.

UNLOAD:  [for CP/M].  Converts binary to Intel hex.  Only supports the

original phase 1 Intel hex specification.  Distributed as UNLOAD*.ARK

(may be .LBR some places) where * is the version number.  UNLOAD23.ARK

was the latest at this writing.

HELLO:  [for CP/M *and* MS-DOS].  Writes the string Hello. to the screen.

Big deal huh?  The point is the *same* physical .COM file works under *both*

CP/M and MS-DOS!!  Distributed as HELLO.ARK and HELLO.ZIP.  Includes source.

INDIO:  [for CP/M]  Shows how to do indirect I/O (ala Z-80) on an

8080 or 8085 processor.  NO dedicated memory locations, ROMable routine.

Source only.  Distributed as INDIO.ARK. (or .LBR)

CAPS:  Old timers like me are used to command line input in UPPER case.

CAPS is a tiny TSR that sets caps lock when an application exits.  Most

folks won't like/need this program.  Distributed as CAPS*.ZIP where

* is the current version number.  CAPS10.ZIP was the latest at this

writing.  Includes source.

Discontinued programs:

The following programs have been written in the past.  I no longer support

them, most (but not all) are gone from my board, but they may still be

floating around "out there".  Feel free to use them if you find them, but

don't ask me about them...  Most were quick hacks anyway...

SPLIT.ZIP:  Splits a binary input file into two files, one containing the

ODD bytes the other containing the EVEN bytes.  Useful for programming two

EPROMs on a 16 bit bus.  Included source. (I think...)

FIXHEX.ZIP:  Used to fix certain deficient Intel hex files.  Would strip off

any extended address records, and ensure that a correct EOF record was on

the end.  Fixed the old MAC/ASM :000000  (incorrect) EOF record.

FONT.ZIP:  Reprogram EGA and VGA character fonts.  Great for getting rid

of that ugly fat exclamation point!  Define your on-screen fonts any way

you like.  Included source.

SYSTEM.ARK: [for CP/M]  Allows jumping to any place in memory.  Input in

hex or decimal.  Included source.

Dead programs:

The following programs have been written in the past.  I no longer support

them, ALL are gone from my board, but they may still be floating around "out

there".  I DO NOT RECOMMEND USE OF ANY OF THESE, AND REQUEST THAT IF YOU

FIND THEM, YOU ASK THE SYSOP OF THE BOARD YOU DOWNLOADED THEM FROM TO

DELETE THEM ASAP!!  These are obsolete and no-longer of value.

OLDUSER:  Designed to automate the process of deleting "old" users from

a Spitfire BBS system.  Has long since been replaced by an official

Spitfire function.  Only worked with a very old versions of Spitfire.

SFZIPDOR:  A program designed to allow Spitfire callers the ability to

extract and selectively download members of any archive format.  Worked

on my board, but pulled from distribution due to in-stablities of the

program.  Plus it won't work above 9600 baud.  Don't use this program!!!

A replacement is possibly in the works, either from myself or from Spitfire.

MSGUNDEL:  Or something like that, I no-longer remember the exact name.

Program would globally un-delete any deleted messages in a Spitfire BBS

system.  Program is no-longer compatable with Spitfire, AND is no

longer needed as Spitfire now allows the Sysop to turn off message deletion.