This file contains information about the files on your JAWS disk.

Read it to learn how to run JAWS or JAWSDEMO, select

synthesizers, change communication parameters, and other

pertinent information.  For tips on using JAWS with specific

application programs and specific configuration files refer to

"WORDPROC.DOC", "SPREADSH.DOC", and "EMULATOR.DOC".  These files

are not meant to replace the JAWS manual which is also available

on disk, the files have the extension ".MAN".  You can also refer

to the JAWS Installation cassette tape and JAWS Training tapes

for more detailed information.

USING YOUR JAWS SCREEN READER

Your distribution disk will contain some of the following files:

"JAWS.EXE"  is the front end, it is installed with your name on

it.  It also should know what synthesizer you will be using as a

default or first choice.  When you execute JAWS  by entering

"JAWS" and pressing enter this program displays the JAWS logo,

your name and address, the copyright information, and then

schedules a TSR (Terminate and Stay Resident) program based on

your choice of synthesizer.

If you have a demo disk you will find "JAWSDEMO.EXE" instead of

"JAWS.EXE".  In the following instructions you should substitute

the file name "JAWSDEMO" wherever we use the file name "JAWS".

"JAWSDEMO.EXE" is the demonstration front end, it does some

initializing procedures then schedules the main part of the JAWS

system, the TSR program.

Files starting with "J_" and ending with the extension "TSR" are

the various TSR (Terminate and Stay Resident) programs for the

various synthesizers.  If you have a demo disk the TSR files

begin with "J_" and have the extension of ".EXE".

"J_ECHOXT.TSR" is the JAWS TSR program for the Echo PC from

Streit Electronics.

Remember that the JAWS system actually consists of 2 files: the

JAWS.EXE or JAWSDEMO.EXE front end and the TSR program.  When you

run JAWS you should set the default disk drive to the one where

the JAWS files are.  If they are in the "A" drive be sure you

have a "A>" prompt before entering "JAWS".  If you have copied

the JAWS files over to a subdirectory on a hard disk then default

your system to that disk drive and subdirectory.

To default to a particular disk drive enter the letter of the

disk followed by a ":", e.g. "C:" and press enter.  To default to

a given subdirectory first default to the proper disk, then enter

"CD\" followed by the name of the subdirectory, e.g. "cd\JAWS"

and press enter.  Then you can execute JAWS and the default TSR

program by entering "JAWS" and pressing enter.

To execute a JAWS TSR That is not the default one simply follow

the "JAWS" with the name of the synthesizer, e.g. "JAWS BNS" for

the Braille 'N Speak, or "JAWS ACCENT" for the Accent PC

synthesizer.  Note that in this case the name of the synthesizer

does not include the "J_" or the ".TSR".

To execute the JAWS demo type in the name "jawsdemo" followed by

the name of the synthesizer you want to demo.  For example:

JAWSDEMO S200

JAWSDEMO ACCENT

JAWSDEMO BNS

JAWSDEMO PROSE

One or both of these files, the front end and/or the TSR, can be

preceded by a disk drive letter designator and a path name to

tell DOS where to find it.

Now we will explain more about using the particular versions of

JAWS.

Be sure you have an Echo PC, which is a small box (5 inches by 4

inches by 1.5 inches) that connects to the computer with a serial

cable.  The cable must have a female (smooth surface) connector.

If the cable has a male (rough surface) connector it is probably

an Echo GP, the box is the same but the cable is wrong.  This

version of JAWS will not work with the Echo PC+, which is an

internal card or circuit board that fits inside the computer.

The 4 dip switches on the bottom of the Echo box should be on,

on, on, off.  That means if you hold it with the cables pointing

up and the dip switches facing you, starting on the left and

going to the right the switches would be up, up, up and down.  If

you have an Echo PC the cable should plug directly into the

Serial port on your IBM or compatible computer.  If you have an

Echo GP you should get another cable for the PC from the

manufacturer.  JAWS will try to communicate using Com1 at 9600

Baud, no parity, 7 data bits, 1 stop bit.  The dip switches

mentioned above will set up your Echo accordingly.  The "XT" in

the name means external as opposed to internal.  Here some

samples of how to run the JAWS software:

"JAWS ECHOXT"

"JAWSDEMO ECHOXT COM2:9600,n,8,1"

"JAWS ECHOXT COM1:1200,E,7,1"

After booting up JAWS you may notice a squeek each time you press

a key.  This is the "interuptible speech" feature of jaws:

whenever the echo is speaking and it is interupted it will

squeek.  You can disable the interupting by: enter the JAWS menu

(control /), select V for voice, K for keyboard, I for interupt,

F for ofF, press enter, then alt-escape.

For more details on the voice controls, the menus, and other JAWS

functions please refer to the JAWS training tapes or the manual.