SMCPWRN miniport driver for Intel Windows NT [tm] 3.51

Version v2.41

06-Jul-1995

1. Copyright Notice:

First Edition, April 1994

Updated, December 1994

Updated, February 1995

Updated, April 1995

Digital Equipment Corporation makes no representations that

the use of its products in the manner described in this

publication will not infringe on existing or future patent

rights, nor do the descriptions contained in this

publication imply the granting of licenses to make, use, or

sell equipment or software in accordance with the

description.

Copyright Digital Equipment Corporation 1994.

All rights reserved.

Do not redistribute the driver files.

Intel is a trademark of Intel Corporation.

Windows, Windows NT, and Windows95 are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

DEC, DECchip, Digital, and the DIGITAL logo, are

trademarks of Digital Equipment Corporation.

All other trademarks and registered trademarks are the

property of their respective holders.

2. Overview:

This file explains how to install the Windows NT driver

for x86-based platforms.  The driver is currently supported for

SMC's EtherPower Ethernet and Fast Ethernet adapters

3. Requirements:

A.  Hardware Requirements:

80386 or higher microprocessor based computer

with an EISA or PCI bus.

EtherPower PCI adapter

B.  Software Revision Requirements:

Windows NT Version 3.51 or later

4. Files and directories:

---------------------------------------------------------------------

INSTALL.DOC       This file.

---------------------------------------------------

SMCPWRN.SYS       The driver.

---------------------------------------------------

OEMSETUP.INF      The driver's setup file for Windows NT

---------------------------------------------------------------------

5. Installing the Hardware Boards:

A. Installing SMC's EtherPower PCI Ethernet Controller:

Refer to SMC's EtherPower the board's User Guide.

6. Installing the SMCPWRN Driver on Windows NT:

1. From the Windows NT Main Menu, select the Control Panel icon.

2. From the Control Panel menu select the Network icon.

If a dialog box asks you "Do you want to install NT

Network?" select YES and Continue. Then place

the Windows NT CDROM in the CD-ROM reader.

Select the drive that matches the configuration of your system

and complete the install.

3. You will see several messages. To add a network adapter, select Yes

and then Continue

4. For each additional message, select continue.

5. From the Add Network Adapter dialog box, scroll through the menu to

select Other.

6. When the Insert Diskettes menu prompts you for a disk, insert the

SMCPWRN driver installation diskette for Windows NT into drive A.

Then select OK.

7. Select your option

[SMC8432 EtherPower PCI Adapter      ]

[SMC8432e EtherPower PCI Adapter     ]

[SMC8434 EtherPower2 PCI Adapter     ]

[SMC9332 EtherPower10/100 PCI Adapter]

from the Select OEM Option box, then select OK.

8. Modify the default distribution path in the Windows NT

Setup box to point to the correct drive and path, then click

on [Continue]

9. For PCI options, a Dialog box pops up to select the:

DataRate:

------------------------------------------------------------------------

SMC 8432

-------------------------------------------------------------------------

o AutoDetect                  In AutoDetect mode the driver

checks first the Twisted Pair port.

If the Link Test fails the driver

checks the BNC port. If no carrier is

detected the driver switches to the AUI

port.

o Twisted_Pair (10BaseT)

o Twisted_Pair Full_Duplex    To connect to Full Duplex hubs

o Twisted_Pair No_Link_Test   To connect hubs not supporting

the Link Integrity Test

o BNC (10Base2)               BNC connector

o AUI (10Base5)              15_pin connector

------------------------------------------------------------------------

SMC 9332

-------------------------------------------------------------------------

o AutoDetect                 In AutoDetect mode the driver

checks the 100BaseTx and 10BaseT ports.

If no link is detected the driver

selects the 100BaseTx port.

o 100Mbps                    To connect 100BaseTx Ethernet hubs

o 100Mbps Full_Duplex        To connect 100BaseTx Full Duplex hubs

o 10Mbps                     To connect 10BaseT Ethernet hubs

10. Upon completion, the selected adapter is added to the

Installed Adapter Cards list of the Network Setting box

(the number prefixing the adapter is the adapter number).

11. Click on [Close] to complete the Network Setting

initialization.

12. If needed, modify the values in the registry (see section 7 below).

13. Shutdown and reboot the system.

7. Installing multiple PCI boards:

Repeat the procedure above.

8. Parameters

To overwrite the driver default values, add or edit the

following values in the Registry SMCPWRN<adapter_number> Parameters

subtree:

NetworkAddress = REG_SZ { xx-xx-xx-xx-xx-xx }

Mandatory if no Serial ID PROM is plugged in

Overwrite the Serial ID PROM if plugged in

NetworkAddress 00-00-00-00-00-00 is ignored

ReceiveBuffers = REG_DWORD {8..64} (default: 16)

Number of buffers allocated by the

driver in the Receive ring.

MapRegisters = REG_DWORD {4,8,16}   (default: 16(Alpha) | 8)

Number of map registers allocated

by the adapter.

CacheLineSize = REG_DWORD { 16 | 32 | 64 | 128 }

16 bytes

32 bytes (default)

64 bytes

128 bytes

The following values allow to overwrite the SMCPWR default setting.

Refer to the DECchip 21040, DECchip 21041 & DECchip 21140 Engineering

specifications for detailled description:

SnoozeMode = REG_DWORD { 0 | 1 }  (DC21041 only)

CFDA<SM>    0 Disabled (default)

1 Enabled

BurstLength = REG_DWORD  { 1 | 2 | 4 | 8 | 16 | 32 }

CSR0<PBL>  1  doubleword

2  doublewords

4  doublewords

8  doublewords   (default)

16  doublewords

32  doublewords

FifoArbitration = REG_DWORD { 0 | 1 }

CSR0<BAR>   0 Rcv/Txm Arbitration (default)

1 Round_Robin

StoreAndForward = REG_DWORD { 0 | 1 }  (DC21140 only)

Select the StoreAndForward mode if high

PCI latencies causes Transmit Underruns

CSR6<SF>    1 Enabled

0 Disabled (default)

TransmitThreshold = REG_DWORD { 72 | 96  | 128 | 160 |

256 | 512 | 1024        }

CSR6<TTM,TR>  72 bytes

96 bytes (DC2104X's default)

128 bytes

160 bytes (DC21140's default)

256 bytes (DC21140 only)

512 bytes (DC21140 only)

1024 bytes (DC21140 only)

StopBackoffCounter = REG_DWORD { 0 | 1 }

CSR6<SB>    0  Disabed (Default)

1  Enabled

BackPressure = REG_DWORD { 0 | 1 }

CSR6<BP>    0  Disabed (default)

1  Enabled

CaptureEffect = REG_DWORD { 0 | 1 }

CSR6<CA>    0  Disabed (Default)

1  Enabled

9. Error messages

The error messages are logged in the Entry Log and can be seen using the Entry Log Viewer (in

WNT's Administration Tools group).

Each error message has a standard Ndis error code and an driver's additional code.

The additional codes are:

DC21X4_ERRMSG_REGISTRY            0x01

DC21X4_ERRMSG_ALLOC_MEMORY        0x02

DC21X4_ERRMSG_SROM                0x03

DC21X4_ERRMSG_MEDIA               0x04

DC21X4_ERRMSG_LOAD_CAM            0x05

DC21X4_ERRMSG_SYSTEM_ERROR        0x06

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT  0x07

9.1 Driver initialization errors:

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_REGISTRY

The AdapterType Registry's key is missing or its value is

unsupported by the adapter board

o NDIS_ERROR_CODE_ADAPTER_NOT_FOUND

No board matching the AdapterCFID Registry's key value was found

plugged into the PCI bus or in the slot specified in the Registry

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_ALLOC_MEMORY

Not enough memory to allocate space for the Adapter data block

o NDIS_ERROR_CODE_INVALID_VALUE_FROM_ADAPTER

DC21X4_ERRMSG_SROM

Invalid data read from the adapter's serial ROM

o NDIS_ERROR_CODE_NETWORK_ADDRESS

No readable burnt_in or software configured station address

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_MEDIA

The selected media port is not supported by the board.

o NDIS_ERROR_CODE_INTERRUPT_CONNECT

Interrupt initialization failed

o NDIS_ERROR_CODE_TIMEOUT

DC21X4_ERRMSG_LOAD_CAM

DC21X4 CAM initialization failed

9.2 Driver run time errors:

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_SYSTEM_ERROR

System_Error interrupt (bus parity error)

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT

Transmit Jabber timer expired