___________________________________________________________________________

NDIS Driver Installation Summary for Microsoft LAN Manager

==========================================================

(These steps may be different for other OEM versions of LAN Manager.

Refer to the LAN Manager Installation Guide for more detailed

installation instructions.)

1.   Run SETUP from the fixed disk LANMAN directory (for OS/2)

or LANMAN.DOS directory (for DOS).

2.   Select the Action menu's "Import network drivers" command

and select the National Semiconductor EtherNODE*16AT driver

from the list of drivers on the Drivers diskette.

(This driver is used for EtherNODE*16AT, 16ATX or 16AT-T).

3.   Select the Action menu's "View/modify" command and "Remove"

the currently installed hardware device driver.

4.   Select the National Semiconductor EtherNODE*16AT device

driver from the list of available device drivers and "Add"

the device driver to the list of installed device drivers.

(This driver is used for EtherNODE*16AT, 16ATX or 16AT-T).

5.   Select the Action menu's "Save" command to save the selected

driver setup.

6.   Select the Exit menu's "Exit" command (or use function key F3)

to exit the SETUP program.

7.   Use a text editor program to modify the PROTOCOL.INI file

as described below in the PROTOCOL.INI reference section.

(The PROTOCOL.INI file is in the LANMAN directory [for OS/2]

or LANMAN.DOS directory [for DOS]). If you are not using the

default IOBASE address or INTERRUPT number on the EtherNODE

board, these values must be changed in the PROTOCOL.INI file.

If you have more than one EtherNODE board installed, the

DRIVERNAME for all but the first one must be changed.

___________________________________________________________________________

NDIS PROTOCOL.INI Reference

===========================

This section describes the PROTOCOL.INI keywords used for the

Microsoft EtherNODE*16AT NDIS device driver v1.02N.

IOBASE - This keyword describes the IO base address used by the

adapter. It must match the jumper settings as described in the

installation manual. The values of IOBASE which the EtherNODE

board supports range from 300H to 360H in increments of 20H.

The IOBASE default is 300H. For the EtherNODE*16ATX and

EtherNODE*16AT-T boards, IOBASE can also be 120H, 140H, or 160H.

INTERRUPT - This keyword describes the interrupt used by the

adapter. No other device installed in the computer may use this

interrupt. Its value must match the jumper setting as described

in the installation manual. INTERRUPT can range from 2 to 5

inclusive; the default is 3. For the EtherNODE*16ATX and

EtherNODE*16AT-T boards, INTERRUPT can also be 7, 10, or 11.

INTERRUPT values 10 or 11 are only valid for 16-bit slot operation

in AT or higher class computers (not XT or AT 8-bit slots).

DRIVERNAME - This keyword describes the device driver's name. The

default is MS2000$ for any of the EtherNODE boards. If more than

one adapter is installed, the device driver names increment as

follows:

MS20002$, MS20003$, MS20004$, ..., MS20009$

Following is an example PROTOCOL.INI fragment:

[NSC2000_NIF]

Drivername = MS2000$

Interrupt = 3

IoBase = 0x300

Trademarks:

LAN Manager is a trademark of Microsoft Corporation.

OS/2 is a trademark of International Business Machines Corporation.